| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-HO-CMD and AAI-NBR-ADV ASN.1 code |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Joey Chou Xiangying Yang Intel Hyunjeong Kang Samsung | Joey.Chou@intel.com Xiangying.yang@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-HO-CMD and AAI-NBR-ADV ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-HO-CMD and AAI-NBR-ADV ASN.1 code

Joey Chou, Xiangying Yang

Intel

Hyunjeong Kang

Samsung

Introduction

The “delta Information” attribute in the AAI-NBR-ADV and AAI-HO-CMD messages are to indicate the SFH attributes that have been changed between current carrier / preceding carrier and the reference carrier. Each SFH attribure should only be present when such attribute is changed. Therefore, each SFH attribute should be optional. But, the ASN.1 code in AAI-NBR-ADV and AAI-HO-CMD messages in delta information does not provide OPTIONAL tag on per SFH attribute basis.

This contribution provides text to fix the delta information issue and other issues.

Proposed Text

Remedy 1:

In the P802.16m/D9, page1032, line 49, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Handover Command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-HO-CMD ::= SEQUENCE {

mode		CHOICE {

hoCmd 		HandoverCommand, 	-- 0b00

zsCmd		ZoneSwitchCommand, 	-- 0b01

hoReject	NULL,				-- 0b10

reserved	NULL				-- 0b11

}

}

HandoverCommand ::= SEQUENCE {

hoReentryMode			CHOICE {

bbeHO	NULL,

ebbHO	EBBHOInfo

},

disconnectTimeOffset	INTEGER(0..255),

resourceRetainTime		INTEGER(0..255),

targetBSList			SEQUENCE (SIZE(0..maxNewABSIndices)) OF TargetBSInfo

}

EBBHOInfo ::= SEQUENCE {

-- "interleavingMode" is used to avoid using

-- OPTIONAL to those hoReentryXXXX parameters

interleavingMode		CHOICE {

-- hoReentryInterleavingInterval > 0

withInterleaving	HoReentryInterleavingInfo,

-- hoReentryInterleavingInterval = 0(multicarrier EBB)

noInterleaving		NULL

},

servingPhyCarrierID		PhyCarrierIndex		OPTIONAL

}

HoReentryInterleavingInfo ::= SEQUENCE {

hoReentryInterleavingInterval	INTEGER(1..256),

hoReentryInterval				INTEGER(1..256),

hoReentryIteration				INTEGER(1..8)

}

TargetBSInfo ::= SEQUENCE {

targetBSID					BSID,

saPreambleIndex				PreambleIndex,

faIndex						FAIndex						OPTIONAL, -- Only included for the target legacy BS

actionTime					INTEGER(0..255),

seamlessHO					BOOLEAN,

cdmaRngFlag					CHOICE {

offsetInfo			OffsetInfo,			-- CDMA_RNG_FLAG=0: skip CDMA ranging

dedicatedRngInfo	DedicatedRngInfo	-- CDMA_RNG_FLAG=1: perform CDMA ranging

},

reentryProcessOptimization		BIT STRING(SIZE(5)),

rngInitDeadline				INTEGER(0..255),

preassignedSTID				STID 						OPTIONAL,

preassignedMAPMaskKey		BIT STRING(SIZE(15))		OPTIONAL,

serviceLevelPrediction		ServiceLevelPrediction,

targetPhyCarrierID			PhyCarrierIndex				OPTIONAL,

channelBandwidth			INTEGER(0..255)				OPTIONAL,

cpLength					CPLength					OPTIONAL,

preassignedCarrierList		SEQUENCE (SIZE(0..maxPreassignedCarriers)) OF

PreassignedCarrierInfo 		OPTIONAL,

preallocatedBasicCID		BasicCID					OPTIONAL,

sfhDeltaInfo				SEQUENCE {

s-SFHChangeCount		INTEGER(0..15),

s-SFHApplicationTime	BOOLEAN,

sfhDeltaInfo 				SEQUENCE {

sfhSubpacket1         OptSFHSubpacket1 OPTIONAL,

sfhSubpacket2         OptSFHSubpacket2 OPTIONAL,

sfhSubpacket3         OptSFHSubpacket3 OPTIONAL }	OPTIONAL

}

}

OffsetInfo ::= SEQUENCE {

offsetData			INTEGER(0..127) OPTIONAL,

offsetControl		INTEGER(0..127) OPTIONAL,

}

DedicatedRngInfo ::= SEQUENCE {

dedicatedCDMARngCode	INTEGER(0..31)		OPTIONAL,

rngOptIndex				INTEGER(0..7)		OPTIONAL,

rngOptSubframeIndex		INTEGER(0..3)		OPTIONAL,

}

ServiceLevelPrediction ::= ENUMERATED {

noServiceAvailable(0),

partiallyServiceAvailable(1),

allServiceAvailable(2),

noServiceLevelPrediction(3)

}

PreassignedCarrierInfo ::= SEQUENCE {

carrierStatusIndication		BOOLEAN,

physicalCarrierIndex		PhyCarrierIndex

}

ZoneSwitchCommand ::= SEQUENCE {

hoReentryMode				BOOLEAN,

resourceRetainTime			INTEGER(0..255)		OPTIONAL,

faIndex						FAIndex				OPTIONAL,

actionTime					INTEGER(0..255),

preambleIndexLZone			BIT STRING (SIZE(8)),

preallocatedBasicCID		BasicCID			OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Neighbor Advertisement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-NBR-ADV ::= SEQUENCE {

changeCount					INTEGER (0..7),

totalNumberOfCellTypes		INTEGER (1..8),

cellType					ENUMERATED {macro, micro, macro-hotzone, femto, relay, r1-lzone, spare1, spare2},

totalNumberOfSegments		INTEGER (1..16),

segmentIndex				INTEGER (0..15),

startingABSIndex			INTEGER (0..255),

nbrABSInfoList				SEQUENCE (SIZE(1..maxNeighborABSs)) OF NeighborABSInfo,

nbrR1BSInfoList				SEQUENCE (SIZE(1..maxNeighborR1BSs)) OF NeighborR1BSInfo	OPTIONAL,

-- For ABS type whose system info are not included in AAI_NBR-ADV

cellTypeInfo				CellTypeInfo		OPTIONAL,

-- Optional LDM parameters included when they are to be changed

ldmParameters				LDMParameters		OPTIONAL

}

CellTypeInfo ::= SEQUENCE {

phyCarrierIDList	SEQUENCE(SIZE(1..maxPhyCarrierIndices)) OF PhyCarrierIndex OPTIONAL,

rangeIDCell SEQENCE{

startIDCell			IDCell,

endIDCell			IDCell

} OPTIONAL,

...

}

NeighborABSInfo ::= SEQUENCE {

bsID					BSID,

macVersion				MacProtocolVersion,

cpLength				CPLength,

carrierInfoList			SEQUENCE (SIZE(1..maxCarriers)) OF CarrierInfo,

nbrSpecificTrigger		Triggers		OPTIONAL

}

CarrierInfo ::= SEQUENCE {

idCell					IDCell,

aPreambleTxPower		INTEGER(0..255),

phyCarrierIndex			PhyCarrierIndex	,

pgid					PGID,

sfhChangeCount 			INTEGER (0..15),

sfhEncFmt 				CHOICE {

-- All parameters of SFHSubpacket shall be included

fullSubpkt				SFHSubpacket,

-- Parameters of SFHSubpacket are partially included

deltaInfoCurrentCxr		OPTSFHSubpacket,

-- Parameters of SFHSubpacket are partially included

deltaInfoPrecedingCxr	OPTSFHSubpacket,

noSFHIncluded			NULL

}

}

NeighborR1BSInfo ::= SEQUENCE {

bsID					BSID,

r1PreambleIndex			BIT STRING (SIZE(8)),

phyModeID				INTEGER(0..65535),

channelBW				ENUMERATED {five-mhz, seven-mhz, eightPoint75-mhz, ten-mhz},

r1BSPhyCarrierIndex		PhyCarrierIndex

}

LDMParameters ::= SEQUENCE {

newAI			INTEGER(0..15),

newUAI			INTEGER(0..255),

newSFO			INTEGER(0..511)

}

--

-- Parameters of SFH IEs

-- All variables in SFH-SP1, SFH-SP2 and SFH-SP3 will be OPTIONAL

-- so that the SFHSbpacket structure can be reused by CarrierInfo

-- for different sfhEncFormat

--

SFHSubpacket ::= SEQUENCE {

sfh-sp1		SFHSubpacket1		OPTIONAL,

sfh-sp2		SFHSubpacket2		OPTIONAL,

sfh-sp3		SFHSubpacket3		OPTIONAL

}

OptSFHSubpacket ::= SEQUENCE {

sfh-sp1        OptSFHSubpacket1,

sfh-sp2        OptSFHSubpacket2,

sfh-sp3        OptSFHSubpacket3

}

FreqPartitionReuse ::= ENUMERATED {

reusePartition1,

reusePowerBoostedPartition3

}

MaxRetransmissions ::= ENUMERATED {

four,

eight

}

IotCorrectionValue ::= ENUMERATED {

plus1dB,

plus0Dot5dB,

zerodB,

minus0Dot5dB

}

DlHarqfdbkA-Maps ::= CHOICE {

fiveMhzChannel                 INTEGER (4|8|12|16),

tenMhzChannel                  INTEGER (8|16|24|32),

twentyMhzChannel               INTEGER (16|32|48|64)}

PwrControlRscSize ::= ENUMERATED {

noUseOfPcAMap,

seven,

fourteen,

twentyFour}

A-MapMcsSelection ::= ENUMERATED {

reuse1-qpsk-1half-1forth-reuse3-qpsk-1half,

reuse1-qpsk-1half-1eighth-reuse3-qpsk-1forth}

CellBar ::= ENUMERATED {

cellNotBarred,

cellBarred }

RxProcessingTime ::= ENUMERATED {

threeSubframes,

fourSubframes}

CellSpecRngConfig ::= ENUMERATED {

femtoAbs,

nonFemtoAbs}

UcasDcas-Sb0-Mb0 ::= CHOICE {

fftSize2048                    BIT STRING (SIZE(5)),

fftSize1024                    BIT STRING (SIZE(4)),

fftSize512                     BIT STRING (SIZE(3))}

UcasDcas-i ::= CHOICE {

fftSize2048                    BIT STRING (SIZE(3)),

fftSize1024                    BIT STRING (SIZE(2)),

fftSize512                     BIT STRING (SIZE(1))}

UlSubframesSounding ::= INTEGER {noSoundingSymbol (0),

oneAaiSubframe   (1),

twoAaiSubframe   (2),

threeAaiSubframe (3),

fourAaiSubframe  (4)} (0..7)

SFHSubpacket1 ::= SEQUENCE {

superframe INTEGER (0..255),

absMacIdLsb BIT STRING (SIZE(12)),

ulHarqFbkChannels DlHarqfdbkA-Maps,

dlHarqFbkAMaps HarqfdbkChannels,

powerControlRscSize PwrControlRscSize

,

dlFreqPartLocation FreqPartitionReuse,

ulFreqPartLocation FreqPartitionReuse,

aMapMcsSelection A-MapMcsSelection

,

dcasSb0 UcasDcas-Sb0-Mb0,

dcasMb0 UcasDcas-Sb0-Mb0,

dcasI UcasDcas-i,

frameConfigurationIndex INTEGER(0..63),

ulSubframesSounding UlSubframesSounding,

absEirp INTEGER (1..64),

cellBar CellBar

,

ulMaxReTx MaxRetransmissions,

dlMaxReTx MaxRetransmissions,

rxProcessingTime RxProcessingTime

,

cellSpecificRangingConfig CellSpecRngConfig

,

wirelessManOdfma FeatureSupport,

choice CHOICE {

fdmBasedUlPuscZone SEQUENCE {

rchAllocPeriodicity INTEGER (0..3),

rchSubframeOffset INTEGER (0..3),

startRPCode INTEGER (0..15),

rpCodePartition INTEGER (0..15),

ulPermBase INTEGER (0..127)

},

femtocell SEQUENCE {

rchAllocPeriodicity INTEGER (0..3),

rchSubframeOffset INTEGER (0..3),

startRPCode INTEGER (0..15),

rpCodePartition INTEGER (0..15),

rchTxTiming INTEGER (0..7),

ucasSb0 UcasDcas-Sb0-Mb0,

ucasMb0 UcasDcas-Sb0-Mb0,

ucasI UcasDcas-i

},

other SEQUENCE {

rchAllocPeriodicity INTEGER (0..3),

rchSubframeOffset INTEGER (0..3),

startRPCode INTEGER (0..15),

rpCodePartition INTEGER (0..15),

cyclicShiftedRPCodes INTEGER (0..3),

nsRchFormat INTEGER (0..1),

ucasSb0 UcasDcas-Sb0-Mb0,

ucasMb0 UcasDcas-Sb0-Mb0,

ucasI UcasDcas-i

}

},

...

}

UlBandwidth ::= INTEGER {fft512 (0),

fft1024 (1),

fft2048 (2)} (0..7)

FddInfo ::= SEQUENCE {

ulDlFreqSeparation INTEGER (0..63),

ulBandwidth UlBandwidth}

NetworkConfig ::= ENUMERATED {aaiNetworkConfig,

wirelessManOfdmaConfig}

DsacUsac ::= CHOICE {

fftSize2048                    BIT STRING (SIZE(5)),

fftSize1024                    BIT STRING (SIZE(4)),

fftSize512                     BIT STRING (SIZE(3))}

DfpscUfpsc ::= CHOICE {

fftSize2048                    BIT STRING (SIZE(3)),

fftSize1024                    BIT STRING (SIZE(2)),

fftSize512                     BIT STRING (SIZE(1))}

DfpcUfpc ::= CHOICE {

fftSize2048                    BIT STRING (SIZE(4)),

fftSize1024                    BIT STRING (SIZE(3)),

fftSize512                     BIT STRING (SIZE(3))}

MimoMidamble ::= ENUMERATED {

notTransmittedInRelayZone,

transmittedInRelayZone}

SFHSubpacket2 ::= SEQUENCE {

macVersion INTEGER (0..15),

duplexingMode CHOICE {

notFdd                NULL,

fdd                   FddInfo},

absMacIdMsb BIT STRING (SIZE(36)),

networkConfig NetworkConfig

,

olRegion INTEGER (0..1),

dsac DsacUsac,

dfpc DfpscUfpsc,

dfpsc DfpscUfpsc,

usac DsacUsac,

ufpc DfpscUfpsc,

ufpsc DfpscUfpsc,

amsTxPowerLimitation INTEGER (0..31),

eiRxP INTEGER (0..31),

mimoMidamble MimoMidamble

,

...

}

SsfhChangeCycle ::= INTEGER {sixteen (0),

thirtyTwo (1),

sixtyFour (2)} (0..15)

NumTxAntennas ::= INTEGER {twoAntenna   (0),

fourAntenna  (1),

eightAntenna (2)} (0..4)

UlBwReqChannelInfo ::= INTEGER {

firstAaiSubframein1stFrame   (0),

firstAaiSubframein1stAnd2ndFrame (1),

firstAaiSubframeinEveryFrame (2),

first2AaiSubframeinEveryFrame (3),

first4AaiSubframeinEveryFrame (4)} (0..7)

D-LbsZoneConfig ::= ENUMERATED {noLbsZoneTransmission,

zonePeriodicity4,

zonePeriodicity16,

zonePeriodicity32}

SFHSubpacket3 ::= SEQUENCE {

ssfhChangeCycle SsfhChangeCycle

,

preambleSeqSoftPartitioning INTEGER (0..15),

ulIotCorrectionValues SEQUENCE {

iotCorrValueSounding IotCorrectionValue,

iotCorrValueFP0 IotCorrectionValue,

iotCorrValueFP1 IotCorrectionValue,

iotCorrValueFP2 IotCorrectionValue,

iotCorrValueFP3 IotCorrectionValue

},

ulFeedbackSize INTEGER (0..15),

numTxAntennas NumTxAntennas

,

spSchedulingPeriodicity INTEGER (0..15),

hoRangingBackoffStart INTEGER (0..15),

hoRangingBackoffEnd INTEGER (0..15),

initialRangingBackoffStart INTEGER (0..15),

initialRangingBackoffEnd INTEGER (0..15),

ulBwReqChannelInfo INTEGER (0..7),

bandwidthRequestBackoffStart INTEGER (0..15),

bandwidthRequestBackoffEnd INTEGER (0..15),

fpPowerConfig INTEGER (0..7),

dLbsZoneConfig D-LbsZoneConfig

,

nspChangeCount INTEGER (0..15),

scdCount INTEGER (0..15),

...

}

OptSFHSubpacket1 ::= SEQUENCE {

superframe INTEGER (0..255) OPTIONAL,

absMacIdLsb BIT STRING (SIZE(12)) OPTIONAL,

ulHarqFbkChannels DlHarqfdbkA-Maps OPTIONAL,

dlHarqFbkAMaps HarqfdbkChannels OPTIONAL,

powerControlRscSize PwrControlRscSize OPTIONAL,

dlFreqPartLocation FreqPartitionReuse OPTIONAL,

ulFreqPartLocation FreqPartitionReuse OPTIONAL,

aMapMcsSelection A-MapMcsSelection OPTIONAL,

dcasSb0 UcasDcas-Sb0-Mb0 OPTIONAL,

dcasMb0 UcasDcas-Sb0-Mb0 OPTIONAL,

dcasI UcasDcas-i OPTIONAL,

frameConfigurationIndex INTEGER(0..63) OPTIONAL,

ulSubframesSounding UlSubframesSounding OPTIONAL,

absEirp INTEGER (1..64) OPTIONAL,

cellBar CellBar OPTIONAL,

ulMaxReTx MaxRetransmissions OPTIONAL,

dlMaxReTx MaxRetransmissions OPTIONAL,

rxProcessingTime RxProcessingTime OPTIONAL,

cellSpecificRangingConfig CellSpecRngConfig OPTIONAL,

wirelessManOdfma FeatureSupport OPTIONAL,

choice CHOICE {

fdmBasedUlPuscZone SEQUENCE {

rchAllocPeriodicity INTEGER (0..3) OPTIONAL,

rchSubframeOffset INTEGER (0..3) OPTIONAL,

startRPCode INTEGER (0..15) OPTIONAL,

rpCodePartition INTEGER (0..15) OPTIONAL,

ulPermBase INTEGER (0..127) OPTIONAL

},

femtocell SEQUENCE {

rchAllocPeriodicity INTEGER (0..3) OPTIONAL,

rchSubframeOffset INTEGER (0..3) OPTIONAL,

startRPCode INTEGER (0..15),

rpCodePartition INTEGER (0..15) OPTIONAL,

rchTxTiming INTEGER (0..7) OPTIONAL,

ucasSb0 UcasDcas-Sb0-Mb0 OPTIONAL,

ucasMb0 UcasDcas-Sb0-Mb0 OPTIONAL,

ucasI UcasDcas-i OPTIONAL

},

other SEQUENCE {

rchAllocPeriodicity INTEGER (0..3) OPTIONAL,

rchSubframeOffset INTEGER (0..3) OPTIONAL,

startRPCode INTEGER (0..15) OPTIONAL,

rpCodePartition INTEGER (0..15) OPTIONAL,

cyclicShiftedRPCodes INTEGER (0..3) OPTIONAL,

nsRchFormat INTEGER (0..1) OPTIONAL,

ucasSb0 UcasDcas-Sb0-Mb0 OPTIONAL,

ucasMb0 UcasDcas-Sb0-Mb0 OPTIONAL,

ucasI UcasDcas-i OPTIONAL

}

},

...

}

OptSFHSubpacket2 ::= SEQUENCE {

macVersion INTEGER (0..15) OPTIONAL,

duplexingMode CHOICE {

notFdd                NULL,

fdd                   FddInfo},

absMacIdMsb BIT STRING (SIZE(36)) OPTIONAL,

networkConfig NetworkConfig OPTIONAL,

olRegion INTEGER (0..1) OPTIONAL,

dsac DsacUsac OPTIONAL,

dfpc DfpscUfpsc OPTIONAL,

dfpsc DfpscUfpsc OPTIONAL,

usac DsacUsac OPTIONAL,

ufpc DfpscUfpsc OPTIONAL,

ufpsc DfpscUfpsc OPTIONAL,

amsTxPowerLimitation INTEGER (0..31) OPTIONAL,

eiRxP INTEGER (0..31) OPTIONAL,

mimoMidamble MimoMidamble OPTIONAL,

...

}

OptSFHSubpacket3 ::= SEQUENCE {

ssfhChangeCycle SsfhChangeCycle OPTIONAL,

preambleSeqSoftPartitioning INTEGER (0..15) OPTIONAL,

ulIotCorrectionValues SEQUENCE {

iotCorrValueSounding IotCorrectionValue OPTIONAL,

iotCorrValueFP0      IotCorrectionValue OPTIONAL,

iotCorrValueFP1      IotCorrectionValue OPTIONAL,

iotCorrValueFP2      IotCorrectionValue OPTIONAL,

iotCorrValueFP3      IotCorrectionValue OPTIONAL

} OPTIONAL,

ulFeedbackSize INTEGER (0..15) OPTIONAL,

numTxAntennas NumTxAntennas ,

spSchedulingPeriodicity INTEGER (0..15) OPTIONAL,

hoRangingBackoffStart INTEGER (0..15) OPTIONAL,

hoRangingBackoffEnd INTEGER (0..15) OPTIONAL,

initialRangingBackoffStart INTEGER (0..15) OPTIONAL,

initialRangingBackoffEnd INTEGER (0..15) OPTIONAL,

ulBwReqChannelInfo UlBwReqChannelInfo OPTIONAL,

bandwidthRequestBackoffStart INTEGER (0..15) OPTIONAL,

bandwidthRequestBackoffEnd INTEGER (0..15) OPTIONAL,

fpPowerConfig INTEGER (0..7) OPTIONAL,

dLbsZoneConfig D-LbsZoneConfig OPTIONAL,

nspChangeCount INTEGER (0..15) OPTIONAL,

scdCount INTEGER (0..15) OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Remedy 3:

In the P802.16m/D9, page129, line 28, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

| If (CDMA_RNG_FLAG==0){ |  |  |  |
| --- | --- | --- | --- |
| offsetData | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | optional |
| offsetControl | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | optional |
| Pre-assigned STID | 12 | STID assigned to the AMS by the T-ABS |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------