| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up text of the AAI_RNG-ACK message (D9 - 16.2.3.3) |  |
| Date Submitted | 2010-10-26 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | Sponsor Ballot on IEEE p802.16m D9 |  |
| Abstract | This contribution proposes clean-up text of the AAI_RNG-ACK message. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up text of the AAI_RNG-ACK message (D9 - 16.2.3.3)

Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the clean-up text for the AAI_RNG-ACK message in the IEEE 802.16m DRAFT Amendment [1]. According to D9, it is sent by the ABS in a DL resource allocated by Broadcast assignment A-MAP IE when the ABS receives ranging preamble code(s). Also, it can be transmitted to specific user for the following two purposes:

1. Response of dedicated ranging preamble code(s);

2. Transmission of PHY-level corrections based on measurements that have been made on other received data or MAC messages.

Because the current draft is described only for the first purpose, we propose the text for the second purpose.

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[Remedy #1: Adopt the following modification in page 105, line 1, section 16.2.3.3]

16.2.3.3 AAI-RNG-ACK

Table 680—AAI-RNG-ACK message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| } else if (transmitted in a DL resource allocated by DL basic assignment A-MAP IE) { |  |  |  |
| Ranging Status | 2 | Indicate whether ranging preamble code or UL burst is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue |  |
| If (Ranging Status == 0b01) { |  |  |  |
| …… | …… | …… |  |

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

- References

[1] IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”