| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-EMBS-CFG ASN.1 code |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-EMBS-CFG ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-EMBS-CFG ASN.1 code

Joey Chou

Intel

Introduction

This contribution proposes fixes to AAI-EMBS-CFG ASN.1 code, in order to align with AAI-EMBS-CFG table.

Proposed Text

Remedy 1:

In the P802.16m/D9, page1077, line 17, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- E-MBS Configuration

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-EMBS-CFG ::= SEQUENCE {

lifetime                       INTEGER (0..15),

embsZoneIDInfoArray            SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

mapResourceIndex             INTEGER (0..2047),

mapSizeOffset                INTEGER (0..31),

aaiNbrAdvChangeCount         INTEGER (0..7),

nbrEmbsZoneInfoArray         SEQUENCE (SIZE(0..8)) OF SEQUENCE {

nbrEMBSZoneID              EMBSZoneID,

carrierIndex             PhyCarrierIndex OPTIONAL,

nbrABSInfoArray          SEQUENCE (SIZE(1..256)) OF SEQUENCE {

nbrAbsIndex            INTEGER (0..255)} OPTIONAL,

-- 16 MSBs is the current E-MBS ID and FID, and 16 LSBs is the

-- new E-MBS ID and FID

embsFIDMappingArray      SEQUENCE (SIZE(0..15)) OF SEQUENCE {

embsFIDMapping         BIT STRING(SIZE(32))} OPTIONAL

}

},

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------