| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-EMBS-CFG and MIMO ASN.1 code |  |
| Date Submitted | 2010-11-03 |  |
| Source(s) | Joey Chou Intel Wookbong Lee Jeongki Kim LGE | Joey.Chou@intel.com wookbong.lee@lge.com jeongki.kim@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-EMBS-CFG and MIMO ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-EMBS-CFG and MIMO ASN.1 code

Joey Chou

Intel

Wookbong Lee, Jeongki Kim

LGE

Introduction

This contribution proposes fixes to AAI-EMBS-CFG and MIMO ASN.1 code, in order to align with AAI-EMBS-CFG table.

Proposed Text

Remedy 1:

In the P802.16m/D9, page1077, line 17 to line 53, Replace with  the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- E-MBS Configuration

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-EMBS-CFG ::= SEQUENCE {

lifetime                       INTEGER (0..15),

embsZoneIDInfoArray            SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID                   EMBSZoneID,

mapResourceIndex             INTEGER (0..2047),

mapSizeOffset                INTEGER (0..31),

aaiNbrAdvChangeCount         INTEGER (0..7),

nbrEmbsZoneInfoArray         SEQUENCE (SIZE(1..8)) OF SEQUENCE {

nbrEMBSZoneID              EMBSZoneID,

carrierIndex             PhyCarrierIndex OPTIONAL,

nbrABSInfoArray          SEQUENCE (SIZE(1..256)) OF SEQUENCE {

nbrAbsIndex            INTEGER (0..255)},

embsFIDMappingArray      SEQUENCE (SIZE(0..15)) OF SEQUENCE {

currentEmbsId          EMBSID,

currentFid             FID,

newEmbsId              EMBSID,

newFid                 FID

}

} OPTIONAL

},

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D9, page1060, line 17, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- MIMO Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

RowOf2X2Matrix ::= SEQUENCE {

rows                           CHOICE {

firstRow                     SEQUENCE {

diagonalEntry              Diagonal,

offDiagonalEntry           OffDiagonal},

secondRow                    SEQUENCE {

empty                      NULL,

diagonalEntry              Diagonal}

}

}

RowOf4X4Matrix ::= SEQUENCE {

rows                           CHOICE {

firstRow                     SEQUENCE {

diagonalEntry              Diagonal,

c2offDiagonalEntry         OffDiagonal,

c3offDiagonalEntry         OffDiagonal,

c4offDiagonalEntry         OffDiagonal},

secondRow                    SEQUENCE {

empty1                     NULL,

diagonalEntry              Diagonal,

c3offDiagonalEntry         OffDiagonal,

c4offDiagonalEntry         OffDiagonal},

thirdRow                     SEQUENCE {

empty1                     NULL,

empty2                     NULL,

diagonalEntry              Diagonal,

c4offDiagonalEntry         OffDiagonal},

fourthRow                    SEQUENCE {

empty1                     NULL,

empty2                     NULL,

empty3                     NULL,

diagonalEntry              Diagonal}

}

}

RowOf8X8Matrix ::= SEQUENCE {

rows                           CHOICE {

firstRow                     SEQUENCE {

diagonalEntry              Diagonal,

c2offDiagonalEntry         OffDiagonal,

c3offDiagonalEntry         OffDiagonal,

c4offDiagonalEntry         OffDiagonal,

c5offDiagonalEntry         OffDiagonal,

c6offDiagonalEntry         OffDiagonal,

c7offDiagonalEntry         OffDiagonal,

c8offDiagonalEntry         OffDiagonal},

secondRow                    SEQUENCE {

empty1                     NULL,

diagonalEntry              Diagonal,

c3offDiagonalEntry         OffDiagonal,

c4offDiagonalEntry         OffDiagonal,

c5offDiagonalEntry         OffDiagonal,

c6offDiagonalEntry         OffDiagonal,

c7offDiagonalEntry         OffDiagonal,

c8offDiagonalEntry         OffDiagonal},

thirdRow                     SEQUENCE {

empty1                     NULL,

empty2                     NULL,

diagonalEntry              Diagonal,

c4offDiagonalEntry         OffDiagonal,

c5offDiagonalEntry         OffDiagonal,

c6offDiagonalEntry         OffDiagonal,

c7offDiagonalEntry         OffDiagonal,

c8offDiagonalEntry         OffDiagonal},

fourthRow                    SEQUENCE {

empty1                     NULL,

empty2                     NULL,

empty3                     NULL,

diagonalEntry              Diagonal,

c5offDiagonalEntry         OffDiagonal,

c6offDiagonalEntry         OffDiagonal,

c7offDiagonalEntry         OffDiagonal,

c8offDiagonalEntry         OffDiagonal},

fifthRow                     SEQUENCE {

empty1                     NULL,

empty2                     NULL,

empty3                     NULL,

empty4                     NULL,

diagonalEntry              Diagonal,

c6offDiagonalEntry         OffDiagonal,

c7offDiagonalEntry         OffDiagonal,

c8offDiagonalEntry         OffDiagonal},

sixthRow                     SEQUENCE {

empty1                     NULL,

empty2                     NULL,

empty3                     NULL,

empty4                     NULL,

empty5                     NULL,

diagonalEntry              Diagonal,

c7offDiagonalEntry         OffDiagonal,

c8offDiagonalEntry         OffDiagonal},

seventhRow                   SEQUENCE {

empty1                     NULL,

empty2                     NULL,

empty3                     NULL,

empty4                     NULL,

empty5                     NULL,

empty6                     NULL,

diagonalEntry              Diagonal,

c8offDiagonalEntry         OffDiagonal},

eighthRow                    SEQUENCE {

empty1                     NULL,

empty2                     NULL,

empty3                     NULL,

empty4                     NULL,

empty5                     NULL,

empty6                     NULL,

empty7                     NULL,

diagonalEntry              Diagonal}

}

}

-- Diagonal entry of correlation matrix as defined

-- in section 16.3.6.2.5.5

Diagonal ::= INTEGER (0..1)

-- Off Diagonal entry of correlation matrix as defined

-- in section 16.3.6.2.5.5

OffDiagonal ::= INTEGER (0..15)

MfmBitmap ::= BIT STRING {

mfm0 (0),

mfm1 (1),

mfm2 (2),

mfm3 (3),

mfm4 (4),

mfm5 (5),

mfm6 (6),

mfm7 (7)

} (SIZE(8))

Mfm0InfoSet ::= SEQUENCE {

wbstcRate StcRateSet OPTIONAL,

widebandCqi INTEGER (0..15)

}

Mfm1InfoSet ::= SEQUENCE {

widebanCqi INTEGER (0..15)

}

Mfm2InfoSet ::= SEQUENCE {

bestSubbandIndex BestSubbands OPTIONAL,

stcRate StcRateSet OPTIONAL,

subbandCqi SubbandCqiSetChoice

}

Mfm3InfoSet ::= SEQUENCE {

bestSubbandIndex BestSubbands OPTIONAL,

stcRate StcRateSet OPTIONAL,

subbandCqi SubbandCqiSetChoice,

subbandPmi SubbandPmiSetChoice

}

Mfm4InfoSet ::= SEQUENCE {

wbstcRate StcRateSet OPTIONAL,

widebandCqi INTEGER (0..15),

widebandPmi WidebandPmiSet OPTIONAL

}

Mfm5InfoSet ::= SEQUENCE {

bestSubbandIndex BestSubbands OPTIONAL,

subbandCqi SubbandCqiSetChoice,

subbandStream SubbandStreamSetChoice

}

Mfm6InfoSet ::= SEQUENCE {

bestSubbandIndex BestSubbands OPTIONAL,

subbandCqi SubbandCqiSetChoice,

subbandPmi SubbandPmiSetChoice

}

Mfm7InfoSet ::= SEQUENCE {

widebandCqi INTEGER (0..15),

widebandPmi WidebandPmiSet OPTIONAL

}

WidebandPmiSet ::= CHOICE {

nt2 BIT STRING (SIZE(3)),

nt4CodeBook0 BIT STRING (SIZE(6)),

nt4CodeBook1 BIT STRING (SIZE(4)),

nt8 BIT STRING (SIZE(4))

}

BestSubbands ::= CHOICE {

fiveM BestSubbandForFiveM,

tenM BestSubbandForTenM,

twentyM BestSubbandForTwentyM

}

BestSubbandForFiveM ::= CHOICE {

best1 INTEGER (0..3)

}

BestSubbandForTenM ::= CHOICE {

best1 INTEGER (0..9),

best6 INTEGER (0..209)

}

BestSubbandForTwentyM ::= CHOICE {

best1 INTEGER (0..20),

best6 INTEGER (0..54263),

best12 INTEGER (0.. 293929)

}

StcRateSet ::= CHOICE {

maxMt2 BIT STRING (SIZE(1)),

maxMt3Or4 BIT STRING (SIZE(2)),

maxMtMoreThan4 BIT STRING (SIZE(3))

}

SubbandCqiSetChoice ::= CHOICE {

best1SubbandCqi INTEGER (0..15),

-- For best 6 or full feedback for 512 FFT belongs here

best6SubbandCqiSet SEQUENCE (SIZE (1..6)) OF INTEGER (0..15),

-- For best 12 or full feedback for 1024 FFT belongs here

best12SubbandCqiSet SEQUENCE (SIZE (1..12)) OF INTEGER (0..15),

-- full feedback for 2048 FFT belongs here

best21SubbandCqiSet SEQUENCE (SIZE (1..21)) OF INTEGER (0..15)

}

SubbandPmiSetChoice ::= CHOICE {

best1SubbandPmi PmiSet,

-- For best 6 or full feedback for 512 FFT belongs here

best6SubbandPmiSet SEQUENCE (SIZE (1..6)) OF PmiSet,

-- For best 12 or full feedback for 1024 FFT belongs here

best12SubbandPmiSet SEQUENCE (SIZE (1..12)) OF PmiSet,

-- full feedback for 2048 FFT belongs here

best21SubbandPmiSet SEQUENCE (SIZE (1..21)) OF PmiSet

}

SubbandStreamSetChoice ::= CHOICE {

best1SubbandStream StreamSet,

-- For best 6 or full feedback for 512 FFT belongs here

best6SubbandStreamSet SEQUENCE (SIZE (1..6)) OF StreamSet,

-- For best 12 or full feedback for 1024 FFT belongs here

best12SubbandStreamSet SEQUENCE (SIZE (1..12)) OF StreamSet,

-- full feedback for 2048 FFT belongs here

best21SubbandStreamSet SEQUENCE (SIZE (1..21)) OF StreamSet

}

StreamSet ::= CHOICE {

mmi0MaxMt2 BIT STRING (SIZE(1)),

mmi0MaxMt3Or4 BIT STRING (SIZE(2)),

mmi1 BIT STRING (SIZE(1))

}

PmiSet ::= CHOICE {

nt2 BIT STRING (SIZE(3)),

nt4CodeBook0 BIT STRING (SIZE(6)),

nt4CodeBook1 BIT STRING (SIZE(4)),

nt8 BIT STRING (SIZE(4))

}

-- Number of reports are same as N_multiBS_reports transmitted in

-- Feedback_Polling_A-MAP IE

MultiBsRpt ::= SEQUENCE {

pmi CHOICE {

nt2 BIT STRING (SIZE(3)),

nt4 BIT STRING (SIZE(4)),

nt8 BIT STRING (SIZE(4))

},

ictRelatedReport CHOICE {

ict00or01 Ict00or01Set,

ict10 SEQUENCE {

pmi              INTEGER (0..7)

},

ict11 SEQUENCE {

pmi                  INTEGER (0..7),

relativeAmp          INTEGER (0..15) -- (-1*relativeAmp)dB

}

}

}

Ict00or01Set ::= SEQUENCE {

tempBsid INTEGER (0..15),

measurementMetric MeasurementMetric,

pmiSubsetSize PmiSubsetSize,

pmiCoordinationSubset PMICoordinationSubset OPTIONAL

}

PMICoordinationSubset ::= ENUMERATED {

levelN1,

levelN2

}

MeasurementMetric ::= ENUMERATED {

dot25dB,

dot50dB,

onedB,

greaterThan1dot5dB

}

PmiSubsetSize ::= ENUMERATED {

pmi,

multiplePmi

}

MultiBSMIMORequestInfo ::= CHOICE {

singleBSPrecoding SEQUENCE {

nipValueForSingleBS NipvalueforsingleBS,

tempBSID SEQUENCE (SIZE (1..8)) OF INTEGER (0..15)

},

multiBSJointMIMOProcessing SEQUENCE {

bitmapForRequestedAdjABSs BIT STRING (SIZE(8)),

nipValueForJoint Nipvalueforjoint

}

}

NipvalueforsingleBS ::= ENUMERATED {

nipth1,

nipth1plusdot5dB,

nipth1plusonedB,

nipth1plusgreaterThan1dot5dB

}

Nipvalueforjoint ::= ENUMERATED {

nipth2,

nipth2plusdot5dB,

nipth2plusonedB,

nipth2plusgreaterThan1dot5dB

}

PmiCombinationRatio ::= ENUMERATED {

dot125,

dot375,

dot625,

dot875

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Single-BS MIMO Feedback

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SBS-MIMO-FBK ::= SEQUENCE {

matrix CHOICE {

matrix2X2 SEQUENCE (SIZE(2)) OF RowOf2X2Matrix,

matrix4X4 SEQUENCE (SIZE(4)) OF RowOf4X4Matrix,

matrix8X8 SEQUENCE (SIZE(8)) OF RowOf8X8Matrix

} OPTIONAL,

mfmBitmap MfmBitmap,

mfm0Info Mfm0InfoSet OPTIONAL,

mfm1Info Mfm1InfoSet OPTIONAL,

mfm2Info Mfm2InfoSet OPTIONAL,

mfm3Info Mfm3InfoSet OPTIONAL,

mfm4Info Mfm4InfoSet OPTIONAL,

mfm5Info Mfm5InfoSet OPTIONAL,

mfm6Info Mfm6InfoSet OPTIONAL,

mfm7Info Mfm7InfoSet OPTIONAL

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multiple-BS MIMO Feedback

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MBS-MIMO-FBK ::= SEQUENCE {

ictInfo                        CHOICE {

ict0b10                      SEQUENCE {

cqi INTEGER (0..15)},

ict0b11                      SEQUENCE {

maxAmpBs BOOLEAN OPTIONAL,

-- Present if S-ABS does not have the maximum channel amplitude

-- among collaborative ABS

maxAmpBsIndex INTEGER (0..7) OPTIONAL,

-- Present if S-ABS does not have the maximum channel amplitude

-- (relativeServingBs+1)dB

relativeServingBs INTEGER (0..7) OPTIONAL}

},

multiBsReport SEQUENCE (SIZE(1..8)) OF MultiBsRpt

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multi-BS MIMO Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MBS-MIMO-REQ ::= SEQUENCE {

multiBSMIMORequest MultiBSMIMORequestInfo,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multi-BS MIMO Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MBS-MIMO-RSP ::= SEQUENCE {

adjABSBitmapMultiBSMIMO BIT STRING (SIZE(8)),

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multi-BS MIMO SBP Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MBS-MIMO-SBP ::= SEQUENCE {

nbrInfoArray SEQUENCE (SIZE(1..8)) OF SEQUENCE {

pmiMin BIT STRING (SIZE(4..6)),

isl ENUMERATED {

one,

two,

three,

four

}

},

pcr PmiCombinationRatio,

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Multi-BS Sounding Calibration

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-MBS-SOUNDING-CAL ::= SEQUENCE {

superframe ENUMERATED {

two,

four,

six,

eight

},

frame INTEGER (0..3),

soundingAAISubframe INTEGER (0..7),

soundingSubbandBitmap BIT STRING (SIZE(6..24)),

decimationOffset INTEGER (0..63),

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------