| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-L2-XFER ASN.1 code |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-L2-XFER ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-L2-XFER ASN.1 code

Joey Chou

Intel

Introduction

This contribution proposes changes to AAI-L2-XFER ASN.1 code

Proposed Text

Remedy 1:

In the P802.16m/D9, page1080, line 63, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AAI-L2-XFER ::= SEQUENCE {

transferType                   INTEGER {

gnssAssistance (1),

lbsMeasurement (2),

deviceBootstrap (3),

wirelessMAN (4),

oratMsg (5),

sms (6),

mihFrame (7),

relaySupport (8),

emergencyAlert (9)

-- values 10 to 127 are reserved

-- values 128 to 255 are for

-- vendor-specific types

} (0..255),

transferSubtype                CHOICE {

oratMsg                      INTEGER {

geran (1),

utran (2),

e-utran (3),

td-scdma (4),

cdma2000 (5)} (0..15),

sms                          INTEGER {

smsData (1),

smsConfirmation (2)} (0..15),

mihFrame                     INTEGER {

serviceManagement (1),

eventService (2),

commandService (3),

informationService (4)} (0..15)

} OPTIONAL,

payload                        OCTET STRING (SIZE(1..9999)) OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------