| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for Vendor ID and Vendor-Specific Information (16.2.12.12) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Hyunjeong Kang, Youngkyo Baek, Jaejeong Shim Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to clarify Vendor ID and Vendor-specific information in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for Vendor ID and Vendor-Specific Information

Hyunjeong Kang, Youngkyo Baek, Jaejeong Shim

Samsung Electronics

- Introduction

Vendor ID, Vendor-specific classification parameters, Vendor-specific QoS parameters, and Vendor-specific PHS parameters are present in AAI-DSx message, but the specific information on the parameters is missing in 16m/D9.

In addition, as legacy standard (i.e., 802.16e and 802.16-2009), 802.16m should allow for a vendor to use any MAC control message with vendor-specific information.

Therefore we propose the usage of Vendor ID and Vendor-specific information for 802.16m system similar to that described in section 11.1.5, 11.1.6 and 11.13.9.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Add a new subclause on line 6 of page 429 as follows:]

16.2.12.12.7 Vendor-specific information

Vendor-specific information for AMSs, if present, shall be encoded in the vendor-specific information field (VSIF) using the Vendor ID field to specify which tuples apply to which vendor’s products. The Vendor ID shall be the first parameter embedded inside VSIF. If the first parameter inside VSIF is not a Vendor ID, then the VSIF parameters shall be discarded. When used as a subfield of Vendor-specific information i.e. Vendor-specific classification parameters, Vendor-specific PHS parameters, Vendor-specific QoS parameters, the Vendor ID identifies the Vendor ID of the AMSs that are intended to use this information.

This configuration setting may appear multiple times. The same Vendor ID may appear multiple times. This configuration setting may be nested inside a Packet classification configuration setting, a Service flow configuration setting. However, there shall not be more than one Vendor ID inside a single VSIF.

Example: Configuration with vendor-A-specific fields and vendor-B-specific fields:

(VSIF #1)

Vendor ID of Vendor A

Vendor A Specific parameter #1

Vendor A Specific parameter #2

(VSIF #2)

Vendor ID of Vendor B

Vendor B Specific parameter #1

Vendor B Specific parameter #2

NOTE - The Vendor-specific information may present in any MAC control messages based on vendor-specific function.

16.2.12.12.7.1 Vendor-specific classification parameters

This allows vendors to encode vendor-specific classification rule parameters. The Vendor ID shall be the first parameter embedded inside vendor-specific classification rule parameters. If the first parameter inside vendor specific classification rule parameters is not a Vendor ID, then the Vendor-specific classification rule parameters shall be discarded

16.2.12.12.7.2 Vendor-specific PHS parameters

This allows vendors to encode vendor-specific PHS parameters. The Vendor ID shall be the first parameter embedded inside vendor-specific PHS parameters. If the first parameter inside vendor-specific PHS parameters is not a Vendor ID, then the Vendor-specific PHS parameters shall be discarded.

16.2.12.12.7.3 Vendor-specific QoS parameters

This allows vendors to encode vendor-specific QoS parameters. The Vendor ID shall be the first parameter embedded inside vendor-specific QoS parameters. If the first parameter inside vendor-specific QoS parameters is not a Vendor ID, then the Vendor-specific QoS parameters shall be discarded

[Remedy 2:]

[Insert a reference subclause for Vendor ID on line 21 of page 227 (Table 734) as follows:]

| If (Vendor-specific classification parameters are needed) { |  | See 16.2.12.12.7.1 |  |
| --- | --- | --- | --- |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the AMS or ABS MAC address. |  |
| } |  |  |  |

[Remedy 3:]

[Insert a reference subclause for Vendor-specific PHS parameters on line 26 of page 228 (Table 737) as follows:]

| Vendor-specific PHS parameters | variable | See 16.2.12.12.7.2 | Present if needed |
| --- | --- | --- | --- |

[Remedy 4:]

[Insert the following texts for Vendor ID on line 23 of page 244 (Table 737):]

| If (Vendor-specific classification parameters are needed) { |  | See 16.2.12.12.7.1 |  |
| --- | --- | --- | --- |
| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the AMS or ABS MAC address. |  |
| } |  |  |  |

[Remedy 5:]

[Insert a reference subclause for Vendor-specific PHS parameters on line 11 of page 245 (Table 737) as follows:]

| Vendor-specific PHS parameters | variable | See 16.2.12.12.7.2 | Present if needed |
| --- | --- | --- | --- |

[Remedy 6:]

[Change the Vendor-specific QoS Parameter on line 26 of page 420 (Table 781) as follows:]

| Vendor-specific QoS Parameter | variable | Refer to 16.2.12.12.7.3 11.13.9 for the usage of this parameter 3byte-Vendor ID shall be the first information of this parameter |
| --- | --- | --- |

[Remedy 7:]

[Add a new parameter after line 26 of page 109 (Table 682) as follows:]

| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the AMS or ABS MAC address. | Present to indicate the vendor ID of the AMS sending this message if vendor specific information is used |
| --- | --- | --- | --- |

[Add a new parameter after line 43 of page 112 (Table 683) as follows:]

| Vendor ID | 24 | Vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the AMS or ABS MAC address. | Present to indicate the vendor ID of the AMS sending this message if vendor specific information is used |
| --- | --- | --- | --- |

[Insert the followings after line 33 of page 998 (Annex R.2):]

vendorID OCTET STRING (SIZE(3)) OPTIONAL,

[Insert the followings after line 60 of page 1000 (Annex R.2):]

vendorID OCTET STRING (SIZE(3)) OPTIONAL,

-------------------------------  Text End  ---------------------------------------------------