| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for common MAC in multicarrier operation (16.2.8.1) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Hyunjeong Kang, Jaeweon Cho, Ji-Yun Seol Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to remove a term ‘common MAC’ from 16m draft. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for common MAC in multicarrier operation

Hyunjeong Kang, Jaeweon Cho, Ji-Yun Seol

Samsung Electronics

- Introduction

The basic assumption on 802.16m multi-carrier is that AMS’s MAC context, operation, state and so on are created and handled as one AMS regardless of number of carriers. The expression ‘Common MAC’ has been used to present this assumption, but it is not clear how ‘Common MAC’ is working in an ABS and an AMS.

The expression ‘Common MAC’ rather gives confusion because there is no criteria to identify which function should be under ‘Common MAC’ and which function should not be under ‘Common MAC’.

Without the expression ‘Common MAC’, the multi-carrier operation can be described to present the basic assumption. Therefore, we propose to delete the expression ‘Common MAC’ from the draft.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Modify the text on line 4 of page 370 as follows:]

When supporting multicarrier operations, an common MAC in ABS may utilize the radio resources in the primary carrier and one or more of the secondary carriers.

[Remedy 2:]

[Modify the text on line 58 of page 374 as follows:]

The construction and transmission of MAC PDU is the same as that in single carrier operation. For each service flow the ARQ operates for a common MAC as defined in 16.2.13.

[Remedy 3:]

[Modify the text on line 24 of page 375 as follows:]

The STID and all FIDs assigned to an AMS are unique identifiers for a common MAC and which used over all the carriers of the AMS. The service setup/change messages (i.e., DSx messages) are transmitted only through the AMS's primary carrier. The service flow is defined for an AMS a common MAC entity and AMS's QoS context represented by an SFID is applicable across primary carrier and secondary carrier(s) and collectively applied to all carriers of the AMS.

[Remedy 4:]

[Modify the text on line 26 of page 387 as follows:]

Given that a common MAC manages both serving and target primary carriers, The network reentry procedures at the target primary carrier is not required.

[Remedy 5:]

[Delete the ‘Common MAC’ box under AMS and the ‘Common MAC’ box under ABS in Figure 426 (line 6, page 388):]

[Remedy 6:]

[Delete the ‘Common MAC’ box under ABS in Figure 427 (line 36, page 388):]

-------------------------------  Text End  ---------------------------------------------------