| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for Group Parameter Create/Change (16.2.12.12) |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | Hyunjeong Kang, Jungshin Park, Jaejeong Shim Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to clarify Group Parameter Create/Change and Coupled Group Create/Change in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for Group Parameter Create/Change

Hyunjeong Kang, Jungshin Park, Jaejeong Shim

Samsung Electronics

- Introduction

Group parameter create/change and Coupled create/change parameter are used to set up or update more than one connections through an AAI-DSA/DSC transaction. In each parameter, common or non-common service flow encodings is defined, but how to use the common or non-common service flow encodings is not clear. Therefore we propose the details on the common/non-common parameters for Group parameter create/change and Coupled create/change parameter in this contribution.

In addition, missing information from Service flow/convergence sublayer parameters (Table 781) is proposed.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Change the parameters on line 22 of page 425 to line 40 of page 426 (Table 781) as follows:]

| Group Parameter Create/Change { | variable | Only one instance of the Group Parameter Create/Change may be included in AAI-DSA or AAI-DSC message. |
| --- | --- | --- |
| Common for Group Create/Change | variable | Common service flow encodings that are common to all flows specified in Group Parameter Create/Change. All the rules and settings that apply to the service flow encodings when used in an AAI-DSA or AAI-DSC message apply to the contents encapsulated in this parameter. Except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, Service flow/convergence sublayer parameters in Table 781 may be encapsulated in this field. Shall be included only once in a Group Parameter Create/Change as the first attribute. |
| Qty SFID Request | M5 | Qty SFID Request is the quantity of service flows of the same common parameter set configuration. Shall only be sent by the AMS as the last attribute of Group Parameter Create/Change in AAI-DSA-REQ or AAI-DSC-REQ. |
| Group FID List | N x 4 | List of N FIDs The maximum value of N is 32. |
| Group FID Paraemeter List { | variable | Encapsulates an FID of an service flow and associated non-common service flow encodings for that service flow, specified in this Group Parameter Create/Change. All the rules and settings that apply to the service flow encodings when used in an AAI-DSA or AAI-DSC message apply to the contents encapsulated in this parameter. If included, shall be the last attribute of Group Parameter Create/Change. The maximum number of Group FID Parameter Lists is 32. |
| FID | 4 | FID of a service flow in the Group FID Parameter List. |
| Non-Common for Group Create/Change | variable | Encapsulates the non-common service flow encodings that are specific to individual service flows specified in the Group FID Parameter List. All the rules and settings that apply to the service flow encodings when used in an AAI-DSA or AAI-DSC message apply to the contents encapsulated in this parameter. Except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, Service flow/convergence sublayer parameters in Table 781 may be encapsulated in this field. |
| } |  |  |
| } |  |  |
| Coupled Group Create/Change { | variable | This parameter encapsulates service flow encodings for Coupled UL and DL unicast service flows in an AAI-DSA or AAI-DSC message. |
| Common for Coupled Group | variable | The common service flow encodings that are common to the Coupled UL and DL unicast service flows of an AMS in this Coupled Parameter Group Create/Change parameter. Except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, Service flow/convergence sublayer parameters in Table 781 may be encapsulated in this field. If included, this parameter shall be the first attribute of Coupled Group Create/Change parameter. |
| Qty Coupled SFID request | M5 | Qty Coupled SFID request is the quantity of Coupled UL and DL unicast service flows of an AMS, of the same common parameter set configuration. Used only as the last attribute of a Coupled Parameter Group Create/Change in an AAI-DSA-REQ or an AAI-DSC-REQ. |
| Coupled FID List | N x 8 | N x (4 MSB for DL FID and 4 LSB for UL FID) The maximum number of N is 16. |
| Coupled FID Parameter List { | variable | Encapsulates an FID of an service flow and associated non-common service flow encodings for that service flow, specified in this Coupled Parameter Group Create/Change. All the rules and settings that apply to the service flow encodings when used in an AAI-DSA or AAI-DSC message apply to the contents encapsulated in this parameter. If included, shall be the last attribute of Coupled¦ Parameter Create/Change. The maximum number of Coupled FID Parameter Lists is 32. |
| FID | 4 | FID of a service flow in the Coupled FID Parameter List. |
| Non-Common for Coupled Group | variable | Encapsulates the non-common service flow encodings that are specific to individual service flows specified in the Coupled FID Parameter List. All the rules and settings that apply to the service flow encodings when used in an AAI-DSA or AAI-DSC message apply to the contents encapsulated in this parameter. Except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, Service flow/convergence sublayer parameters in Table 781 may be encapsulated in this field. |
| } |  |  |
| } |  |  |

[Remedy 2:]

[Modify the texts on line 52 of page 230 to line 60 of page 231 (Table 734) as follows:]

| If (Group Parameter Create/Change is needed) { |  | Refer to 16.2.12.128.6 |  |
| --- | --- | --- | --- |
| Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. Shall be included only once in a Group Parameter Create/Change as the first attribute. |  |
| Qty SFID request | 58 | The number of service flows to create | Shall only be sent by an AMS as the last attribute of Group Parameter Create/Change |
| For(i=0; i<N-FIDs; i++) { |  | FIDs of service flows created with this Group Parameter Create/Change N-FIDs is the number of FIDs The maximum value of N-FIDs is 32. |  |
| FID | 4 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Noncommon; i++) { |  | N-FIDs-Noncommon is the number of non-common service flow IDs The maximum value of N-FIDs-Noncommon is 32. |  |
| FID | 4 |  |  |
| Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in N-FIDs-Noncommon Group FID Parameter List Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. |  |
| } |  |  |  |
| } |  |  |  |
| If (Coupled Group Create/Change is needed) { |  | Refer to 16.2.12.12 |  |
| Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. This parameter shall be the first attribute of Coupled Group Create/Change. |  |
| Qty Coupled SFID request | 58 | The number of coupled DL service flow and UL service flow of the same common parameter set configuration to create This parameter is the last attribute of a Coupled Group Create/Change |  |
| For(i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of DL and UL service flow (4 MSB for DL FID and 4 LSB for UL FID) N-FIDs-Coupled is the number of FIDs The maximum value of N-FIDs-Coupled is 16. |  |
| FID | 8 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs The maximum value of N-FIDs-Coupled-Noncommon is 32. |  |
| FID | 4 |  |  |
| Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. |  |
| } |  |  |  |
| } |  |  |  |

[Remedy 3:]

[Modify the texts on line 49 of page 247 to line 55 of page 248 (Table 737) as follows:]

| If (Group Parameter Create/Change is needed) { |  | Refer to 16.2.12.128.6 |  |
| --- | --- | --- | --- |
| Common for Group Create/Change | variable | Common service flow encodings that are common to all service flows specified in Group Parameter Create/Change. Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. Shall be included only once in a Group Parameter Create/Change as the first attribute. |  |
| Qty SFID request | 58 | The number of service flows to change Shall be the last attribute of Group Parameter Create/Change | Shall only be sent by an AMS |
| For(i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of service flows created with this Group Parameter Create/Change N-FIDs is the number of FIDs The maximum value of N-FIDs is 32. |  |
| FID | 4 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Noncommon; i++) { |  | N-FIDs-Noncommon is the number of non-common service flow IDs The maximum value of N-FIDs-Noncommon is 32. |  |
| FID | 4 |  |  |
| Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. |  |
| } |  |  |  |
| } |  |  |  |
| If (Coupled Group Create/Change is needed) { |  | Refer to 16.2.12.12 |  |
| Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. This parameter shall be the first attribute of Coupled Group Create/Change parameter. |  |
| Qty Coupled SFID request | 58 | The number of coupled DL service flow and UL service flow of the same common parameter set configuration to change This parameter is the last attribute of a Coupled Group Create/Change |  |
| For(i=0; i<N-FIDs-Coupled; i++) { |  | FIDs of DL and UL service flow 4 MSB for DL FID and 4 LSB for UL FID N-FIDs-Coupled is the number of FIDs The maximum value of N-FIDs-Coupled is 16. |  |
| FID | 8 |  |  |
| } |  |  |  |
| For(i=0; i<N-FIDs-Coupled-Noncommon; i++) { |  | N-FIDs-Coupled-Noncommon is the number of non-common coupled service flow IDs The maximum value of N-FIDs-Coupled-Noncommon is 32. |  |
| FID | 4 | Flow identifier |  |
| Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List Service flow/convergence sublayer parameters in Table 781, except FID, SFID, E-MBS service related information, Group Parameter Create/Change related information and Coupled Group Create/Change related information, may be encapsulated in this field. |  |
| } |  |  |  |
| } |  |  |  |

[Remedy 4:]

[Add the following parameters after line 21 of page 425 (Table 781):]

| SDU Size | 8 | Number of bytes Default=49 |
| --- | --- | --- |
| Non_ARQ_reordering_timeout | 6 | 0 > and <= 32. Unit is PHY frame (5ms) |
| Classification Rule Priority | 8 | //packet classification rule parameter 0-255 |
| Protocol field | 8 | //packet classification rule parameter Protocol |
| IP Masked Source Address | 8 (IPv4) or 32 (IPv6) | //packet classification rule parameter src, smask |
| IP Masked Destination Address | 8 (IPv4) or 32 (IPv6) | //packet classification rule parameter dst, dmask |
| Protocol Source Port Range | 32 | //packet classification rule parameter sportlow, sporthigh |
| Protocol Destination Port Range field | 32 | //packet classification rule parameter dportlow, dporthigh |
| Associated PHSI | 8 | //packet classification rule parameter Index value |
| Packet Classification Rule Index | 16 | //packet classification rule parameter Packet Classification Rule Index |
| IPv6 Flow Label field | 24 | //packet classification rule parameter Flow Label |
| Classification Action Rule | 8 | //packet classification rule parameter Bit 0: 0 = none. 1 = Discard packet Bit 1-7: Reserved. |
| IP Type of Service | 8 | //packet classification rule parameter Bit 0-Bit 1: reserved. Shall be set to 0b00 Bit 2-Bit 7: DSCP value |
| PHS DSC Action field | 2 | //packet classification rule parameter 0: Add PHS Rule 1: Set PHS Rule 2: Delete PHS Rule 3: Delete all PHS Rules |
| PHSI field | 8 | //packet classification rule parameter Index value |
| PHSS field | 8 | //packet classification rule parameter Number of bytes in the suppression string |
| PHSF field | PHSS / 8 | //packet classification rule parameter String of bytes suppressed |
| PHSM field | PHSS / 8 | //packet classification rule parameter Bit 0: 0 = Don't suppress first byte of the suppression field 1 = Suppress first byte of the suppression field Bit 1: 0 = Don't suppress second byte of the suppression field 1 = Suppress second byte of the suppression field Bit x: 0 = Don't suppress (x + 1) byte of the suppression field 1 = Suppress (x + 1) byte of the suppression field |
| PHSV field | 1 | //packet classification rule parameter 0: Verify 1: Don't verify |
| Classifier DSC Action | 8 | 0: DSC Add classifier 1: DSC Release classifier 2: DSC Delete classifier |
| IEEE 802.3/Ethernet Destination MAC Address parameter | 12 | dst.msk |
| IEEE 802.3/Ethernet Source MAC Address parameter | 12 | src, msk |
| Ethertype/IEEE 802.2 SAP | 3 | type, eprot1, eprot2 |
| IEEE 802.1D User Priority field | 2 | pri-low, pri-high Valid range: 0-7 for pri-low and pri-high |
| IEEE 802.1Q VLAN ID field | 2 | vlan_id1, vlan_id2 |

[Remedy 5:]

[Modify the ASN.1 code in Annex R.2 (page 1014, line 55) as follows:]

groupParameterCreateChange SEQUENCE {

-- ??? this part of the table is obscure

commonQosParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL

} OPTIONAL,

qtySfid INTEGER (10..32255) OPTIONAL,

groupFidArray SEQUENCE (SIZE(1..32)) OF SEQUENCE {

FID

} OPTIONAL,

fidArray SEQUENCE (SIZE(1..3216)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL

} OPTIONAL

} OPTIONAL,

coupledGroupCreateChange SEQUENCE {

-- ??? this part of the table is obscure

commonQoSParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL

} OPTIONAL,

qtyCoupledSfid INTEGER (10..32255) OPTIONAL,

coupledFidArray SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid INTEGER (0..25515) OPTIONAL} OPTIONAL,

couplednonCommonFidArray SEQUENCE (SIZE(1..3216)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL

} OPTIONAL

} OPTIONAL,

[Remedy 6:]

[Modify the ASN.1 code in Annex R.2 (page 1017, line 55) as follows:]

groupParameterCreateChange SEQUENCE {

-- ???? this part of the table is obscure

commonQosParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL,

ClassifierDSCAction OPTIONAL

} OPTIONAL,

qtySfid INTEGER (10..32255) OPTIONAL,

groupFidArray SEQUENCE (SIZE(1..32)) OF SEQUENCE {

FID

} OPTIONAL,

fidArray SEQUENCE (SIZE(1..3216)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL,

ClassifierDSCAction OPTIONAL

} OPTIONAL

} OPTIONAL,

coupledGroupCreateChange SEQUENCE {

-- ??? this part of the table is obscure

commonQoSParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL,

ClassifierDSCAction OPTIONAL

} OPTIONAL,

qtyCoupledSfid INTEGER (10..32255) OPTIONAL,

coupledFidArray SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid INTEGER (0..25515) OPTIONAL} OPTIONAL,

couplednonCommonFidArray SEQUENCE (SIZE(1..3216)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters SEQUENCE {

QosParameter OPTIONAL,

QosParameterSetType OPTIONAL,

ServiceClassName OPTIONAL,

GlobalServiceClassName OPTIONAL,

CsSpecification OPTIONAL,

DataDelivryServices OPTIONAL,

ArqAttribute OPTIONAL,

ClassificationRule OPTIONAL,

EthernetAttributes OPTIONAL,

RohcAttributes OPTIONAL,

sduInterArrival Interval OPTIONAL,

timeBase Interval OPTIONAL,

sduSize INTEGER (0..255) OPTIONAL,

targetSaid INTEGER (0..255) OPTIONAL,

nonArqReorderTimeout INTEGER (1..32) OPTIONAL,

PacketErrorRate OPTIONAL,

macHeaderType ENUMBERATED {

agmh,

spmh

} OPTIONAL,

ClassifierDSCAction OPTIONAL

} OPTIONAL

} OPTIONAL,

-------------------------------  Text End  ---------------------------------------------------