| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for AAI-DREG-RSP with REQ-Duration (16.2.3.22) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Rakesh Taori Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to clarify REQ-Duration parameter in AAI-DREG-RSP with Action code 0x06. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for AAI-DREG-RSP with REQ-Duration

Hyunjeong Kang, Yeongmoon Son, Rakesh Taori

Samsung Electronics

- Introduction

According to the texts on line 1 of page 477, when an ABS decides to reject AMS-initiated idle mode request and sends an AAI-DREG-RSP as a response to an AAI-DREG-REQ message, the ABS may include REQ-Duration in the AAI-DREG-RSP message. But this scenario is not specified in the AAI-DREG-RSP message with Action code=0x06.

Therefore, this contribution proposes to include REQ-Duration parameter in an AAI-DREG-RSP with Action code = 0x06 as an optional field.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Add the followings on line 43 of page 166 (before If (Action code==0x07):]

| If (Action code == 0x06) { |  |  |  |
| --- | --- | --- | --- |
| REQ-Duration | 8 | Used to indicate waiting value for the AAI-DREG-REQ message with De-registration_Request_Code=0x01 0..28-1: measured in frames | May be present |
| } |  |  |  |

[Remedy 2:]

[Modify the ASN.1 code in Annex R.2 (page 1003, line 44) as follows:]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- DeRegistration Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-DREG-RSP ::= SEQUENCE {

actionCode CHOICE {

attempyNewNtwkEntry NULL,			-- 0x00

listenAndNoTx NULL,				-- 0x01

listenAndTxOnControlConnection NULL,		-- 0x02

returnAndTxOnActiveConnection NULL,		-- 0x03

amsTerminateNormalOperation NULL,		-- 0x04

initiateIdleMode InitiateIdleMode,		-- 0x05

rejectIdleModeInitiationRequest RejectIdleModeInitiationRequestNULL, -- 0x06

allowIdleModeInitiationRequest AllowIdleModeInitiationRequest,	--0x07

allowConnectionInfoRetention NULL,		-- 0x08

rejectConnectionInfoRetention NULL,		-- 0x09

...

}

}

InitiateIdleMode ::= SEQUENCE {

pagingCycle PgCycle,

pagingOffset PgOffset,

pagingControllerId PCID,

pagingGroupId PGID,

-- if Network Configuration indicates ABS is attached to the advanced network

deRegId DREGID OPTIONAL,

idleModeRetainInfo IdleModeRetain,

reqDuration INTEGER (0 .. 255)	OPTIONAL

}

RejectIdleModeInitiationRequest ::= SEQUENCE {

reqDuration INTEGER (0 .. 255)	OPTIONAL

}

AllowIdleModeInitiationRequest ::= SEQUENCE {

pagingCycle PgCycle,

pagingOffset PgOffset,

pagingControllerId PCID,

pagingGroupId PGID,

-- if Network Configuration indicates ABS is attached to the advanced network

deRegId DREGID OPTIONAL,

idleModeRetainInfo IdleModeRetain

}

-------------------------------  Text End  ---------------------------------------------------