| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes on AMS Maximum Power Reporting for Multicarrier |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Rongzhen Yang, Xiangying Yang, Hujun Yin Intel Corp | Rongzhen.Yang@intel.com |
| Re: | P802.16m/D9 SB |  |
| Abstract | Modifications are proposed for AMS Maximum Power Reporting for Multicarrier |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 10) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes on Uplink Power Status Reporting Header for Multicarrier Enhancement

Rongzhen Yang, Xiangying Yang, Hujun Yin

Intel Corp

- Introduction

The PSR (Power Status Reporting) Header defined in current IEEE 802.16m D8 is sent by AMS to ABS to provide very important reference information for ABS uplink resource scheduling: When ABS receives the PSR, the transmission PSD of each channel in AMS can be estimated in ABS side, and then ABS can compare the AMS power consumption with AMS maximum transmission power, to check the reasonability of AMS uplink resource allocation size for avoiding AMS power shortage.

It means that the AMS maximum power needs to be transmitted to ABS by AMS as one key reference information. In the contribution C802.16m-10/1172 at session 69, it already proposed:

- Maximum Tx Power reported by AMS AAI-SBC-REQ message should be changed from optional to mandatory;

- The detail of Maximum Tx Power expression is defined;

The proposed changes in C802.16m-10/1172 are reasonable to support single carrier case. However, we need to continue enhance this topic for multicarrier case to answer below questions:

- The Maximum Tx Power defined in AAI-SBC-REQ is for single carrier or for all carriers?

- How ABS to know the maximum Tx power of each carrier in multi-carrier case?

In this contribution, we enhance the current D9 text for better multicarrier supporting on this topic.

- References

[1] Draft Amendment, P802.16m/D9, Oct. 2010.

- Text proposal for inclusion in the 802.16m amendment

Instruction to editor;

Black text: existing D9 text

Red and strike out text: for deletion

Blue and underlined text: for addition

[Remedy 1:]

[Change the text from line 9 on the page 838, section 16.3.8.4 as follows:]

============================== Start of Proposed Text =================================

An AMS shall report the maximum available power of primary carrier by MAX Tx Power field when the AMS transmits AAI-SBC-REQ. , and shall report the maximum available power of each activated carrier when the AMS send AAI-CM-IND.

For interference level control, current interface level of each ABS may be shared among ABSs.

============================== End of Proposed Text =================================

[Remedy 2:]

[Change the text from line 9 on the page 109, section 16.2.3.4 as follows:]

============================== Start of Proposed Text =================================

| M | MAX Tx Power | 24 | The maximum available power of primary carrier: Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. Each unsigned 8-bit integer specifies the maximum transmitted power value in dBm. The maximum transmitted power is quantized in 0.5 dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. If AMS does not support 64-QAM, the AMS shall report the value of 0x00 for Bit 15-23. |  |
| --- | --- | --- | --- | --- |

============================== End of Proposed Text =================================

[Remedy 3:]

[Change the text from line 3 on the page 271, section 16.2.3.54 as follows:]

============================== Start of Proposed Text =================================

Table 748 – AAI-CM-CMDIND Message Field Description

| Field | Size(bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action Code | 1 | Used to indicate the purpose of this message 0b0: secondary carrier management 0b1: primary carrier change | Mandatory |
| MAX Tx Power | 24 | The maximum available power of activated carrier: Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. Each unsigned 8-bit integer specifies the maximum transmitted power value in dBm. The maximum transmitted power is quantized in 0.5 dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. If AMS does not support 64-QAM, the AMS shall report the value of 0x00 for Bit 15-23. | Mandatory |

============================== End of Proposed Text =================================