| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes on AMS Maximum Power Reporting for Multicarrier |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Rongzhen Yang, Xiangying Yang, Hujun Yin Intel Corp Jeongho Park Samsung Electronics Pei-Kai Liao MediaTek Inc | Rongzhen.Yang@intel.com |
| Re: | P802.16m/D9 SB |  |
| Abstract | Modifications are proposed for AMS Maximum Power Reporting for Multicarrier |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 10) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes on Uplink Power Status Reporting Header for Multicarrier Enhancement

Rongzhen Yang, Xiangying Yang, Hujun Yin

Intel Corp

Jeongho Park

Samsung Electronics

Pei-Kai Liao

MediaTek Inc

- Introduction

The PSR (Power Status Reporting) Header defined in current IEEE 802.16m D8 is sent by AMS to ABS to provide very important reference information for ABS uplink resource scheduling: When ABS receives the PSR, the transmission PSD of each channel in AMS can be estimated in ABS side, and then ABS can compare the AMS power consumption with AMS maximum transmission power, to check the reasonability of AMS uplink resource allocation size for avoiding AMS power shortage.

It means that the AMS maximum power needs to be transmitted to ABS by AMS as one key reference information. In the contribution C802.16m-10/1172 at session 69, it already proposed:

- Maximum Tx Power reported by AMS AAI-SBC-REQ message should be changed from optional to mandatory;

- The detail of Maximum Tx Power expression is defined;

The proposed changes in C802.16m-10/1172 are reasonable to support single carrier case. However, we need to continue enhance this topic for multicarrier case to answer below questions:

- The Maximum Tx Power defined in AAI-SBC-REQ is for single carrier or for all carriers?

- How ABS to know the maximum Tx power of each carrier in multi-carrier case?

In this contribution, we enhance the current D9 text for better multicarrier supporting on this topic.

- References

[1] Draft Amendment, P802.16m/D9, Oct. 2010.

- Text proposal for inclusion in the 802.16m amendment

Instruction to editor;

Black text: existing D9 text

Red and strike out text: for deletion

Blue and underlined text: for addition

[Remedy 1:]

[Change the text from line 9 on the page 838, section 16.3.8.4 as follows:]

============================== Start of Proposed Text =================================

An AMS shall report the maximum available power of the carrier for initial network entry by MAX Tx Power field when the AMS transmits AAI-SBC-REQ. , and shall report the maximum power of available carriers when the AMS send AAI-MC-REQ.

For interference level control, current interface level of each ABS may be shared among ABSs.

============================== End of Proposed Text =================================

[Remedy 2:]

[Change the text from line 9 on the page 109, section 16.2.3.4 as follows:]

============================== Start of Proposed Text =================================

| M | MAX Tx Power | 24 | The maximum available power of the carrier for initial network entry. Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. Each unsigned 8-bit integer specifies the maximum transmitted power value in dBm. The maximum transmitted power is quantized in 0.5 dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. If AMS does not support 64-QAM, the AMS shall report the value of 0x00 for Bit 15-23. |  |
| --- | --- | --- | --- | --- |

============================== End of Proposed Text =================================

[Remedy 3:]

[Change the text from line 3 on the page 271, section 16.2.3.54 as follows:]

============================== Start of Proposed Text =================================

Table 745—AAI-MC-REQ message format

| Field | Size (bits) | Notes | Condition |
| --- | --- | --- | --- |
| Global Support | 1 | Indicates whether the AMS can simultaneously process all the carriers supported by the S-ABS 0b0: The AMS can only process a subset of the carriers supported by the SABS 0b1: The AMS can simultaneously process all carriers supported by the SABS | Mandatory |
| If (Global_Support==0){ |  |  |  |
| Number of Candidate Combinations (N) | 2 |  | Shall be present if Global_Support is equal to zero |
| For (i=0, i<N; i++) { |  |  |  |
| Number of Candidate Assigned Carrier (Nc) | 3 |  | Shall be present if Global_Support is equal to zero. Note that N cannot be zero. |
| For (j=0, j<Nc; j++) { |  |  |  |
| Physical Carrier Index | 6 | The carriers AMS can simultaneously process | Shall be present if Global_Support is equal to zero. Note that Nc cannot be zero. |
| } |  |  |  |
| DL/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. | Shall be present if Global_Support is equal to zero |
| } |  |  |  |
| } |  |  |  |
| Support of data transmission over guard sub-carrier in between two adjacent carriers that are subcarrier aligned | 1 | 0b0 = not supported 0b1=support | Mandatory |
| for(i =0; i < Num of Power Sharing Groups; i++) { |  | Power Sharing Group is a set of carriers sharing uplink transmission power with a limitation (MAX Tx Power). Maximum value of “Num of Power Sharing Groups” is 16. |  |
| MAX Tx Power | 24 | The maximum available power of one power sharing group: Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. Each unsigned 8-bit integer specifies the maximum transmitted power value in dBm. The maximum transmitted power is quantized in 0.5 dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. If AMS does not support 64-QAM, the AMS shall report the value of 0x00 for Bit 15-23. | Mandatory |
| for(j=0; j< Num of Carriers in one Power Sharing Group; j++) { |  | Maximum value of “Num of Carriers in one Power Sharing Group” is 16; |  |
| Physical Carrier Index | 6 | The relevant carrier belongs to the Power Sharing Group. | Mandatory |
| } |  |  |  |
| } |  |  |  |

If the carriers within a candidate combination are a subset of carriers from one power sharing group, the MAX Tx Power of the power sharing group represents the total maximum transmission power over those carriers.

============================== End of Proposed Text =================================

[Remedy 4:（ASN.1 Coding Changes）]

[Change the text from line 3 on the page 1054, ASN.1 code of AAI-MC-REQ as follows:]

============================== Start of Proposed Text =================================

guardSubcarrierSupport	BOOLEAN,

powerSharingGroupsDescription		SEQUENCE (SIZE(1..16)) OF SEQUENCE {

maxTxPower	SEQUENCE  {

maxTxPowerForQPSK 		INTEGER(0..255),

maxTxPowerFor16QAM 	INTEGER(0..255),

maxTxPowerFor64QAM 	INTEGER(0..255) },

carrersInGroup		SEQUENCE (SIZE(1..16))  OF  PhyCarrierIndex

}

}

============================== End of Proposed Text =================================