| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reduction of transmission of the AAI_TRF-IND message |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Youngsoo Yuk and Jin Sam Kwak LG Electronics | giwon.park@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Reduction of transmission of the AAI_TRF-IND message in 16m/D9. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reduction of transmission of the AAI_TRF-IND message

Giwon Park, Kiseon Ryu, Youngsoo Yuk and Jin Sam Kwak

LG Electronics

Motivation

- Transmission overhead of AAI_TRF-IND message

The operation of sleep cycle resetting in D9:

When the TIMF is set to 1 and if the length of current AMS’s sleep cycle is long enough and the ABS continuously sends the DL traffic to the AMS, the ABS unnecessarily sends the AAI_TRF-IND message to the AMS because AMS resets its sleep cycle as soon as it receives the positive traffic indication. That is, ABS frequently sends the AAI_TRF-IND message to the AMS because of current algorithm of sleep cycle resetting. Figure 1 shows the operation of the sleep cycle resetting in D9.

Figure 1. Sleep cycle reset operation of D9.

- Solution

If the ABS has DL unicast data that will continuously send to the AMS, the ABS may not transmit the AAI_TRF-IND message to the AMS. Because the AMS considers that traffic indication is positive when the AAI-TRF-IND message is lost or not detected by the AMS, but unicast data is received by the AMS. This proposal can reduce the number of AAI_TRF_IND message that ABS transmits. Figure 2 shows if the ABS has DL unicast data that will continuously send to the AMS, the ABS does not transmit the AAI_TRF-IND message to the AMS.

Figure 2. Reduction of transmission of the AAI_TRF-IND message

Text proposal for inclusion in the P802.16m/D9

========================== Start of Proposed Text ====================================================

[Editor’s Note: Modify the following text in “16.2.17.2.3.1Traffic indication” in line 29, page 471]

If the traffic indication is enabled for an AMS by TIMF = 1 in AAI-SLP-REQ/RSP and SLPID is assigned, the ABS shall transmit a traffic indication message or DL unicast data during the Listening Window. If the ABS has successive DL unicast data that will transmit to the AMS beyond the Initial Sleep Cycle, the ABS may transmit the only DL unicast data without AAI_TRF-IND message to the AMS to reduce the overhead of transmission of the AAI_TRF-IND message.

========================== End of Proposed Text =====================================================