| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Error handling of the SLPID update in sleep mode |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Youngsoo Yuk and Jin Sam Kwak LG Electronics Yeongmoon Son Samsung Electronics Jaesun Cha ETRI Shantidev Mohanty Intel Corporation | giwon.park@lge.com ym1004.son@samsung.com jscha@etri.re.kr shantidev.mohanty@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Error handling of the SLPID update in 16m/D9. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Error handling of the SLPID update in sleep mode

Giwon Park, Kiseon Ryu, Youngsoo Yuk and Jin Sam Kwak

LG Electronics

Yeongmoon Son

Samsung Electronics

Jaesun Cha

ETRI

Shantidev Mohanty

Intel Corporation

Motivation

- Loss of the AAI_TRF-IND message including SLPID_update

When the ABS sends the AAI_TRF-IND message including updated SLPID if the AMS didn’t receive the AAI_TRF-IND message, the AMS can’t update the updated own SLPID because the IEEE 802.16m sleep mode does not support the acknowledgement of the AAI_TRF-IND message. In that case, AMS can misunderstand the AAI_TRF-IND message because AMS decides the positive traffic indication based on its SLPID.

Figure 1 shows the example of error case of SLPID update.

Figure 1. Loss of AAI_TRF-IND message including SLPID update

- Solution

When the ABS sends the AAI_TRF-IND message including updated SLPID if the AMS didn’t receive the AAI_TRF-IND message or DL unicast data, AMS sends the AAI_TRF_IND-REQ message. The ABS responds to the AMS by sending AAI-TRF_IND-RSP message including the updated SLPID. Through this solution, although AMS misses the AAI_TRF-IND message including the updated SLPID, it can update the updated own SLPID via AAI_TRF_IND-RSP message.

Figure 2 shows error handling of SLPID update via AAI_TRF_IND-RSP message.

Figure 2. SLPID update in AAI_TRF_IND-RSP message

Text proposal for inclusion in the P802.16m/D9

========================== Start of Proposed Text ==============================

[Editor’s Note: Add the following text in “16.2.17.2.3.1Traffic indication” in line 57, page 471]

SLPID may be updated via the AAI-TRF-IND-RSP message. When the ABS has sent the AAI-TRF-IND message including SLPID update of the AMS and it receives AAI-TRF-IND-REQ message from the AMS, the ABS shall include this parameter in AAI-TRF-IND-RSP message.

[Editor’s Note: Modify the field descriptions in “16.2.3.29 AAI_TRF_IND-RSP” in page 181]

16.2.3.29 AAI_TRF_IND-RSP

| Table 706—AAI-TRF_IND-RSP Message Field Description |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-TRF_IND-RSP message_format() { | — | — |  |
| Frame_Number | 10 | The least significant 10 bits of the frame number in which incoming LW will start 0~1023 |  |
| Sleep Cycle Length | 10 | The length of Sleep Cycle which contains the next scheduled Listening Window. If the AMS receives the traffic indication during the next scheduled Listening Window, the (current) Sleep Cycle shall be set to this value at that time. Value: 0~1023 Sleep Cycle Length = Value + 1 |  |
| SLPID | 10 | New SLPID to replace the AMS’s old SLPID with. When the ABS has sent the AAI-TRF-IND message including SLPID update of the AMS and it receives AAI-TRF-IND-REQ from the AMS, the ABS shall include this parameter in AAI-TRF-IND-RSP. | Shall be included when the SLPID is updated. |
| } | — | — |  |

========================== End of Proposed Text ==============================