| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Corrections to the Subsection about MAC Control Messages with CMAC Protection (16.2.4.8) |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC Joey Chou Intel | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-3 for P802.16m/D9. |  |
| Abstract | The contribution proposes corrections to the subsection about MAC control messages with CMAC protection. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Corrections to the Subsection about MAC Control Messages with CMAC Protection (16.2.4.8)

Lei Wang

InterDigital Communications, LLC

Joey Chou

Intel

- Introduction

There are multiple problems with the newly added text in subsection "16.2.4.8 MAC Control Messages with CMAC Protection", e.g.,

- in Figure 397, why does a MAC message need to divide into multiple N-bytes segments, each having padding? Also, it shows all those segments are put on top of each other in the payload portion. Should they concatenated? In addition, was the left side MAC control message intended to show the case without CMAC?

- Why does it has to show the PER encoding box in the diagrams? It does not add any value, instead, it add confusion.

We propose to changes to this subsection to correct/clarify those identified issues.

- Suggested changes in the 802.16m/D9

The following is the proposed change in the 802.16m/D9. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 50, page 295 to line 3 page 296

Change the text in line 50, page 295 to line 3 page 296 as follows:

The CMAC protected MAC control message consists of MAC control message content ASN.1 encoded data followed by the CMAC tuple. MAC control message content represents the MAC control message in the binary data format that is generated by the PER encoder. A CMACI (CMAC Indicator) flag is defined in the MAC control message to indicate whether the MAC control message is protected using CMAC or not. CMACI set to "1"indicates that MAC control message is protected using CMAC and CMAC tuple is present after the ASN.1 binary data in the MAC control message. CMACI set to '0' or absence of CMACI in the MAC control message indicates that MAC control message is not protected using CMAC.

Figure 397 illustrates the construction of a MAC PDU carrying CMAC protected MAC control message. If a MAC control message has CMACI set to "0" or does not have CMACI, its MAC control message content ASN.1 binary data and padding bit (if needed for byte alignment), shall be the MAC PDU payload that has the size M = N bytes, where N is the number of bytes of the MAC control message content data plus padding when needed. If a MAC control message has CMAC set to "1", its MAC control message content ASN.1 binary data and padding bits are sent to the AES-CMAC algorithm to generate the CMAC value. The CMAC protected MAC control message consists of MAC control message content ASN.1 encoded data followed by the CMAC tuple.

The MAC PDU payload shall contain the MAC control message content ASN.1 binary data, padding bits (if any), and the CMAC tuple, and have the size M = N + 12 bytes, where N is the number of bytes of the MAC control message content data plus padding when needed.

Suggested change #2: line 4 to 43, page 296, Figure 397

Replace line 4 to 43 on page 296 by the following:

Figure 397—MAC PDU construction with / without CMAC protection

Suggested change #3: line 1, page 297, Figure 398

Replace Figure 398 in line 1 page 297 by the following figure:

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”