| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover trigger conditions cleanup (Section 16.2.6.2) |  |
| Date Submitted | 2010-11-09 |  |
| Source(s) | Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0051: IEEE P802.16m Sponsor Ballot Recirculation #3 Section 16.2.6.2 |  |
| Abstract | This contribution provides text cleanup regarding HO trigger conditions. |  |
| Purpose | To be discussed and adopted by WG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handover trigger conditions cleanup (Section 16.2.6.2)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides text cleanup regarding handover trigger conditions.

Following items needs to be updated in the current triggering conditions (Table 768).

- Triggering condition should be differentiated by intra-FA and inter-FA as supported by 802.16-2009 (pp. 1189, DCD Type 121: Default HO RSSI and CINR averaging parameter)

  - For intra-FA, an ABS does not have to allocate scanning intervals, and interference level between the ABS and an intra-FA BS is higher than inter-FA case, in general:
An AMS needs to measure the channel with longer averaging duration (than the inter-FA case)

  - For inter-FA, an ABS needs to allocate scanning intervals, and interference level between the ABS and an inter-FA BS is lower than intra-FA case, in general:
An AMS needs to measure the channel with shorter averaging duration (than the intra-FA case)

- ‘ABS type’ field should be located outer ‘for’ loop. (editorial)

  - ‘ABS type’ does not have to be duplicated in the ‘for’ loop.

# Proposed Text

Adopt the following TWO remedies.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy #1: Update Table 768 in pp. 342, line 20 in D9 as follows:]

| Name | Length (Bits) | Value |
| --- | --- | --- |
| Number of Triggers | 6 | Total number of Triggers that are defined |
| for (i=0; i <= Number of Triggers; i++) { |  |  |
| Number of conditions | 2 | The number of conditions that are included in this trigger (see loop definition below). When more than one conditions are included, this trigger is referred to as a complex trigger and is the logical AND combination of all the included conditions. |
| ABS type | 4 | ABS type of T-ABS for this Trigger definition: (Any, Macro ABS, Macro Hot-zone ABS, Femto ABS, etc.). A value representing "any" means this trigger applies to all T-ABSs. This value of ABS type field shall be ignored for triggers with Type= 0x3 or the Function=0x5 or 0x6 in Table 769 0x0: Any 0x1: Macro ABS 0x2: Macro Hot-zone ABS 0x3: Femto ABS 0x4: R1 BS 0x5-0xF: Reserved |
| Trigger averaging parameter for intra-FA measurement | 8 | The averaging parameter used for averaging this trigger metric according to equation (4) for T-ABS (which is defined in ABS type). If not present, the default trigger averaging parameter in AAI-SCD is used 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |
| Trigger averaging parameter for inter-FA measurement | 8 | The averaging parameter used for averaging this trigger metric according to equation (4) for T-ABS (which is defined in ABS type). If not present, the default trigger averaging parameter in AAI-SCD is used 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |
| for (j=0; j<= Number of conditions; j++) { |  |  |
| ABS type | 4 | ABS type of T-ABS for this Trigger definition: (Any, Macro ABS, Macro Hot-zone ABS, Femto ABS, etc.). A value representing "any" means this trigger applies to all T-ABSs. This value of ABS type field shall be ignored for triggers with Type= 0x3 or the Function=0x5 or 0x6 in Table 769 0x0: Any 0x1: Macro ABS 0x2: Macro Hot-zone ABS 0x3: Femto ABS 0x4: R1 BS 0x5-0xF: Reserved |
| Type/Function/Action | 9 | See Table 769 for description |
| Trigger Value | 8 | Trigger value is the value used in comparing measured metric for determining a trigger condition. |
| Trigger averaging parameter | 8 | The averaging parameter used for averaging this trigger metric according to equation (4) for T-ABS (which is defined in ABS type). If not present, the default trigger averaging parameter in AAI-SCD is used 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |
| } |  |  |
| } |  |  |

[Remedy #2: Update Table 708 in pp. 342, line 27 in D9 as follows:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| Trigger definitions | Variable | Refer to Table 768 |  |
| Default Trigger Averaging Parameter for Intra-FA measurement | 8 | Indicates default alpha averaging parameter for intra-FA measurement. 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |  |
| Default Trigger Averaging Parameter for Inter-FA measurement | 8 | Indicates default alpha averaging parameter for inter-FA measurement. 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |  |
| … |  |  |  |

[Remedy #3: Update ASN.1 code in pp. 1042, line 16 in D9 as follows:]

--

-- Triggers

-- Designed based on Table 768 in D9

--

maxNumberOfTriggersINTEGER ::= 64

maxNumberOfConditionsINTEGER ::= 4

Triggers ::= SEQUENCE {

triggerListSEQUENCE (SIZE(1..maxNumberOfTriggers)) OF TriggerInfo

}

TriggerInfo ::= SEQUENCE {

absType ENUMERATED {any, macro, macro-hotzone, femto, r1,

spare1, spare2, spare3, spare4, spare5,

spare6, spare7, spare8, spare9, spare10, spare11} ,

triggerAveParamForIntra ENUMERATED {one, half, quarter, one-8th, one-16th,

one-32th, one-64th, one-128th, one-256th, one-512th } OPTIONAL,

triggerAveParamForInter ENUMERATED {one, half, quarter, one-8th, one-16th,

one-32th, one-64th, one-128th, one-256th, one-512th } OPTIONAL,

conditionListSEQUENCE (SIZE(1..maxNumberOfConditions)) OF ConditionInfo

}

ConditionInfo ::= SEQUENCE {

absTypeENUMERATED {any, macro, macro-hotzone, femto, r1,

spare1, spare2, spare3, spare4, spare5,

spare6, spare7, spare8, spare9, spare10, spare11} ,

typeFuncActionTypeFuncAction,

triggerValueINTEGER (0..255),

triggerAveParamENUMERATED {one, half, quarter, one-8th, one-16th,

one-32th, one-64th, one-128th, one-256th, one-512th }

}

[Remedy #4: Update ASN.1 code in pp. 978, line 55 in D9 as follows:]

triggers TriggerConditions,

defaultTriggerAveParamForIntra ENUMERATED {one, half, quarter, one-8th, one-16th,

one-32th, one-64th, one-128th, one-256th, one-512th },

defaultTriggerAveParamForInter ENUMERATED {one, half, quarter, one-8th, one-16th,

one-32th, one-64th, one-128th, one-256th, one-512th },

------------------------------------------------------ End of Proposed Text ------------------------------------------------------