| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Femto ABS reselection by AMS (16.4.7.5) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Yi-Ting Lin, Yung-Lan Tseng, Chun-Yen Hsu, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen Institute for Information Industry (III) | E-mail: eating@iii.org.tw |
| Re: | Sponsor ballot IEEE P802.16m/D8 : 16.4.7.5 |  |
| Abstract | This contribution proposes the text modification for Femto ABS reselection |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Femto ABS reselection by AMS (16.4.7.5)

Yi-Ting Lin, Yung-Lan Tseng, Chun-Yen Hsu, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee, Chiu-Wen Chen

Institute for Information Industry (III)

# Introduction

To help the AMS to make a decision if the Femto ABS is accessible or not, the Femto ABS provides its CSGID in the AAI-RNG-RSP. However, the number of CSGID(s) of the Femto ABS may not be one. We propose the multiple CSGIDs condition and corresponding AAI-RNG-RSP message modification for that purpose.

# Reference

[1] IEEE P802.16m/D9, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” Oct. 2010.

- Text proposal for inclusion in the 802.16m Draft 9

----------------------------------------------------------------------------------------------------------------------------------------------------------------

[Modify line 25, page 894 in Section 16.4.7.5]

Suggested Remedy 1

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.4.7.5 Femto ABS reselection by AMS

If the received CSGID(s) from the AMS does not match any of the CSGID(s) of the Femto ABS itself, and the Femto ABS is a CSG-Closed Femto ABS, the Femto ABS shall send a AAI-RNG-RSP and in the AAI-RNG-RSP it indicates the rejection of access for this AMS. In order to help the AMS to attach to nearby Femto ABSs, the Femto ABS provides "Redirection Info" to the AMS in the AAI-RNG-RSP message. The "Redirection Info" consists of the ABSID, preamble index and center frequency of other nearby cells. Since the Femto ABS cannot be sure that the AMS is not its member as the AMS may not have included all the CSGIDs in its whitelist in AAI-RNG-REQ, the Femto ABS provides its CSGID(s) in the AAI-RNG-RSP in this case so that the AMS can perform the accessibility check for the Femto ABS.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------

[Modify line 54, page 99 in Table 679 of Section 16.2.3.2

Suggested Remedy 2

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.2.3.2 AAI-RNG-RSP

Table 679 – AAI-RNG-RSP message Field Description

| Field | Size (bits) | Value | Description |
| --- | --- | --- | --- |
| for (i=0; i<N_CSG_IDs; i++) { |  | N_CSG_IDs is the number of CSGIDs belong to the Femto ABS |  |
| CSGID | Variable | CSGID of the Femto ABS sending the AAI_RNG-RSP. | Optional. If included, to help the AMS make the decision if the Femto ABS is accessible or not. |
| } |  |  |  |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------