| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Broadcast A-A-MAP IE for Multicast (16.3.5.5.2.4) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Zheng Yan-Xiu, Yu-Chuan Fang ITRI | zhengyanxiu@itri.org.tw +886-3-5914715 |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | The contribution clarifies the text about broadcast assignment A-MAP IE for multicast. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Broadcast A-A-MAP IE for Multicast

Zheng Yan-Xiu, Yu-Chuan Fang

ITRI

- Introduction

In IEEE P802.16m/P8[1], message type indicator distinguishes broadcast and multicast. However, in IEEE P802.16m/P9 [2], the associated text is modified. This contribution clarify benefits of distinguish between broadcast and multicast by CRC mask. Each one of these benefits deserves our consideration to use message type indicator to distinguish broadcast and multicast.

- Benefits Clarification

The first benefit is implementation complexity reduction. Based on the current design, chip shall verify the 12 bits masking code to distinguish broadcast and multicast. When message type indicator is used to distinguish both broadcast and multicast, two bits are enough to verify if it is for multicast purpose. AMS could process the IE with unified approach.

AMS can discard multicast IE with an easy way. In our point of view, broadcast and multicast have different application scenarios. Broadcast is used often but multicast is not. Based on our observation on 3GPP LTE standardization, multicast is still not widely discussed as other cases. It does not mean multicast useless. We really know multicast is used for special application. PTT is one application. M2M also raises attention due to management. Another observation is that this is generally not used for human communication. Therefore, differentiating broadcast from multicast by message type indicator facilitates AMS operation. In general, AMS can discard this multicast IE (disable the multicast function) by the message type indicator to reduce the overhead.

Future extension is further benefit. Based on current design, the multicast function is bundled with broadcast function. If we need some modification to increase functionality of multicast, it would be hard to separate both. Since we assume multicast and broadcast have different application natures or even devices, it would be more proper to separate both functionalities based on message type indicator. We can introduce other functions by adding other CRC mask for multicast only. For example, M2M devices can just read the associated CRC mask to differentiate other purposes.

From implementation complexity, application scenario and future extension points of view, each one deserves our consideration to separate both functionalities.

- References

[1] IEEE P802.16m/D8, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[1] IEEE P802.16m/D9, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text Changes

[Remedy-1: Change the following text, at line 1 in page 646, section 16.3.5.5.2.4, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 841 – Assignment A-MAP IE Types

| A-MAP IE Type | Usage | Property |
| --- | --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE | Unicast |
| 0b0001 | UL Basic Assignment A-MAP IE | Unicast |
| 0b0010 | DL Subband Assignment A-MAP IE | Unicast |
| 0b0011 | UL Subband Assignment A-MAP IE | Unicast |
| 0b0100 | Feedback Allocation A-MAP IE | Unicast |
| 0b0101 | UL Sounding Command A-MAP IE | Unicast |
| 0b0110 | CDMA Allocation A-MAP IE | Unicast |
| 0b0111 | DL Persistent Allocation A-MAP IE | Unicast |
| 0b1000 | UL Persistent Allocation A-MAP IE | Unicast |
| 0b1001 | Group Resource Allocation A-MAP IE | Multicast |
| 0b1010 | Feedback Polling A-MAP IE | Unicast |
| 0b1011 | BR-ACK A-MAP IE | Multicast |
| 0b1100 | Broadcast Assignment A-MAP IE | Broadcast/Multicast |
| 0b1101 | Reserved | NA. |
| 0b1110 | Reserved | NA. |
| 0b1111 | Extended Assignment A-MAP IE | NA. |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-2: Change the following tables, at line 36 in page 646, section 16.3.5.5.2.4, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

Table 842—Description of CRC Mask

| Masking Prefix (1 bit MSB) | Remaining 15 bit LSBs |  |
| --- | --- | --- |
| 0b0 | Type indicator | Masking code |
|  | 0b000 | 12 bit STID |
|  | 0b001 | Refer to Table 843 |
|  | 0b010 | Refer to Table 844 |
| 0b1 | 15 bit RA-ID: The RA-ID is derived from the AMS' random access attributes (i.e., superframe number (LSB 5 bits), frame_index (2 bits), preamble code index for ranging or BR (6 bits) and opportunity index for ranging or BR (2 bits)) as defined below: RA-ID = (LSB 5bits of superframe number ¦ frame_index ¦ preamble_code_index ¦ opportunity_index) |  |

Table 843—Description of the Masking Code for type indicator 001

| Decimal Value | Description |
| --- | --- |
| 0 | Used to mask Broadcast Assignment A-MAP IE when Function index is 0b00 or 0b01 for broadcast or ranging channel assignment |
| 1 | Used to mask BR-ACK A-MAP IE |
| 2-128 | Used to mask Group Resource Allocation A-MAP IE (group ID) |
| 129 | Used to mask Broadcast Assignment A-MAP IE when Function index is 0b10 |
| Others | reserved |

[New a table:]

Table 844—Description of the Masking Code for type indicator 010

| Decimal Value | Description |
| --- | --- |
| 4095 | Used to mask Broadcast Assignment A-MAP IE for multicast assignment |
| Others | reserved |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-3: Change the following text, at line 31 in page 700, section 16.3.5.5.2.4.13, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

…

*A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 842. If Function index == 0b00 or 0b01, tThe CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b001. If Function index == 0b10, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b010.

…

-------------------------------------------------End proposed text----------------------------------------------------------------