| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text change for frame structure: co-existence with TD-SCDMA/LTE ratio (16.3.3.7) |  |
| Date Submitted | 2010-10-26 |  |
| Source(s) | Zheng Yan-Xiu, Chung-Lien Ho, Chang-Lan Tsai, Yu-Chuan Fang ITRI | zhengyanxiu@itri.org.tw +886-3-5914715 |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | The contribution proposes frame structures which co-existing with TD-SCDMA/LTE |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text change for frame structure: co-existence with TD-SCDMA/LTE ratio (16.3.3.7)

Zheng Yan-Xiu, Chung-Lien Ho, Chang-Lan Tsai, Yu-Chuan Fang

ITRI

- Introduction

In P802.16m/D9 [1], the frame structure can support that IEEE 802.16m to co-exist with TD-SCDMA/LTE in 1/8 CP ratio by puncturing 2 OFDM symbols. In other words, 6% capacity loss is expected. This contribution provides some analysis for the timing alignment between TDD systems. Based on our analysis, the used OFDM symbols could be the same and most cases can be supported as well.

- TD-SCDMA frame structure and the associated constraints

TD-SCDMA has two TDD configurations: symmetric and asymmetric DL/UL configurations. Fig. 1 illustrates the symmetric DL/UL configuration. The subframe spans 5ms and is composed of DwPTS (75μs), GP (75μs), UpPTS (125μs), 7 slots (675μs x7=4725μs). Downlink is composed of DwPTS, and downlink slots. Uplink is composed of UpPTS and uplink slots.

Fig. 1: TD-SCDMA frame structure of symmetric DL/UL configuration.

Therefore, for the symmetric DL/UL allocation, downlink duration occupies 2775 μs and uplink duration occupies 2150 μs. For the asymmetric DL/UL allocation, downlink duration occupies 3450 μs and uplink duration occupies 1475 μs.

- TD-LTE frame structure and the associated constraints

TD-LTE applies 10ms frame structure which is composed of 10 subframes of 1ms. In general, each subframe is composed of 14 or 12 OFDM symbols depending on which type of cyclic prefix used. A special subframe is composed of Downlink Pilot Time Slot (DwPTS), Uplink Pilot Time Slot (UpPTS) and guard period (GP).

Figure 2 and Figure 3 illustrate two exemplary frame structures for TD-LTE. For TD-LTE, it has two kinds of periodicity, 5ms and 10ms. Comparing to IEEE 802.16m, 5ms TD-LTE frame structure could co-exist with IEEE 802.16m but the 10ms TD-LTE frame structure can not co-exist with IEEE 802.16m.

Figure 2: Exemplary frame structure with 5ms switch-point periodicity.

Figure 3: Exemplary frame structure with 10ms switch-point periodicity.

Table 1 further provides uplink-downlink frame configuration for TD-LTE. D, S and U denote downlink, special subframe and uplink. Configuration 0, 1, 2 can co-exist with IEEE 802.16m can co-exist with configurations 0, 1, 2 by 5ms periodicity but can not co-exist with configurations 3, 4, 5 and 6. Therefore, our embodiment will provide frame structure co-existing with configurations 0, 1, 2. Tables 2 and 3 further illustrate the special subframe configuration for TD-LTE. Based on configurations 0, 1 and 2 and special subframe configuration, we have some downlink duration and uplink duration shown in Appendix A.

Table 1: Uplink-downlink frame configuration for TD-LTE.

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

Table 2: Special subframe configuration for TD-LTE with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DwPTS | GP (NCP) | UpPTS (NCP in UL) | GP (ECP) | UpPTS (ECP in UL) |
| 0 | 215.21μs | 713.23μs | 71.56μs | 701.22μs | 83.57μs |
| 1 | 645.12μs | 283.32μs |  | 271.31μs |  |
| 2 | 716.68μs | 211.76μs |  | 199.75μs |  |
| 3 | 797.06μs | 131.38μs |  | 119.37μs |  |
| 4 | 859.81μs | 68.63μs |  | 56.62μs |  |
| 5 | 215.21μs | 641.66μs | 143.13μs | 617.63μs | 167.16μs |
| 6 | 645.12μs | 211.75μs |  | 187.72μs |  |
| 7 | 716.68μs | 140.19μs |  | 116.16μs |  |
| 8 | 788.25μs | 68.62μs |  | 44.59μs |  |

Table 3: Special subframe configuration for TD-LTE with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- |
|  | DwPTS | UpPTS (NCP in UL) | UpPTS (ECP in UL) |
| 0 | 250μs | 71.35μs | 83.33μs |
| 1 | 666.67μs |  |  |
| 2 | 750μs |  |  |
| 3 | 833.33μs |  |  |
| 4 | 250μs | 142.71μs | 166.67μs |
| 5 | 666.67μs |  |  |
| 6 | 750μs |  |  |

- Methodology of co-existence

Figure 4 further plots the timing relationship between TD-LTE and IEEE 802.16m. Denote by D_LTE, U_LTE, G and F the downlink duration, uplink duration, guard period frame duration of LTE, where F = D_LTE + U_LTE + G =5ms. Denote by D_16m, U_16 and TTG the downlink duration, uplink duration and transmit transition gap of IEEE 802.16m. Denote by RTD the supported round trip delay for IEEE 802.16m, where RTD=2d/c, d is the supported distance between mobile station and base station and c is light speed.

In this figure, we find three equations to support co-existence as follows:

- D_LTE+G/2 > D_16m+RTD/2

- U_LTE+G/2 > U_16m+RTD/2

- RTD<TTG

Guard period is larger than the supported round trip delay for the LTE system. Therefore, the downlink propagation delay can be as large as G/2. The timing advance can also be as large as G/2 in uplink. Therefore, if the first equation is not satisfied and uplink timing advance is G/2 for TD-LTE, IEEE 802.16m system will interfere TD-LTE with high probability. For the second equation, we have the same argument. The third equation describes the TTG shall be larger than the round trip delay for IEEE 802.16m. If not, IEEE 802.16m system can not support such round trip delay. In the following, we will provide the suggested frame configuration for 1/8 and 1/16 CP ratios based on this three equation.

Figure 4: Exemplary co-existence between LTE and IEEE 802.16m.

Before we provide further analysis results, we assume the maximum supported distance is 5km for IEEE 802.16m and the RPD is 33.33μs as described in IEEE C802.16m-09/2124r1 [2]. Under this assumption, uplink and downlink duration and requirements, we have allowed OFDM symbols for 1/8 and 1/16 cyclic prefix ratio in Appendixes B and C.

According to Appendix B, we will discuss three configurations for 1/8 cyclic prefix ratio.

For configuration 0, we can have some configurations A:B=14 (13):33, 17:30, 17:31, 18:30 and 15:33, where A:B denote the number OFDM symbols in downlink versus that in uplink. In downlink 14 and 15 does not suit for IEEE 802.16m and therefore 17:30 is more proper configuration. This has been defined in IEEE 802.16m frame configuration set.

For Configuration 1, we can use these configurations A:B=24:23, 26:21(20), 27:20, 27:21(20) and 25:23. In this case, 27:20 might be more proper to support most used LTE frame configuration with fewer guard period. The frame configuration is shown in Table 4. The configuration can be used to co-exist with TD-SCDMA with symmetric configuration.

For Configuration 2 we can use these configurations A:B=34:14 (13), 34:13, 36:12, 37:10, 37:11 and 36:11. The 34:13 is more preferable and the frame configuration is shown in Table 4. This configuration can co-exist with TD-SCDMA with asymmetric configuraiton.

Table 4: Frame Configuration for 1/8 CP Ratio.

| D:U | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 6:2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type2 | 105.714/ 60 |
| 5:3 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | DL Type3 | UL Type1 | UL Type2 | UL Type2 | 105.714/ 60 |

According to Appendix C, we will discuss three configurations for 1/16 cyclic prefix ratio.

For Configuration 0, we can use these configurations A:B=15:35, 16:35, 18:32, 19:32, 19:31, 17:33 and 18:33 to co-exist with TD-LTE. The configuration 18:32 is suggested configuration to co-exist with TD-LTE with best performance.

For Configuration 1, we can use these configurations A:B=26:24, 28(27):22, 29:21(20), 29:22(20) and 25:25. In this case, the suggested configuration is 26:24 and 29:20. The frame configuration is shown in Table 5. The 29:20 can be used to co-exist with TD-SCDMA symmetric configuration.

For Configuration 2, we can use these configurations A:B=36:14(13), 38:12, 39:11, 36:15 (13) and 39:12. In this case, the suggested configuration is 36:13 with D:L=6:2. The frame configuration is shown in Table 5. This configuration can co-exist with TD-SCDMA with asymmetric configuration.

Table 5: Frame Configuration for 1/16 CP Ratio.

| D:U | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 6:2 | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type2 | 179.996/ 60 |
| 5:3 | DL Type1 | DL Type2 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type2 | UL Type2 | 179.996/ 60 |
| 4:4 | DL Type1 | DL Type2 | DL Type2 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | 82.853/ 60 |
| 3:5 | DL Type1 | DL Type2 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | UL Type2 | UL Type2 | 82.853/ 60 |

- Conclusion

This contribution provides frame structure to support TDD co-existence with TD-SCDMA/LTE with 1/8 and 1/16 CP ratio.

- References

[]	IEEE P802.16m/D9, DRAFT Amendment to IEEE Standard for Local and Metropolitan Area Networks – Part 16: Air interference for fixed and mobile broadband wireless access systems, Oct. 2010.

[]	IEEE C802.16m-09/2124r1, Multi-RAT Coexistence Analysis and Frame Configuration (15.3.3), IEEE 802.16m Session 63, Sep. 2009.

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

[Remedy: Change texts in Section 16.3.3.7 in Page 497 Table 803 Line 1 as follows:]

<\ins>

Table 803—Frame Configuration and Indexing (5/10/20MHz channel bandwidth)

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - AAI subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (µs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - Length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| 1 | 5/10/20 | 1/16 | 0 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 2 | 5/10/20 | 1/16 | 1 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 3 | 5/10/20 | 1/16 | 2 | TDD | 4:4 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 4 | 5/10/20 | 1/16 | 3 | TDD | 3:5 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 5 | 5/10/20 | 1/16 | 4 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 | N/A |
| 6 | 5/10/20 | 1/16 | 5 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type2 | 179.996/ 60 |
| 7 | 5/10/20 | 1/16 | 6 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type2 | UL Type2 | 179.996/ 60 |
| 8 | 5/10/20 | 1/16 | 7 | TDD | 4:4 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type2 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | 82.853/ 60 |
| 9 | 5/10/20 | 1/16 | 8 | TDD | 3:5 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | UL Type2 | UL Type2 | 82.853/ 60 |
| 610 | 5/10/20 | 1/8 | 0 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | 105.714 / 60 |
| 711 | 5/10/20 | 1/8 | 1 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | 105.714 / 60 |
| 812 | 5/10/20 | 1/8 | 2 | TDD | 4:4 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | 105.714 / 60 |
| 913 | 5/10/20 | 1/8 | 3 | TDD | 3:5 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | 105.714 / 60 |
| 1014 | 5/10/20 | 1/8 | 4 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | N/A |
| 1115 | 5/10/20 | 1/8 | 5 | TDD | 5:3 | 3 | 2 | 3 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | Not used | Not Used | Not Used | 105.714 / 60 |
| 1216 | 5/10/20 | 1/8 | 6 | TDD | 5:3 | 3 | 2 | 1 | DL Type1 | DL Type1 | Not Used | Not Used | UL Type1 | Not used | Not Used | Not Used | 105.714 / 60 |
| 1317 | 5/10/20 | 1/8 | 7 | TDD | 5:3 | 2 | 3 | 3 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 1418 | 5/10/20 | 1/8 | 8 | TDD | 5:3 | 2 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 1519 | 5/10/20 | 1/8 | 9 | TDD | 5:3 | 2 | 2 | 3 | DL Type1 | DL Type1 | Not used | UL Type1 | UL Type1 | UL Type1 | Not used | Not Used | 105.714 / 60 |
| 1620 | 5/10/20 | 1/8 | 10 | TDD | 5:3 | 2 | 2 | 1 | DL Type1 | DL Type1 | Not used | Not used | Not Used | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 1721 | 5/10/20 | 1/8 | 11 | TDD | 5:3 | 1 | 4 | 3 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 1822 | 5/10/20 | 1/8 | 12 | TDD | 5:3 | 1 | 4 | 2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 1923 | 5/10/20 | 1/8 | 13 | TDD | 5:3 | 1 | 3 | 3 | DL Type1 | DL Type1 | DL Type1 | Not used | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 2024 | 5/10/20 | 1/8 | 14 | TDD | 5:3 | 1 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not used | Not used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 2125 | 5/10/20 | 1/8 | 15 | TDD | 5:3 | 1 | 2 | 3 | DL Type1 | DL Type1 | Not used | Not used | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 2226 | 5/10/20 | 1/8 | 16 | TDD | 5:3 | 1 | 2 | 1 | DL Type1 | DL Type1 | Not used | Not used | Not used | Not used | UL Type1 | Not Used | 105.714 / 60 |
| 2327 | 5/10/20 | 1/8 | 17 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type1 | 208.571 / 60 |
| 2428 | 5/10/20 | 1/8 | 18 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | 208.571 / 60 |
| 2529 | 5/10/20 | 1/8 | 19 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type3 | DL Type3 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | 311.428 / 60 |
| 30 | 5/10/20 | 1/8 | 20 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type2 | 105.714/ 60 |
| 31 | 5/10/20 | 1/8 | 21 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type3 | DL Type3 | DL Type3 | UL Type1 | UL Type2 | UL Type2 | 105.714/ 60 |
| 2632 | 5/10/20 | 1/8 | 2022 | TDD | 5:3 | 2 | 1 | 3 | DL Type2 | Not Used | Not Used | UL Type1 | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 2733 | 5/10/20 | 1/8 | 2123 | TDD | 5:3 | 1 | 1 | 3 | DL Type2 | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 2834 | 5/10/20 | 1/8 | 2224 | TDD | 6:2 | 1 | 1 | 2 | DL Type2 | Not Used | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 2935 | 5/10/20 | 1/8 | 2325 | TDD | 6:2 | 1 | 2 | 2 | DL Type1 | DL Type1 | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 3036 | 5/10/20 | 1/8 | 2426 | TDD | 6:2 | 1 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 3137 | 5/10/20 | 1/8 | 2527 | TDD | 6:2 | 1 | 4 | 2 | DL Type1 | DL Type1 | DL Type1‘ | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 3238 | 5/10/20 | 1/8 | 2628 | TDD | 6:2 | 1 | 5 | 2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 3339 | 5/10/20 | 1/8 | 2729 | TDD | 6:2 | 2 | 1 | 2 | DL Type2 | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 3440 | 5/10/20 | 1/8 | 2830 | TDD | 6:2 | 2 | 2 | 2 | DL Type1 | DL Type1 | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 3541 | 5/10/20 | 1/8 | 2931 | TDD | 6:2 | 2 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 3642 | 5/10/20 | 1/8 | 3032 | TDD | 6:2 | 2 | 4 | 2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 3743 | 5/10/20 | 1/4 | 0 | TDD | 5:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 |  | 139.988 / 60 |
| 3844 | 5/10/20 | 1/4 | 1 | TDD | 4:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 |  | 139.988 / 60 |
| 3945 | 5/10/20 | 1/4 | 2 | TDD | 3:4 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 |  | 139.988 / 60 |
| 4046 | 5/10/20 | 1/4 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |

<\ins>

-------------------------------  Text End  ---------------------------------------------------

- Appendix A Downlink and Uplink Duration

Table A.1: Downlink and uplink duration for TD-LTE configuration 0 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Downlink | GP (NCP) | Uplink (NCP in UL) | GP (ECP) | Uplink (ECP in UL) |
| 0 | 1215.21μs | 713.23μs | 3071.56μs | 701.22μs | 3083.57μs |
| 1 | 1645.12μs | 283.32μs |  | 271.31μs |  |
| 2 | 1716.68μs | 211.76μs |  | 199.75μs |  |
| 3 | 1797.06μs | 131.38μs |  | 119.37μs |  |
| 4 | 1859.81μs | 68.63μs |  | 56.62μs |  |
| 5 | 1215.21μs | 641.66μs | 3143.13μs | 617.63μs | 3167.16μs |
| 6 | 1645.12μs | 211.75μs |  | 187.72μs |  |
| 7 | 1716.68μs | 140.19μs |  | 116.16μs |  |
| 8 | 1788.25μs | 68.62μs |  | 44.59μs |  |

Table A.2: Downlink and uplink duration for TD-LTE configuration 1 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Downlink | GP (NCP) | Uplink (NCP in UL) | GP (ECP) | Uplink (ECP in UL) |
| 0 | 2215.21μs | 713.23μs | 2071.56μs | 701.22μs | 2083.57μs |
| 1 | 2645.12μs | 283.32μs |  | 271.31μs |  |
| 2 | 2716.68μs | 211.76μs |  | 199.75μs |  |
| 3 | 2797.06μs | 131.38μs |  | 119.37μs |  |
| 4 | 2859.81μs | 68.63μs |  | 56.62μs |  |
| 5 | 2215.21μs | 641.66μs | 2143.13μs | 617.63μs | 2167.16μs |
| 6 | 2645.12μs | 211.75μs |  | 187.72μs |  |
| 7 | 2716.68μs | 140.19μs |  | 116.16μs |  |
| 8 | 2788.25μs | 68.62μs |  | 44.59μs |  |

Table A.3: Downlink and uplink duration for TD-LTE configuration 2 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Downlink | GP (NCP) | Uplink (NCP in UL) | GP (ECP) | Uplink (ECP in UL) |
| 0 | 3215.21μs | 713.23μs | 1071.56μs | 701.22μs | 1083.57μs |
| 1 | 3645.12μs | 283.32μs |  | 271.31μs |  |
| 2 | 3716.68μs | 211.76μs |  | 199.75μs |  |
| 3 | 3797.06μs | 131.38μs |  | 119.37μs |  |
| 4 | 3859.81μs | 68.63μs |  | 56.62μs |  |
| 5 | 3215.21μs | 641.66μs | 1143.13μs | 617.63μs | 1167.16μs |
| 6 | 3645.12μs | 211.75μs |  | 187.72μs |  |
| 7 | 3716.68μs | 140.19μs |  | 116.16μs |  |
| 8 | 3788.25μs | 68.62μs |  | 44.59μs |  |

Table A.4: Downlink and uplink duration for TD-LTE configuration 0 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- |
|  | Downlink | Uplink (NCP in UL) | Uplink (ECP in UL) |
| 0 | 1250μs | 3071.35μs | 3083.33μs |
| 1 | 1666.67μs |  |  |
| 2 | 1750μs |  |  |
| 3 | 1833.33μs |  |  |
| 4 | 1250μs | 3142.71μs | 3166.67μs |
| 5 | 1666.67μs |  |  |
| 6 | 1750μs |  |  |

Table A.5: Downlink and uplink duration for TD-LTE configuration 1 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- |
|  | Downlink | Uplink (NCP in UL) | Uplink (ECP in UL) |
| 0 | 2250μs | 2071.35μs | 2083.33μs |
| 1 | 2666.67μs |  |  |
| 2 | 2750μs |  |  |
| 3 | 2833.33μs |  |  |
| 4 | 2250μs | 2142.71μs | 2166.67μs |
| 5 | 2666.67μs |  |  |
| 6 | 2750μs |  |  |

Table A.6: Downlink and uplink duration for TD-LTE configuration 2 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |
| --- | --- | --- | --- |
|  | Downlink | Uplink (NCP in UL) | Uplink (ECP in UL) |
| 0 | 3250μs | 1071.35μs | 1083.33μs |
| 1 | 3666.67μs |  |  |
| 2 | 3750μs |  |  |
| 3 | 3833.33μs |  |  |
| 4 | 3250μs | 1142.71μs | 1166.67μs |
| 5 | 3666.67μs |  |  |
| 6 | 3750μs |  |  |

- Appendix B Allowed OFDM Symbols for 1/8 Cyclic Prefix Ratio

Table B.1: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 0 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 15.12 | 33.17 | 15.06 | 33.23 |
| 1 | 17.21 | 31.08 | 17.15 | 31.14 |
| 2 | 17.56 | 30.73 | 17.50 | 30.79 |
| 3 | 17.95 | 30.34 | 17.89 | 30.40 |
| 4 | 18.25 | 30.03 | 18.19 | 30.09 |
| 5 | 14.77 | 33.52 | 14.65 | 33.63 |
| 6 | 16.86 | 31.43 | 16.74 | 31.54 |
| 7 | 17.21 | 31.08 | 17.09 | 31.19 |
| 8 | 17.56 | 30.73 | 17.44 | 30.85 |

Table B.2: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 1 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 15.12 | 33.17 | 15.06 | 33.23 |
| 1 | 17.21 | 31.08 | 17.15 | 31.14 |
| 2 | 17.56 | 30.73 | 17.50 | 30.79 |
| 3 | 17.95 | 30.34 | 17.89 | 30.40 |
| 4 | 18.25 | 30.03 | 18.19 | 30.09 |
| 5 | 14.77 | 33.52 | 14.65 | 33.63 |
| 6 | 16.86 | 31.43 | 16.74 | 31.54 |
| 7 | 17.21 | 31.08 | 17.09 | 31.19 |
| 8 | 17.56 | 30.73 | 17.44 | 30.85 |

Table B.3: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 2 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 34.56 | 13.72 | 34.51 | 13.78 |
| 1 | 36.65 | 11.63 | 36.60 | 11.69 |
| 2 | 37.00 | 11.29 | 36.94 | 11.34 |
| 3 | 37.39 | 10.89 | 37.33 | 10.95 |
| 4 | 37.70 | 10.59 | 37.64 | 10.65 |
| 5 | 34.22 | 14.07 | 34.10 | 14.19 |
| 6 | 36.31 | 11.98 | 36.19 | 12.10 |
| 7 | 36.65 | 11.63 | 36.54 | 11.75 |
| 8 | 37.00 | 11.29 | 36.88 | 11.40 |

Table B.4: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 0 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 15.29 | 33.00 | 15.23 | 33.06 |
| 1 | 17.32 | 30.97 | 17.26 | 31.03 |
| 2 | 17.72 | 30.57 | 17.66 | 30.63 |
| 3 | 18.13 | 30.16 | 18.07 | 30.22 |
| 4 | 14.94 | 33.34 | 14.83 | 33.46 |
| 5 | 16.97 | 31.32 | 16.85 | 31.44 |
| 6 | 17.37 | 30.91 | 17.26 | 31.03 |

Table B.5: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 1 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 25.01 | 23.28 | 24.95 | 23.33 |
| 1 | 27.04 | 21.25 | 26.98 | 21.31 |
| 2 | 27.44 | 20.84 | 27.38 | 20.90 |
| 3 | 27.85 | 20.44 | 27.79 | 20.50 |
| 4 | 24.67 | 23.62 | 24.55 | 23.74 |
| 5 | 26.69 | 21.60 | 26.57 | 21.71 |
| 6 | 27.10 | 21.19 | 26.98 | 21.31 |

Table B.6: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 2 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 34.73 | 13.55 | 34.68 | 13.61 |
| 1 | 36.76 | 11.53 | 36.70 | 11.59 |
| 2 | 37.16 | 11.12 | 37.11 | 11.18 |
| 3 | 37.57 | 10.72 | 37.51 | 10.78 |
| 4 | 34.39 | 13.90 | 34.27 | 14.02 |
| 5 | 36.41 | 11.87 | 36.30 | 11.99 |
| 6 | 36.82 | 11.47 | 36.70 | 11.59 |

- Appendix C Allowed OFDM Symbols for 1/16 Cyclic Prefix Ratio

Table C.1: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 0 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 16.01 | 35.12 | 15.95 | 35.18 |
| 1 | 18.22 | 32.91 | 18.16 | 32.97 |
| 2 | 18.59 | 32.54 | 18.53 | 32.60 |
| 3 | 19.00 | 32.12 | 18.94 | 32.19 |
| 4 | 19.33 | 31.80 | 19.26 | 31.86 |
| 5 | 15.64 | 35.49 | 15.52 | 35.61 |
| 6 | 17.85 | 33.27 | 17.73 | 33.40 |
| 7 | 18.22 | 32.91 | 18.10 | 33.03 |
| 8 | 18.59 | 32.54 | 18.47 | 32.66 |

Table C.2: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 1 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 26.30 | 24.82 | 26.24 | 24.89 |
| 1 | 28.52 | 22.61 | 28.45 | 22.67 |
| 2 | 28.88 | 22.24 | 28.82 | 22.31 |
| 3 | 29.30 | 21.83 | 29.24 | 21.89 |
| 4 | 29.62 | 21.51 | 29.56 | 21.57 |
| 5 | 25.93 | 25.19 | 25.81 | 25.32 |
| 6 | 28.15 | 22.98 | 28.02 | 23.10 |
| 7 | 28.52 | 22.61 | 28.39 | 22.74 |
| 8 | 28.88 | 22.24 | 28.76 | 22.37 |

Table C.3: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 2 with normal cyclic prefix in downlink.

| Special subframe configuration | Normal cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 36.60 | 14.53 | 36.54 | 14.59 |
| 1 | 38.81 | 12.32 | 38.75 | 12.38 |
| 2 | 39.18 | 11.95 | 39.12 | 12.01 |
| 3 | 39.59 | 11.54 | 39.53 | 11.60 |
| 4 | 39.92 | 11.21 | 39.85 | 11.27 |
| 5 | 36.23 | 14.90 | 36.11 | 15.02 |
| 6 | 38.44 | 12.69 | 38.32 | 12.81 |
| 7 | 38.81 | 12.32 | 38.69 | 12.44 |
| 8 | 39.18 | 11.95 | 39.05 | 12.07 |

Table C.4: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 0 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 16.19 | 34.94 | 16.13 | 35.00 |
| 1 | 18.33 | 32.79 | 18.27 | 32.86 |
| 2 | 18.76 | 32.36 | 18.70 | 32.43 |
| 3 | 19.19 | 31.94 | 19.13 | 32.00 |
| 4 | 15.82 | 35.31 | 15.70 | 35.43 |
| 5 | 17.97 | 33.16 | 17.84 | 33.28 |
| 6 | 18.40 | 32.73 | 18.27 | 32.86 |

Table C.5: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 1 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 26.48 | 24.64 | 26.42 | 24.71 |
| 1 | 28.63 | 22.50 | 28.57 | 22.56 |
| 2 | 29.06 | 22.07 | 29.00 | 22.13 |
| 3 | 29.49 | 21.64 | 29.42 | 21.70 |
| 4 | 26.12 | 25.01 | 25.99 | 25.13 |
| 5 | 28.26 | 22.87 | 28.14 | 22.99 |
| 6 | 28.69 | 22.44 | 28.57 | 22.56 |

Table C.6: Allowed OFDM symbols for IEEE 802.16m to co-exist with TD-LTE configuration 0 with extended cyclic prefix in downlink.

| Special subframe configuration | Extended cyclic prefix in downlink |  |  |  |
| --- | --- | --- | --- | --- |
|  | Allowed DL OFDM Symbols (NCP in UL) | Allowed UL OFDM Symbols (NCP in UL) | Allowed DL OFDM Symbols (ECP in UL) | Allowed UL OFDM Symbols (ECP in UL) |
| 0 | 36.78 | 14.35 | 36.72 | 14.41 |
| 1 | 38.92 | 12.21 | 38.86 | 12.27 |
| 2 | 39.35 | 11.78 | 39.29 | 11.84 |
| 3 | 39.78 | 11.35 | 39.72 | 11.41 |
| 4 | 36.41 | 14.72 | 36.29 | 14.84 |
| 5 | 38.55 | 12.57 | 38.43 | 12.70 |
| 6 | 38.98 | 12.14 | 38.86 | 12.27 |