| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of S-FH SP3 to include Femto BS Status Bit (Section 16.3.5.5.1.2) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Andreas Maeder, Linghang Fan, Hassan Alkanani, Nader Zein NEC | E-mail: andreas.maeder@nw.neclab.eu linghang.fan@eu.nec.com hassan.alkanani@eu.nec.com nader.zein@eu.nec.com |
| Re: | IEEE 802.16 Working Group Sponsor Ballot Recirculation on P802.16m/D9 |  |
| Abstract | Include Femto ABS status in S-FH SP3 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of S-FH SP3  to include Femto BS Status  Bit (Section 16.3.5.5.1.2)

Andreas Maeder, Linghang Fan, Hassan Alkanani, Nader Zein

NEC

# Motivation

It is beneficial for the network performance to include the status of Femto ABS in the broadcast system information. State refers here to “normal” and “LDM” (low-duty mode), as defined in Section 16.4.3.

The reason is that a macro ABS may want to “wake up” a co-located Femto ABS, i.e. notify it to transit from LDM to normal state, in order to perform a hand-over. Currently, an AMS is cannot directly extract the Femto ABS status from the system information. It has rather to “guess” it from the behavior of the on-off patterns of the Femto ABS transmission scheme. However, this would require heuristics of non-negligible complexity at the MS, increasing overall cost of operation.

Therefore, it is proposed to include a single bit status in the S-FH sub packet 3 to indicate the status of the Femto ABS at scanning time of the AMS.

Remedy:  Modify  table 834 as following

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| SCD Count | 4 | Indicates Configuration Change Count which is associated with the system configuration of the AAI-SCD. |
| Femto_BS_Status | 1 | Indicates current state of Femto BS. 0: normal mode 1: LDM |
| Reserved |  |  |
| } |  |  |

Corresponding ASN.1 code, starting  page 1040, line 57:

FemtoBsStatus ::= ENUMERATED {

normalMode,

ldm}

Note to editor: the same type definition is appearing in the latest revision of contribution C80216m_1302

SFHSubpacket3 ::= SEQUENCE {

ssfhChangeCycle INTEGER {

sixteen (0),

thirtyTwo (1),

sixtyFour (2)

} (0..15),

preambleSeqSoftPartitioning INTEGER (0..15),

ulIotCorrectionValues SEQUENCE {

iotCorrValueSounding IotCorrectionValue,

iotCorrValueFP0 IotCorrectionValue,

iotCorrValueFP1 IotCorrectionValue,

iotCorrValueFP2 IotCorrectionValue,

iotCorrValueFP3 IotCorrectionValue

},

ulFeedbackSize INTEGER (0..15),

numTxAntennas ENUMERATED {

two,

four,

eight

},

spSchedulingPeriodicity INTEGER (0..15),

hoRangingBackoffStart INTEGER (0..15),

hoRangingBackoffEnd INTEGER (0..15),

initialRangingBackoffStart INTEGER (0..15),

initialRangingBackoffEnd INTEGER (0..15),

ulBwReqChannelInfo INTEGER (0..7),

bandwidthRequestBackoffStart INTEGER (0..15),

bandwidthRequestBackoffEnd INTEGER (0..15),

fpPowerConfig INTEGER (0..7),

dLbsZoneConfig ENUMERATED {

noLbsZoneTransmission,

zonePeriodicity4,

zonePeriodicity16,

zonePeriodicity32

},

nspChangeCount INTEGER (0..15),

scdCount INTEGER (0..15),

femtoBSStatus FemtoBsStatus,

...

}