| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of AAI-SCN-REP Message to include Femto BS Status (Section 16.2.3.16) |  |
| Date Submitted | 2010-25-10 |  |
| Source(s) | Andreas Maeder, Linghang Fan, Hassan Alkanani, Nader Zein NEC | E-mail: andreas.maeder@nw.neclab.eu linghang.fan@eu.nec.com hassan.alkanani@eu.nec.com nader.zein@eu.nec.com |
| Re: | IEEE 802.16 Working Group Sponsor Ballot Recirculation on P802.16m/D9 |  |
| Abstract | Modification of AAI-SCN-REP to include Femto BS Status |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of AAI-SCN-REP Message to include Femto BS Status (Section 16.2.3.16)

Andreas Maeder, Linghang Fan, Hassan Alkanani, Nader Zein

NEC

# Motivation

In certain scenarios it is beneficial for the network performance to include the status of a scanned Femto ABS in the AAI-SCN-REP message for a macro ABS. State refers here to “normal” and “LDM” (low-duty mode), as defined in Section 16.4.3.

The reason is that a macro ABS may want to “wake up” a co-located Femto ABS, i.e. notify it to transit from LDM to normal state, in order to perform a hand-over. Since scanning is a passive operation, the MS cannot actively wake up a Femto ABS at this state, and this is also not required.

Remedy:  Modify  table 693 as following

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Neighbor_Request_Indication | 1 | Request indication for system information and the list of the neighbor Femto ABSs | Optional |
| If(Neighbor_Request_Indication == 1) { |  |  |  |
| Neighbor_Request_BS_Type | 3 | Indicates type of neighbor Femto ABSs for which system information is requested. Bit #0: CSG-Closed Femto ABS Bit #1: CSG-Open Femto ABS Bit #2: OSG Femto ABS | Optional |
| Femto_BS_State | 1 | Indicates current state of Femto BS. 0: normal mode 1: LDM | Optional |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs | Optional. Sent by AMS to aid in generation of optimized neighbor list by the ABS |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

Corresponding ASN.1 code:

AAI-SCN-REP ::= SEQUENCE {

reportModeENUMERATED { eventTriggerReport, periodicReport, oneTimeReport },

sabsCarrierReportListSEQUENCE (SIZE(0..63)) OF CarrierReport,

absIndexReportListSEQUENCE (SIZE(0..63)) OF ABSIndexReport,

absFullReportListSEQUENCE (SIZE(0..63)) OF ABSFullReport,

saPreambleReportListSEQUENCE (SIZE(0..15)) OF SAPreambleReport,

neighborRequestSEQUENCE {

requestBSTypeBIT STRING {csgClosedFemto(0),

csgOpenFemto(1),

osgFemto(2) }OPTIONAL,

femtoBSState INTEGER (0..1),

csgIDListSEQUENCE (SIZE(0..63)) OF CSGID

}OPTIONAL

}