| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of AAI-SCN-REP Message to include Femto BS Status (Section 16.2.3.16) |  |
| Date Submitted | 2010-25-10 |  |
| Source(s) | Andreas Maeder, Linghang Fan, Hassan Alkanani, Nader Zein NEC | E-mail: andreas.maeder@nw.neclab.eu linghang.fan@eu.nec.com hassan.alkanani@eu.nec.com nader.zein@eu.nec.com |
| Re: | IEEE 802.16 Working Group Sponsor Ballot Recirculation on P802.16m/D9 |  |
| Abstract | Modification of AAI-SCN-REP to include Femto BS Status |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of AAI-SCN-REP Message to include Femto BS Status (Section 16.2.3.16)

Andreas Maeder, Linghang Fan, Hassan Alkanani, Nader Zein

NEC

# Motivation

In certain scenarios it is beneficial for the network performance to include the status of a scanned Femto ABS in the AAI-SCN-REP message for a macro ABS. State refers here to “normal” and “LDM” (low-duty mode), as defined in Section 16.4.3.

Currently, if a hand-over towards a Femto ABS is performed, a macro ABS will wake-up the Femto ABS regardless of the current operational state of the Femto ABS. However, this may not always be beneficial for the interference level in the network and for energy consumption.

The following scenario describes the problem in more detail. Assumption is that in the coverage area of a macro ABS, two Femto ABS A and Femto ABS B are relatively closely located to each other. Furthermore, an AMS is approaching the vicinity of both Femto ABSs, as illustrated in Figure 1. The AMS performs scanning and sends an AAI-SCN-REP message to the macro ABS, which builds up a HO candidate list and selects the candidate with the strongest signal. This could be Femto ABS B which is in LDM. Therefore, the macro ABS will send a HO-Preparation message to Femto ABS B, which will switch to normal operation mode.

However, if both signal strengths are similar, the connection could also be served by Femto ABS A, which is already in normal operation mode. In such a case, it would be preferable that the macro ABS does not signal Femto ABS B in order to avoid high energy consumption and interference level in the network. In order to avoid this, the macro ABS needs the current operational modes of both Femto ABS to perform the decision in a timely manner. It is therefore beneficial if the current Femto ABS status is included in the AAI-SCN-REP message of the AMS.

Figure 1: Scenario with unnecessary HO to a Femto ABS in LDM

Remedy:  Modify  table 693 as following

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Neighbor_Request_Indication | 1 | Request indication for system information and the list of the neighbor Femto ABSs | Optional |
| If(Neighbor_Request_Indication == 1) { |  |  |  |
| Neighbor_Request_BS_Type | 3 | Indicates type of neighbor Femto ABSs for which system information is requested. Bit #0: CSG-Closed Femto ABS Bit #1: CSG-Open Femto ABS Bit #2: OSG Femto ABS | Optional |
| Femto_BS_State | 1 | Indicates current state of Femto BS. 0: normal mode 1: LDM | Optional |
| for (j=0; j<N_CSG_IDs; j++) { |  | N_CSG_IDs is the number of CSG IDs | Optional. Sent by AMS to aid in generation of optimized neighbor list by the ABS |
| CSGID | variable | The CSGID within the Operator ID. It may be part of the BS ID, with certain bits inside indicating its length. If the CSG has single BS, it may be of maximum length which is the LSB-24 bits of the full BS ID. |  |
| } |  |  |  |
| } |  |  |  |

Corresponding ASN.1 code, starting page 1048, line 15

FemtoBsStatus ::= ENUMERATED {

normalMode,

ldm}

Note to editor: the same type definition is appearing in the latest revision of contribution C80216m_1301.

AAI-SCN-REP ::= SEQUENCE {

reportModeENUMERATED { eventTriggerReport, periodicReport, oneTimeReport },

sabsCarrierReportListSEQUENCE (SIZE(0..63)) OF CarrierReport,

absIndexReportListSEQUENCE (SIZE(0..63)) OF ABSIndexReport,

absFullReportListSEQUENCE (SIZE(0..63)) OF ABSFullReport,

saPreambleReportListSEQUENCE (SIZE(0..15)) OF SAPreambleReport,

neighborRequestSEQUENCE {

requestBSTypeBIT STRING {csgClosedFemto(0),

csgOpenFemto(1),

osgFemto(2) }OPTIONAL,

femtoBSStatus FemtoBsStatus,

csgIDListSEQUENCE (SIZE(0..63)) OF CSGID

}OPTIONAL

}