| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Uplink Pilot Power (16.3.10.3) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Sangheon Kim, Jeongho Park, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | sangheon.kim@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE 802.16m/D9 |  |
| Abstract | The contribution proposes the change to boosting level of uplink pilot of the 802.16m draft amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m/D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Uplink Pilot Power (16.3.10.3)

Sangheon Kim, Jeongho Park, Jaeweon Cho, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

In the Session 68.5, the pilot boosting for uplink DLRU with single stream was proposed and accepted only for SU-MIMO. However, according to the modified table in the contribution, the pilot power is boosted for uplink CSM as well. Therefore, this contribution proposes to correct and clarify the pilot boosting level for uplink transmission.

- References

[1] IEEE P802.16m/D9, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, Oct. 2010.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Modify Table 958 on page 877 line 1 as following]

Table 958-Uplink data and pilot subcarriers power

| number of streams Mt | per stream power on data subcarriers Bd | per stream pilot power Bp | per stream pilot boosting level, dB |
| --- | --- | --- | --- |
| 1 | 1 | 1.5849*Mt | 2+10*log10(Mt) |
| 2-4 |  | Mt | 10*log10(Mt) |

|  | number of streams Mt | per stream power on data subcarriers Bd | per stream pilot power Bp | per stream pilot boosting level, dB |
| --- | --- | --- | --- | --- |
| SU-MIMO | 1 | 1 | 1.5849*Mt | 2+10*log10(Mt) |
|  | 2-4 |  | Mt | 10*log10(Mt) |
| UL CSM | 1-4 |  | Mt | 10*log10(Mt) |

---------------------------------------------------  Text End  ---------------------------------------------------