| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the SA-Preamble boosting levels for tone dropping support (16.3.5.1.2) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Chiwoo Lim, Sangheon Kim, Sung-Eun Park, Jaeweon Cho, Heewon Kang Samsung Electronics Co., Ltd. | chiwoo.lim@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE 802.16m/D9 |  |
| Abstract | The contribution proposes the text changes to SA-Preamble boosting levels for tone dropping support in multi-carrier mode of the 802.16m draft amendment. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the SA-Preamble boosting levels for tone dropping support (16.3.5.1.2)

Chiwoo Lim, Sangheon Kim, Sung-Eun Park, Jaeweon Cho, Heewon Kang
Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of the SA-Preamble boosting levels for tone dropping support to be included in the 802.16m amendment.

In the current draft amendment [1], the SA-Preamble boosting level for tone dropping support is no consideration for multi-carrier mode.

In calculation of boosting level, it’s chosen such that the maximum peak value of SA-Preamble in time domain should not exceed the peak value of 16e preamble which has 9dB boosting. However, in the multi-carrier, the peak powers of SA-Preamble with the present boosting levels are exceeded the dynamic range of ABS’s amplifier (which is same as 98%-tile value that is CDF of the peak values of data symbols). So, it’s needed to consider multi-carrier mode. In previous meeting, A-Preamble boosting levels were already modified according to this consideration.

- References

[1] IEEE P802.16m/D9, “P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, 2010-10-06

- Rationale of modifications

In D9 [1], A-Preamble boosting levels are chosen such that the maximum peak value of SA-Preamble in the single-carrier case. However, in the multi-carrier case, the peak powers of SA-Preamble with the present boosting levels are exceeded the dynamic range of ABS’s amplifier (which is same as 98%-tile value that is CDF of the peak values of data symbols).

Simulation results show that the peak powers of SA-Preamble with the present boosting levels are exceeded the 98%-tile value that is CDF of the peak values of data symbols in the multi-carrier. Therefore, the modification of boosting values is required. The modified boosting levels for SA-Preamble are chosen by following rules.

- The maximum peak value of SA-Preamble is same as 98%-tile value that is CDF of the peak values of data symbols.

- For 512 and 1024 FFT cases, the modified value is chosen at the carrier aggregation mode with 3 carriers.

- For 2048 FFT case, the modified value is chosen at the carrier aggregation mode with 2 carriers.

- Proposed changes for 802.16m draft amendment

--------------------------------------------------- Text Start ---------------------------------------------------

[Remedy-1: Modify the table 825 on page 610 as following]

Table 825XXX— SA-Preamble boosting levels for tone dropping support

| BWmin\Ant | 1 | 2 | 4 | 8 |
| --- | --- | --- | --- | --- |
| 6.25 | 1.85 1.56 | 2.61 2.22 | 3.80 3.09 | 5.30 4.26 |
| 7.5 | 1.90 1.60 | 2.67 2.27 | 4.36 3.54 | 6.12 4.92 |
| 8.75 | 1.92 1.62 | 2.59 2.20 | 3.77 3.06 | 6.52 5.24 |
| 11.25 | 1.88 1.71 | 2.63 2.34 | 3.78 3.04 | 5.49 5.18 |
| 12.5 | 1.97 1.80 | 2.73 2.43 | 4.12 3.31 | 6.00 5.66 |
| 13.75 | 1.96 1.79 | 2.74 2.44 | 3.83 3.08 | 6.18 5.83 |
| 15 | 2.00 1.82 | 2.83 2.52 | 4.15 3.33 | 6.69 6.32 |
| 16.25 | 1.97 1.80 | 2.91 2.59 | 3.93 3.16 | 5.71 5.39 |
| 17.5 | 2.04 1.86 | 3.04 2.71 | 4.21 3.38 | 6.12 5.78 |
| 18.75 | 2.00 1.82 | 2.99 2.66 | 4.06 3.26 | 6.29 5.94 |

--------------------------------------------------- Text End ---------------------------------------------------