| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on Calibration Sounding Channel (16.5.1.3.1) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Tsung-Yu Tsai, Chun-Che Chien, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Chun-Yen Hsu, Chiu-wen Chen Institute for Information Industry Whai-En Chen, Shih-Yuan Cheng, National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang, National Central University Yang-Han Lee, Yih Guang Jan, Hsien-Wei Tseng Tamkang University | Voice: +886-2-66000100 E-mail: tyt@.nmi.iii.org.tw |
| Re: | Sponsor ballot IEEE P802.16m/D9 : 16.5.1.3.1 |  |
| Abstract | This contribution proposes an improvement for current sounding based calibration scheme. |  |
| Purpose | To be discussed in the IEEE 802.16m working group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on Calibration Sounding Channel (16.5.1.3.1)

Tsung-Yu Tsai, Chun-Che Chien, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Chun-Yen Hsu, Chiu-wen Chen, (III)

Whai-En Chen ,Shih-Yuan Cheng (Ilan University)

Shiann-Tsong Sheu, Chih-Cheng Yang (National Central University)

Yang-Han Lee, Yih Guang Jan, Hsien-Wei Tseng  (Tamkang University)

# Introduction

The contribution C80216m-10/1136r4 (resolved by comment # B187) in the previous session (#69 in St. Petersburg) had been accepted. However, the text modifications (Remedy #1) were not fully implemented.

In this contribution, based on IEEE P802.16m/D9, the missing approved text in C80216m-10/1136r4 is reappeared here without extra modification.

# Proposed Modificatio

Proposed text Remedy # 1

16.5.1.3 DL Multi-BS Joint MIMO Processing

16.5.1.3.1 Operation procedure

[Insert the following text in line 45, page 906 of P80216m_D9]

For UL sounding based DL CL-MD and Co-MIMO operation, the sounding sequence can be used for DL/UL channel the calibration of TX/RX RF phase mismatch caused by the hardware or over the air (OTA) UL/DL channel phase mismatch caused by the non-ideal channel reciprocity of UL and DL channels involved in DL Multi-BS joint MIMO processing. The calibration process is triggered and setup by AAI_MULTI_BS_SOUNDING-CAL.

Upon receiving AAI_MULTI_BS_SOUNDING-CAL with calibration mode = 0, the AMS shall send the calibration sounding sequence obtained by replacing bk in equation (277) by , where is the phase of downlink channel on the k-th subcarrier of the x-th collaborative ABS (from the first antenna at ABS to the first antenna at AMS) measured via downlink midamble.

ABS uses phases of the received UL calibration sounding sequence as UL TX and DL RX channel RF phase mismatch (i.e. , with being UL channel phase). Each collaborative ABS compensates DL TX and UL RX channel RF phase mismatch.

For the purpose of DL/UL OTAchannel phase calibration, upon receiving AAI_MULTI_BS_SOUNDING-CAL with calibration mode = 1, the AMS shall send the calibration sounding sequence obtained by replacing bk in equation (277) by ,where is the phase of downlink channel of the a-th adjacent ABS (other than the serving ABS) involved in DL Multi-BS joint MIMO processing on the k-th subcarrier, and is the phase of the serving ABS’s downlink channel on the k-th subcarrier. The adjacent ABSs (other than the serving ABS) use phase of the received UL calibration sounding sequence to get the phase difference of OTA UL and OTA DL channel phase plus the serving ABS OTA DL channel phase(i.e. ).

[End of proposed text]