| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to Cleanup MAC Support for Relay Operation (16.6) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | E-mails: alexander.maltsev@intel,com alexey.khoryaev@intel.com |
| Re: | Comments on IEEE P802.16m/D9 for IEEE 802.16 Working Group Sponsor Ballot Topic: Relay (Section 16.6) |  |
| Abstract | This contribution proposes technical and editorial changes to make cleanup in Relay support MAC sections. |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m/D10. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| *Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text to Cleanup MAC Support for Relay Operation

Alexander Maltsev, Alexey Khoryaev, 
Andrey Chervyakov and Mikhail Shilov

Intel Corporation

- Introduction

This contribution provides technical and editorial correction of the text on MAC support in case of Relay operation for Draft 9 of the IEEE 802.16m amendment. The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [1] and the IEEE 802.16m SDD [2], and it follows the style and format guidelines in [3].

The contribution provides following changes:

- It is proposed that AAI-ARS-CONFIG-CMD message is used only for TTR ARS mode.

- The RS-ESI message is renamed to AAI-ARS-ESI and moved from Relay section to MAC message control section.

- The clarification on the relevant to ARS operation fields is provided for a number of MAC control messages to make the current text clean and consistent.

- References

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D9, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D9 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Modify the text in Section 16.6.2.14 ARS Configuration (page 925, line 36) as follows]

16.6.2.14 ARS Configuration

In TTR mode the ARS configuration is based on the transmission of a specific AAI-ARS-CONFIG-CMD message and described in Sections 16.6.2.14.1 and 16.6.2.14.2.

In STR mode the configuration procedures defined for configuration of AMS by ABS shall be used for ARS as well.

16.6.2.14.1 Parameter configuration during TTR ARS network entry

After ARS registration, the ABS shall send a AAI_ARS-CONFIG-CMD message to configure the PHY layer operational parameters for relay operation. When an ARS receives AAI_ARS-CONFIG-CMD message, it shall transmit an AAI_MSG-ACK message. The ARS shall start RS operation and apply the PHY operational parameters from the time specified by 'Super-frame number action' in AAI_ARS-CONFIG-CMD.

16.6.2.14.2 Parameter configuration update for TTR ARS operational mode

During ARS operation mode, when the PHY layer operational parameters for relay operation is changed, an ABS shall send AAI_ARS-CONFIG-CMD message. When an ARS receives AAI_ARS-CONFIG-CMD message, it shall transmit an AAI_MSG-ACK message. The ARS can use Message ACK Extended Header as well as the AAI_MSG-ACK message. The ARS shall apply the system information in SFH or the configuration parameters from the time specified by 'Super-frame number action' in AAI_ARS-CONFIG-CMD.

========================= End of Proposed Text 1==============================

=========================Start of Proposed Text 2 ==============================

[Note to Editor: Modify the text in 16.6.2.16.2 Update of SFH information during ARS operational mode (page 926, line 26) as follows]

16.6.2.16.2 Update of SFH information during ARS operational mode

In TTR mode the ABS shall notify an ARS of SFH information change via a dedicated AAI-ARS-ESI control message. When the essential system information in SFH is changed, an ABS shall send the information through a AAI-ARS-ESI RS_ESI control message in the AAI DL Relay Zone. If an ARS receives AAI-ARS-ESI RS_ESI message, it shall transmit AAI-MSG-ACK AAI_MSG-ACK message. The ARS shall apply the information from the super-frame specified by Super-Frame Number Action in the AAI-ARS-ESI RS_ESI.

In STR mode an ARS shall perform an update of SFH as described in the subclause 16.2.24.

========================= End of Proposed Text 2 ==============================

=========================Start of Proposed Text 3 ==============================

[Note to Editor: 1) Insert new section after section 16.2.3.57 AAI-ARS-CONFIG-CMD message format at page 277, line 17 2) Move Table  971—RS_ESI message format from page 926, line 36 to this section as follows]

16.2.3.XX AAI-ARS-ESI Message

An ABS shall use AAI-ARS-ESI message to perform update of SFH information during ARS operational mode.

Table XXX— AAI-ARS-ESI Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| [Insert contents of RS_ESI message here] |  |  |  |

========================= End of Proposed Text 3 ==============================

=========================Start of Proposed Text 4 ==============================

[Note to Editor: Modify the text in Table 682—AAI-SBC-REQ message Field Description (page 109, line 27) as follows]

Table 682 – AAI-SBC-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If (ARS is a sender of AAI-SBC-REQ) { |  |  | //only available during ARS network entry phase |
| Relay mode | 1 | 0b0: TTR relay mode 0b1: STR relay mode | Optional. Present when ARS is a sender of AAI-SBC-REQ |
| if (Relay mode == 0b0){ |  |  |  |
| ARSTTG | 6 | ARSTTG value (μs). It shall be less than 50μs. | Optional. Present when ARS is a sender of AAI-SBC-REQ and for TTR Relay mode only (Relay mode == 0b0). |
| ARSRTG | 6 | ARSRTG value (μs). It shall be less than 50μs. | Optional. Present when ARS is a sender of AAI-SBC-REQ and for TTR Relay mode only (Relay mode == 0b0). |
| } |  |  |  |
| Relay mode | 1 | 0b0: TTR relay 0b1: STR relay |  |
| } |  |  |  |
| } |  |  |  |

========================= End of Proposed Text 4 ==============================

=========================Start of Proposed Text 5 ==============================

[Note to Editor: Modify the text in Table 708—AAI-SCD Message Field Description (page 188, line 60) as follows]

Table 708 – AAI-SCD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| If (relay is supported) { |  |  |  |
| If (ABS is the sender of AAI-SCD and ABS has attached TTR ARS in the cell){ |  |  |  |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | 0 – 1. 0b0: the ABS does not allocate resources to the AMS in the AAI DL Relay zone; 0b1: the ABS may allocate resources to the AMS in the AAI DL Relay zone. | Optional. Present when ABS is the sender of AAI-SCD and ABS has attached TTR ARS in the cell. |
| } |  |  |  |

========================= End of Proposed Text 5 ==============================

=========================Start of Proposed Text 6 ==============================

[Note to Editor: Modify the text in Section 16.2.3.57 AAI-ARS-CONFIG-CMD message format (page 275, line 44) as follows]

16.2.3.57 AAI-ARS-CONFIG-CMD message format

An ABS shall use AAI-ARS-CONFIG-CMD message to configure the TTR mode ARS PHY layer operational parameters.

Table 751—AAI-ARS-CONFIG-CMD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | 0b0: the ABS does not allocate resources to the AMS in the AAI DL Relay zone; 0b1: the ABS may allocate resources to the AMS in the AAI DL Relay zone. The indicator which signalizes if the ABS may allocate resources to the AMS in the AAI DL Relay zone and AAI UL Relay zone. It has 1 bit to represent the values {0, 1}. If it is set to 1 then the AMSs are allocated in Relay zones | Mandatory. |
| MIMO Midamble indication in AAI DL Relay zone | 1 | 0b0: MIMO midamble is not transmitted in AAI DL Relay zone 0b1: MIMO midamble is transmitted in AAI DL Relay zone | Mandatory. |
| If(AAI _Relay_zone_AMS_allocation_indicator == 0b0){ |  |  |  |
| R_DCASSB0 | 5/4/3 | AAI _DL_Relay_zone_Cell_Specific_Parameter Indicates the number of subband-based CRUs in FP0 for AAI DL Relay zone. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Optional. Present when AAI _Relay_zone_AMS_allocation_indicator == 0b0 |
| R_DCASMB0 | 5/4/3 | AAI _DL_Relay_zone_Cell_Specific_Parameter Indicates the number of miniband-based CRUs in FP0 for AAI DL Relay zone. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Optional. Present when AAI _Relay_zone_AMS_allocation_indicator == 0b0 |
| R_DCASi | 3/2/1 | AAI _DL_Relay_zone_Cell_Specific_Parameter Indicates the number of total allocated CRUs, in a unit of a subbands, for FPi (i>0) for AAI DL Relay zone. See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Optional. Present when AAI _Relay_zone_AMS_allocation_indicator == 0b0 |
| R_UCASSB0 | 5/4/3 | AAI _UL_Relay_zone_Cell_Specific_Parameter Indicates the number of subband-based CRUs in FP0 for AAI UL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Optional. Present when AAI _Relay_zone_AMS_allocation_indicator == 0b0 |
| R_UCASMB0 | 5/4/3 | AAI _UL_Relay_zone_Cell_Specific_Parameter Indicates the number of miniband-based CRUs in FP0 for AAI UL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits | Optional. Present when AAI _Relay_zone_AMS_allocation_indicator == 0b0 |
| R_UCASi | 3/2/1 | AAI _UL_Relay_zone_Cell_Specific_Parameter Indicates the number of total allocated CRUs, in a unit of a subbands, for FPi (i>0) for AAI UL Relay zone. See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit | Optional. Present when AAI _Relay_zone_AMS_allocation_indicator == 0b0 |
| } |  |  |  |

========================= End of Proposed Text 6 ==============================