| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to Cleanup Relay Uplink Physical Structure (16.6.3.5) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov Intel Corporation | E-mails: alexander.maltsev@intel,com alexey.khoryaev@intel.com |
| Re: | Comments on IEEE P802.16m/D9 for IEEE 802.16 Working Group Sponsor Ballot Topic: Relay (Section 16.6) |  |
| Abstract | This contribution proposes technical and editorial changes to make cleanup in Relay Uplink Physical Structure section. |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m/D10. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| *Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text to Cleanup Relay Uplink Physical Structure

Alexander Maltsev, Alexey Khoryaev, 
Andrey Chervyakov and Mikhail Shilov

Intel Corporation

- Introduction

This contribution provides technical and editorial correction of the D9 text [4] in section 16.6.3.5 (Relay Uplink physical structure). The proposed text is developed so, that it is compliant to the IEEE 802.16m SRD [1] and the IEEE 802.16m SDD [2], and it follows the style and format guidelines in [3].

The contribution provides the following changes to the existing text:

- Editorial cleanup for the text in section 16.6.3.5 (e.g. correction of the names of MAC messages, typos)

- The text describing relay operation for FDD frame structure with AAI Relay zone duration equal to one subframe is deleted since there is no such frame configuration supported for TTR ARS and therefore the current text in not relevant anymore.

- References

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/0034r2, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D9, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Proposed Text

Instruction to editor:

Black text: existing D9 text

Red and strike out text: to be deleted

Blue and underline text: to be added

=========================Start of Proposed Text 1 ==============================

[Note to Editor: Modify the text in Section 16.6.3.5 Relay Uplink Physical Structure (page 935, line 3) as follows]

16.6.3.4.1 Advanced preamble for relay

An ARS shall reuses the same PA-preamble and SA-preamble structures as defined in 16.3.5.1.

16.6.3.5 Relay Uplink physical structure

16.6.3.5.1 Cell-specific resource mapping

In the AAI UL Relay zone of ABS and ARS frames, the cell-specific resource mapping shall be performed in accordance to the procedure described in the Section 16.3.4.3. If AAI_Relay_zone_AMS_allocation_indicator field signaled in the AAI-SCD AAI_System Configuration Descriptor and AAI-AAI_ARS-CONFIG-CMD message is equal to 1 which indicates that ABS may allocate AMS transmissions in the AAI Relay zone, then the values of UCASSB,0, UCASi, UCASMB,0 used in the AAI UL Access zone shall be used for cell-specific resource mapping in the AAI UL Relay zones of ABS and ARS frames.

If AAI_Relay_zone_AMS_allocation_indicator field signaled in the AAI-SCD AAI_System Configuration Descriptor and AAI_ARS-CONFIG-CMD message is equal to 0 which indicates that ABS does not allocate resources for AMS transmissions in the AAI Relay zone, then the values of UCASSB,0, UCASi, UCASMB,0 used for cell-specific resource mapping in AAI UL Relay zones of ABS and ARS frames shall be set to the values R_UCASSB,0, R_UCASi, R_UCASMB,0 correspondingly. The values of cell specific AAI Relay zone parameters R_UCASSB,0, R_UCASi, R_UCASMB,0 are explicitly signaled by ABS in the AAI_ARS-CONFIG-CMD message.

16.6.3.5.2 Uplink data subcarrier mapping

The data subcarrier mapping rule in UL Relay zone shall follow description corresponds to the rule in 16.3.9.2.3.

In the ARS TDD and FDD frames, to support an R-RTI insertion in the first OFDMA symbol of the first AAI subframe of the AAI UL Relay zone, the LRUs of the corresponding AAI subframe which are used for data transmissions between ABS and ARS shall be modified in the following way. For the six-symbol PRU case, the remaining 5 OFDMA symbols are formed to be a five-symbol PRU used for the data transmission. For the seven-symbol PRU case, the remaining 6 OFDMA symbols are formed to be a six-symbol PRU for the data transmission.

In the ARS TDD and FDD frames, when R-RTI is equal to Ts, the LRUs of the first AAI subframe of the AAI UL Relay zone used for communication between ABS and ARS shall be transformed as follows. For the six-symbol PRU case, the remaining 5 OFDMA symbols are formed to be a five-symbol PRU used for the data transmission. For the seven-symbol PRU case, the remaining 6 OFDMA symbols are formed to be a six-symbol PRU for the data transmission.

In the ARS FDD, to support an R-TTI insertion in the last OFDMA symbol of the last AAI subframe of the AAI UL Relay zone, the LRUs of the corresponding AAI subframe which are used for data transmissions between ABS and ARS shall be modified in the following way. For the six-symbol PRU case, the remaining 5 OFDMA symbols are formed to be a five-symbol PRU used for the data transmission. For the seven-symbol PRU case, the remaining 6 OFDMA symbols are formed to be a six-symbol PRU for the data transmission.

In the ARS FDD frames, when the AAI UL Relay zone occupies a single AAI subframe, to support a joint insertion of a R-RTI and a R-TTI in the first and the last OFDMA symbols, the LRUs of the corresponding AAI subframe which are used for the data transmission shall be modified in the following way. For the seven-symbol PRU case, the remaining 5 OFDMA symbols are formed to be a five-symbol PRU for the data transmission. The FDD frame structure configurations with the AAI UL Relay zone which comprise of a single type-1 AAI subframe shall not used if both a R-TTI and a R-RTI are present.

16.6.3.6 Uplink Control Structure

In both AAI UL Access zones of ABS/ and ARS frames, all UL control channels described in 16.3.8 are reused.

To support When R-RTI and or R-TTI transition intervals in the AAI UL Relay zone are equal to TS insertion in the AAI UL Relay zone the ARS shall puncture perform resource mapping of fast feedback and BR control channels described in 16.3.89 but shall not transmit them on subcarriers that belong to the OFDMA symbol of the control channel of the AAI subframe where the R-RTI or R-TTI is placed. The ABS shall process the ARS UL control signals, assuming that ARS has punctured the subcarriers and pilots that belong to the OFDMA symbol of the AAI subframe where the R-RTI or R-TTI is placed.

The UL BR channel for ARS is transmitted separately with the UL BR channel for AMS connected to ABS., and t

The UL HARQ and fast feedback channels transmitted by ARS to ABS are transmitted in the same region with those for AMS connected to ABS. In this case, the UL HARQ feedback channel for from ARS is transmitted by puncturing one HMT described in 16.3.8.2.2. In case when the transition interval is inserted in the first OFDMA symbol of the control channel, HMT in the first and second symbols shall be punctured. In case when the transition interval is inserted in the last OFDMA symbol of the control channel, HMT in the last two symbols shall be punctured.

If an R-RTI is present equal to TS, then the ARS shall not use sounding channel in the first AAI subframe of the AAI UL Relay zone. In the ARS FDD frames, if an R-TTI is present equal to TS and the last AAI subframe of the AAI UL Relay zone is a type-1 subframe, then the ABS shall not allocate sounding channel in this AAI subframe.

=========================End of Proposed Text 1 ==============================