| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for flow mapping for persistent scheduling (16.2.7) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Seunghyun Kang, Kiseon Ryu and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com , seunghyun.kang@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to UL persistent allocation |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for flow mapping for persistent scheduling (16.2.7)

Jeongki Kim, Youngsoo Yuk, Seunghyun Kang, Kiseon Ryu and Jin Sam Kwak

- LG Electronics

# Introduction

Because a persistent allocation is intended for a certain service flow, a PA should be usually used for the specified service flow. However, when more urgent traffic (e.g., emergency, etc.) than the traffic on the service flow happens, the urgent traffic should be able to be carried on the PA. The method to solve this issue should minimize the resource utilization effect.

- References

[1] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested Remedy #1: Add the following rows at the end of Table 734, on page 234, line 42]

Table 734 – AAI_DSA-REQ message

| Fields | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| ……….. | .... | ……… | ……….. |
| For (i=0; i<Number of HARQ channels; i++) { |  | Num of HARQ Channels the number of HARQ channels which are assigned to the flow [1..256] |  |
| HARQ Channel Mapping(ACID) | 5 | Indicates the index (ACID) of each HARQ channel. |  |
| } |  |  |  |

[Suggested Remedy #2: Add the following rows at the end of Table 737, on page 251, line 42]

Table 737 – AAI_DSC-REQ message

| Fields | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| ……….. | .... | ……… | ……….. |
| For (i=0; i<Number of HARQ channels; i++) { |  | Num of HARQ Channels the number of HARQ channels which are assigned to the flow [1..256] |  |
| HARQ Channel Mapping(ACID) | 5 | Indicates the index (ACID) of each HARQ channel. |  |
| } |  |  |  |

[Suggested Remedy #3: Add the following section at the end of the Section 16.2.12.12.6, on page 429, line 6]

16.2.12.12.7 HARQ channel mapping

This parameter specifies the set of HARQ channels that is intended to be used for carrying data on this service flow. The HARQ channels may be used either for carrying data on more urgent other service flow (e.g., emergency, more delay-sensitive service, etc.) than this service flow although there are data on this service flow or for carrying data on other service flows when there is no data on this service flow. The set of HARQ channels mapped to a certain flow shall be disjoint from sets of HARQ channels mapped to other flows.

--------------Proposed Texts  ---------------------------------------------------------------------------