| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text to clean up E-MBS parameters in DSC message (16.2.3.47.4) |  |
| Date Submitted | 2010-11-08 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak LG Electronics | E-mail: jeongki.kim@lge.com, youngsoo.yuk@lge.com , kiseon.ryu@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to E-MBS parameters in DSC message |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text to clean up E-MBS parameters in DSC message (16.2.3.47.4)

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jin Sam Kwak

- LG Electronics

# Introduction

When E-MBS ID and FID information for an AMS needs to be updated, the ABS will transmit the AAI_DSC-REQ message including the updated E-MBS ID and FID flow mapping information to the AMS. In this case, the ABS may change unicast available intervals for the AMS based the updated information. However, although the E-MBS ID and FID mapping information is changed, if the E-MBS bursts transmission for changed E-MBS connections are available in previous E-MBS available interval, an ABS may not change the unicast available interval bitmap. When ABS change the unicast available interval bitmap is out of scope due to ABS implementation issue.

In addition, the “Carrier switching mode” and “Unicast available interval bitmap” fields are not E-MBS Zone-specific but AMS-specific. Therefore, their position in the table 737 should be changed as the proposed text.

# Text proposal for the IEEE 802.16m amendment

--------------Proposed Texts ---------------------------------------------------------------------------

[Suggested Remedy#1: Modify Table 737 as follows, on page 246, line 36:]

Table 737 – AAI_DSC_REQ Message Field Description

| Field | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| …… | … | …… | … |
| for(i=1;i<=Num of E-MBS Zone ID to update;i++) { |  | Num of E-MBS Zone ID [1..8] is the number of E-MBS ID to update | Present when ABS initiates AAI-DSC-REQ Present only if Num of E-MBS Zone ID > 0 |
| Service_flow_update_indicator | 1 | Indicator whether "bitmap + new service flow" or "current service flow + new service flow" is included. 0b0: "Service Flow Update Bitmap + New_E-MBS ID and FID" 0b1: "Current_ E-MBS ID and FID and New_ E-MBS ID and FID Mappings" |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| New E-MBS_Zone_ID | 7 | Indicates an E-MBS zone to update | Present only if the E-MBS Zone ID needs to update |
| Physical Carrier Index | 6 | The physical carrier index that is support for the new E-MBS Zone ID to update | Present only if the New E-MBS Zone ID to update is served on a different carrier from the previous one |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSC message 0b1: carrier switching parameters using AAI_E-MBS-REP message | In case of ABS-initiated DSC for carrier switching to update |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | In case of ABS-initiated DSC for carrier switching to update Shall be present if Carrier Switching Mode = 0b0 |
| If (Service_flow_update_indicator == 0b0) { |  |  |  |
| Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. |  |
| for(m=0:m<N_ServiceFlowUpdateBitmap;m++){ |  | N_ServiceFlowUpdateBitmap is the number of bits to be set to 1 of Service Flow Update Bitmap For each bit in the Service Flow Update Bitmap = 1 to indicate the corresponding service flow is updated Range: 0~15 | Presented only if the number of bits set to 1 in Service Flow Update Bitmap > 1 |
| New_E-MBS ID | 12 | New E-MBS identifier. |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| }else if (Service_flow_update_indicator == 0b1) { |  |  |  |
| for(k=0;k<N_E-MBS_FID_Mapping;k++) { |  | N_E-MBS_FID_Mapping is the number of mappings of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| Current E-MBS ID | 12 | current E-MBS identifier |  |
| Current FID | 4 | Current flow ID |  |
| New_E-MBS ID | 12 | New E-MBS identifier |  |
| New FID | 4 | New flow ID |  |
| } |  |  |  |
| } // end of for |  |  |  |
| }// end of for |  |  |  |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval in the AAI-DSC message. 0b1: carrier switching parameters using AAI_E-MBS-REP message | Shall be present when the Unicast Available Interval needs to be updated |
| If (Carrier switching Mode = 0b0) { |  |  |  |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits NMSI = 32 superframes: N = 32 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit |  |
| } |  |  |  |
| If (Group Parameter Create/Change is needed) { |  |  |  |
| … | … | … | … |

[Suggested Remedy#2: Modify the ASN.1 code for AAI-DSC-REQ as follows, on page 1016, the line#51 to on page 1018, the line #38.]

AAI-DSC-REQ ::= SEQUENCE {

fidChangeCount FidChangeCount,

absInitDscInfo AbsInitDscInfo OPTIONAL,

directionIndicator DirIndicator,

serviceClassName ServiceClassName OPTIONAL,

globalServiceClass GlobalServiceClassName OPTIONAL,

qosParameterSet QosParameterSetType OPTIONAL,

qoSParameters QosParameter OPTIONAL,

sduInterArrival Interval OPTIONAL, -- 0.5ms

timeBase Interval OPTIONAL, -- ms

classifierDSCAction ClassifierDSCAction OPTIONAL,

classificationRules ClassificationRule OPTIONAL,

ethernetAttributes EthernetAttributes OPTIONAL,

rohcAttributes RohcAttributes OPTIONAL,

packetErrorRate PacketErrorRate OPTIONAL,

emergencyIndication BOOLEAN OPTIONAL,

embsService EMBSService OPTIONAL,

fullEMBSIDArray SEQUENCE (SIZE(1..8)) OF SEQUENCE {

embsZoneID EMBSZoneID,

newEmbsZoneID EMBSZoneID,

carrierIndex PhyCarrierIndex,

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE(4)),

nmsi4 BIT STRING (SIZE(4)),

nmsi8 BIT STRING (SIZE(8)),

nmsi16 BIT STRING (SIZE(16))} OPTIONAL,

serviceFlowUpdateType CHOICE {

bitmapAndNew SEQUENCE {

serviceFlowUpdateBitmap BIT STRING (SIZE(16)),

embsidFIDMappingArray SEQUENCE (SIZE(1..16)) OF SEQUENCE {

newEMBSID EMBSID,

newFID FID},

currentAndNew SEQUENCE (SIZE(1..16)) OF SEQUENCE {

currentEMBSID EMBSID,

currentFID FID,

newEMBSID EMBSID,

newFID FID}

}

}

},

unicastAvailIntervalBitmap CHOICE {

nmsi2 BIT STRING (SIZE(4)),

nmsi4 BIT STRING (SIZE(8)),

nmsi8 BIT STRING (SIZE(16)),

nmsi16 BIT STRING (SIZE(32))} OPTIONAL,

groupParameterCreateChange SEQUENCE {

-- ???? this part of the table is obscure

commonQosParameters QosParameter OPTIONAL,

qtySfid INTEGER (0..255) OPTIONAL,

fidArray SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters QosParameter OPTIONAL}

} OPTIONAL,

coupledGroupCreateChange SEQUENCE {

-- ??? this part of the table is obscure

commonQoSParameters QosParameter OPTIONAL,

qtyCoupledSfid INTEGER (0..255) OPTIONAL,

coupledFidArray SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL},

couplednonCommonFidArray SEQUENCE (SIZE(1..16)) OF SEQUENCE {

fid INTEGER (0..15) OPTIONAL,

nonCommonQosParameters QosParameter OPTIONAL}

} OPTIONAL,

multicastGroupAddition SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupAddedId INTEGER (0..4095)} OPTIONAL, -- multicast group id to be added

multicastGroupDeletion SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupDeletedId INTEGER (0..4095)} OPTIONAL, -- multicast group id to be deleted

sleepCycleSetting SleepCycleSetting OPTIONAL,

initialBackoffWindowSize INTEGER (0..15) OPTIONAL,

maxBackoffWindowSize INTEGER (0..15) OPTIONAL,

backoffScalingFactor INTEGER (0..15) OPTIONAL,

…

}

--------------Proposed Texts  ---------------------------------------------------------------------------