| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification Text on BR Header Transmission (Section 16.2.2.1) |  |
| Date Submitted | 2010-11-07 |  |
| Source(s) | Hyunkyu Yu, Jeongho Park, Sangheon Kim Yeongmoon Son Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D9 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies the text on BR header transmission. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification Text on BR Header Transmission (Section 16.2.2.1)

Hyunkyu Yu, Jeongho Park, Sangheon Kim, Yeongmoon Son

Samsung Electronics

- Introduction

- Padding bytes transmission:

In coverage loss detection section (16.2.26.1), there is a text specifying “Upon each expiration of the active_ABS_timer, to check whether an AMS is still alive in active mode, the ABS shall grant UL burst to the AMS and the AMS shall transmit a MAC PDU with data or if no data pending to be transmitted, then just with a BR header with BR size set to 0 on the UL grant.”.

Because the AMS does not know whether this UL grant is for coverage loss detection or not, for general cases (regardless of coverage detection), the AMS must transmit a BR header with BR size set to 0 if no data pending to be transmitted.

Furthermore, the transmission of padding bytes in D8 was changed to the transmission of a BR header with BR size set to 0 in the session #69. However, BR header transmission causes ambiguity: for example, which “BR FID” should be used in this case?

Thus, we’re proposing to add general description in Padding section to clarify the operation above: “In active mode, if the ABS grants UL bandwidth to the AMS and the AMS has no pending data to be transmitted, the AMS shall transmit padding bytes through the allocated UL burst.”

- Proposed Text Changes

[Add the following text, at line 36 in page 295, section 16.2.4.7, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.4.7 Padding

Within the data burst, the unused portion shall be initialized to a known state. If the size of the unused portion is only one byte then the unused byte is set to 0xF0. If the size of the unused portion is greater than or equal to two bytes, the unused first byte is set to 0xF0 and the remaining unused bytes are set to 0x00.

In active mode, if the ABS grants UL bandwidth to the AMS and the AMS has no pending data to be transmitted, the AMS shall transmit padding bytes through the allocated UL burst.

-------------------------------------------------End proposed text----------------------------------------------------------------