| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on HARQ Channel Mapping to Flow (Section 16.2.3.47) |  |
| Date Submitted | 2010-11-10 |  |
| Source(s) | Hyunkyu Yu, Sangheon Kim, Seho Kim Samsung Electronics Co., Ltd. Jeongki Kim, Youngsoo Yuk LG Electronics Roshni Srinivasan, Hujun Yin Intel Corporation Lei Wang InterDigital Communications, LLC Yuqin Chen ZTE Corporation | hk.yu@samsung.com jeongki.kim@lge.com, youngsoo.yuk@lge.com roshni.m.srinivasan@intel.com leiw@billeigean.com |
| Re: | Comments on IEEE P802.16m/D9 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution proposes the text about HARQ channel mapping to flow. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on HARQ Channel Mapping to Flow (Section 16.2.3.47)

Hyunkyu Yu, Sangheon Kim, Seho Kim 
Samsung Electronics

Jeongki Kim, Youngsoo Yuk
LG Electronics

Roshni Srinivasan, Hujun Yin
Intel Corporation

Lei Wang

InterDigital Communications, LLC

Yuqin Chen

ZTE Corporation

- Introduction

- HARQ channel mapping:

In uplink transmission, bandwidth allocation for the delay-sensitive traffic (e.g. VoIP) needs to be explicitly mapped to a flow. Otherwise, other service flows may use those resources, degrading the performance of that flow (VoIP). To guarantee the performance of delay-sensitive traffic, we suggest mapping HARQ channels to a flow in DSx message. Note that persistent allocation cannot always be used for VoIP transmission, thus we recommend adopting HARQ channel mapping concept.

- Proposed Text Changes

[Remedy-1: Change the following text, at line 60 in page 367, section 16.2.7, as]

Persistent allocation is a technique used to reduce assignment overhead for connections with periodic traffic pattern and with relatively fixed payload size. To allocate resources persistently to an AMS a single connection, the ABS shall transmit the DL Persistent Allocation A-MAP IE for DL allocations and the UL Persistent Allocation A-MAP IE for UL allocations.

[Remedy-2: Add the following text, at line 40 in page 426, section 16.2.12.12, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.12.12 Service Flow/Convergence Sublayer Parameters

…

Table 781—Service flow/convergence sublayer parameters

| Fields | Size (bits) | Description |
| --- | --- | --- |
| … | … | … |
| Non-Common for Coupled Group | variable | … |
| Number of HARQ Channels | 4 | Indicates the number of HARQ channels which are intended to the flow. |
| fullNumHarqChArr[0…15] |  |  |
| A) HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel. |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-3: Add the following text, at line 6 in page 429, after section 16.2.12.12.6, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.12.12.7 HARQ channel mapping

This parameter specifies the set of HARQ channels that is intended to be used for carrying UL data on this service flow. When this parameter is present in AAI-DSA-REQ or AAI-DSC-REQ, the service flow should be given higher priority for carrying data via the specified set of HARQ channels. A unique set of HARQ channels shall be mapped to each flow for which the HARQ channel mapping is specified.

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-4: Add the following text, at line 44 in page 234, section 16.2.3.47.1, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.3.47.1 AAI-DSA-REQ

…

Table 734 – AAI-DSA-REQ

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| }// ENDIF (Sleep Cycle Setting is included) |  |  |  |
| For (i=0; Number of HARQ channels; i++){ |  | Number of HARQ channels = 1..16 | Present if needed |
| HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel. |  |
| } |  |  |  |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-5: Add the following text, at line 44 in page 251, section 16.2.3.47.4, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.2.3.47.4. AAI_DSC-REQ

…

Table 740 – AAI-DSC-REQ

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| }// ENDIF (Sleep Cycle Setting is included) |  |  |  |
| For (i=0; Number of HARQ channels; i++){ |  | Number of HARQ channels = 1..16 | Present if needed |
| HARQ Channel Mapping | 4 | Indicates the index (ACID) of each HARQ channel. |  |
| } |  |  |  |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-6: Add the following text, at line 26 in page 1015, Annex R, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

…….

multicastGroup 	SEQUENCE (SIZE(1..16)) OF SEQUENCE {

multicastGroupId 	INTEGER (0..4095),

fid 			INTEGER (0..15)} OPTIONAL,

sleepCycleSetting 	SleepCycleSetting OPTIONAL,

harqChannelsList 	SEQUENCE (SIZE (1..16)) OF INTEGER (0..15) OPTIONAL,

...

}

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy-7: Add the following text, at line 36 in page 1018, Annex R, as]

------------------------------------------------Start proposed text----------------------------------------------------------------

…….

sleepCycleSetting 		SleepCycleSetting OPTIONAL,

initialBackoffWindowSize 	INTEGER (0..15) OPTIONAL,

maxBackoffWindowSize 	INTEGER (0..15) OPTIONAL,

backoffScalingFactor 		INTEGER (0..15) OPTIONAL,

harqChannelsList 	SEQUENCE (SIZE (1..16)) OF INTEGER (0..15) OPTIONAL,

...

}

-------------------------------------------------End proposed text----------------------------------------------------------------