| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modification to DL basic assignment A-MAP IE in IEEE802.16m/D9 |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Mohammad A. (Amir) Khojastepour, Narayan Prasad, Sampath Rangarajan, Nader Zein, Linghang Fan NEC Corporation Roshni Srinivasan, Yang-seok Choi Intel Corporation | amir@nec-labs.com nader.zein@eu.nec.com roshni.m.srinivasan@intel.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D9 |  |
| Abstract | This contribution proposes change to DL basic Assignment A-MAP in DL MU-MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modification to the DL basic assignment A-MAP IE in IEEE802.16m/D9

Mohammad A. (Amir) Khojastepour, Narayan Prasad, Sampath Rangarajan, Nader Zein, Linghang Fan

NEC Corporation

Roshni Srinivasan, Yang-seok Choi

Intel Corporation

- Introduction

Both competing IMT-advanced technologies, LTE-A and IEEE 802.16m support 8 streams in the DL for SU-MIMO. In the DL MU-MIMO on the other hand the total number of streams is limited to at most four in LTE-A; thus, it is only possible to have at most 4 co-scheduled users with one stream each. This limitation was removed for IEEE 802.16m and the option for 8 stream transmission is now available in MU-MIMO mode as well as SU-MIMO mode. However, simulation results by several companies in LTE-A have shown that it is difficult to find more than 4 users with enough data to co-schedule on the same sub-band. Pairing 2 users is seen to be the most common case. For an ABS with polarized TX antennas, the possibility of scheduling an MU-MIMO user with 2 streams is important. Thus, both LTE-A and IEEE802.16m allow for assigning up to 2 streams to an MU-MIMO user.

In the current IEEE802.16m draft, support for 8 stream MU-MIMO transmission targets only an ABS with 8 TX antennas since the much more typical case of an ABS with 4 TX will not co-schedule more than 4 streams in the DL. Since it is suspected that the deployment of 4 TX ABS would be the primary target for most companies, this enhancement of 8 streams MU-MIMO may not apply is most cases. On the other hand, with more flexible decoding of the DL basic assignment MAP IE, more support for MLD can be exploited by ABSs with both 4 TX and 8 TX antennas.

This contribution proposes to interpret the Si field in the DL A-MAP for MU-MIMO differently for 4 TX and 8 TX cases. This will retain the current table for 8 stream support for an ABS with 8 TX antennas and provide more entries to support MLD for 3 and 4 streams for a 4TX ABS. The number of the transmit antennas at the base station (Nt) is available in S-SFH SP3.

This contribution proposes to interpret the Si field in the DL A-MAP for MU-MIMO differently for 4 TX ABS in the case it is determined by the ABS that a scheduled AMS can support (quasi-)MLD for 3 or 4 streams. In such a case modulation constellation of the paired user for up-to 4 stream transmission can be provided. In all other cases the current table in D8 is used and modulation constellation of paired user in the case of 2 stream transmission is provided. The number of the transmit antennas at the base station (Nt) is available in S-SFH SP3.

Text Proposal

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

<Modify the section 16.3.5.5.2.4.1 on page 647 Table 844 as shown >

_______________________________Start of proposed text change #1 _____________________________

# 16.3.5.5.2.4.1 DL basic assignment A-MAP IE

Table 848 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

A-MAP IE Type: Defines the structure of the assignment A-MAP IE for the bits in the assignment A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between assignment A-MAP IEs used for the UL/DL, and assignment A-MAP IEs used for resource allocation and control signaling. Additional A-MAP IE types are reserved for future use.

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

Si: Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of the paired user. For Co-MIMO transmission to two users, the ABS shall indicate to one AMS a total number of streams equal to 3 using one of the values 0b01000, 0b01001 or 0b01010, in order to ensure that the AMS will perform channel estimation using one stream of the non-interlaced pilot pattern with 3 streams.

| Table 848 - DL basic assignment A-MAP IE* |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Description/Notes |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Multi-layer encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt ≤ Nt Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Reserved | 2 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for multi-layer encoding |
| if (Nt == 4 and quasi-MLD capability at AMS for 3 or 4 streams is supported in AAI-SBC-RSP){ |  | Parameters for multi-layer encoding for ABS with modulation constellation of paired user for up-to 4 stream transmission. |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user 0b00000: 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: 2 streams with PSI=stream0 and other modulation information not available 0b00100: 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: 2 streams with PSI=stream1 and other modulation information not available 0b01000: 3 streams with PSI=stream0 0b01001: 3 streams with PSI=stream1 0b01010: 3 streams with PSI=stream2 0b01011: 4 streams with PSI=stream0 0b01100: 4 streams with PSI=stream1 0b01101: 4 streams with PSI=stream2 0b01110: 4 streams with PSI=stream3 0b01111: 3 streams with PSI=stream0 and stream1 0b10000: 4 streams with PSI=stream0 and stream1 0b10001: 4 streams with PSI=stream2 and stream3 0b10010: 3 streams with PSI=stream2 and other modulation =QPSK 0b10011: 3 streams with PSI=stream2 and other modulation =16QAM 0b10100: 3 streams with PSI=stream2 and other modulation =64QAM 0b10101: 3 streams with PSI=stream0 and stream1 and other modulation =QPSK 0b10110: 3 streams with PSI=stream0 and stream1 and other modulation =16QAM 0b10111: 3 streams with PSI=stream0 and stream1 and other modulation =64QAM 0b11000: 4 streams with PSI=stream0 and stream1 and other modulation =QPSK 0b11001: 4 streams with PSI=stream0 and stream1 and other modulation =16QAM 0b11010: 4 streams with PSI=stream0 and stream1 and other modulation =64QAM 0b11011: 4 streams with PSI=stream2 and stream3 and other modulation =QPSK 0b11100: 4 streams with PSI=stream2 and stream3 and other modulation =16QAM 0b11101: 4 streams with PSI=stream2 and stream3 and other modulation =64QAM 0b11110: reserved 0b11111: reserved |
| } else { |  | Parameters for multi-layer encoding for ABS |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b00000: 2 streams with PSI=stream0 and other modulation =QPSK 0b00001: 2 streams with PSI=stream0 and other modulation =16QAM 0b00010: 2 streams with PSI=stream0 and other modulation =64QAM 0b00011: 2 streams with PSI=stream0 and other modulation information not available 0b00100: 2 streams with PSI=stream1 and other modulation =QPSK 0b00101: 2 streams with PSI=stream1 and other modulation =16QAM 0b00110: 2 streams with PSI=stream1 and other modulation =64QAM 0b00111: 2 streams with PSI=stream1 and other modulation information not available 0b01000: 3 streams with PSI=stream0 0b01001: 3 streams with PSI=stream1 0b01010: 3 streams with PSI=stream2 0b01011: 4 streams with PSI=stream0 0b01100: 4 streams with PSI=stream1 0b01101: 4 streams with PSI=stream2 0b01110: 4 streams with PSI=stream3 0b01111: 3 streams with PSI=stream0 and stream1 0b10000: 4 streams with PSI=stream0 and stream1 0b10001: 4 streams with PSI=stream2 and stream3 0b10010: 8 streams with PSI=stream0 0b10011: 8 streams with PSI=stream1 0b10100: 8 streams with PSI=stream2 0b10101: 8 streams with PSI=stream3 0b10110: 8 streams with PSI=stream4 0b10111: 8 streams with PSI=stream5 0b11000: 8 streams with PSI=stream6 0b11001: 8 streams with PSI=stream7 0b11010: 8 streams with PSI=stream0 and stream1 0b11011: 8 streams with PSI=stream2 and stream3 0b11100: 8 streams with PSI=stream4 and stream5 0b11101: 8 streams with PSI=stream6 and stream7 0b11110: reserved 0b11111: reserved |
| } |  |  |
| } else { |  |  |
| Reserved | 5 | Reserved bits |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 1 | Reserved bit |
| } |  |  |

_______________________________End of proposed text change #1 ______________________________

<Modify the section 16.2.5.3.5 on page 106 Table 682 as shown >

- _______________________________Start of proposed text change #2 _____________________________

16.2.3.5 AAI-SBC-REQ

An AAI-SBC-REQ message, to which HARQ operation is applied, is transmitted by AMS to negotiate basic capability during network entry.

- Table 682—AAI-SBC-REQ message Field Descriptions

| - Field | - Size (bits) | - Value/Description | - Conditions |
| --- | --- | --- | --- |
| Support for Quasi-MLD capability at AMS for 3 or 4 streams with DL MIMO | 1 | If Bit #0 = 1, Quasi MLD capability at AMS for 3 or 4 streams with DL MIMO is supported |  |

_______________________________End of proposed text change #2 ______________________________

- <Modify the section 16.2.3.6 on page 109 Table 683 as shown >

- _______________________________Start of proposed text change #3 _____________________________

16.2.3.6 AAI-SBC-RSP

- An AAI-SBC-RSP message, to which HARQ operation is applied, is transmitted by ABS in response to a received AAI-SBC-REQ during initialization.

- Table 683—AAI-SBC-RSP message Field Descriptions

| - Field | - Size (bits) | - Value/Description | - Conditions |
| --- | --- | --- | --- |
| Support for Quasi-MLD capability at AMS for 3 or 4 streams with DL MIMO | 1 | If Bit #0 = 1, Quasi MLD capability at AMS for 3 or 4 streams with DL MIMO is supported | Optional |

_______________________________End of proposed text change #3 ______________________________

References:

1. Higuchi, K.  Kawai, H.  Maeda, N.  Taoka, H.  Sawahashi, M. "Experiments on real-time 1-Gb/s packet transmission using MLD-based signal detection in MIMO-OFDM broadband radio access" Page(s): 1141 - 1153, IEEE JSAC June 2006

2. P. Aggarwal, N. Prasad, X. Wang  and M. Madihian, "A Low Complexity MMSE-based Sequential Monte Carlo Algorithm for Near-Optimal MIMO Demodulation", IEEE Transaction on Signal Processing  2007:55(6) pp. 2395-2406