| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Enhancements to DL Closed-loop MU-MIMO in IEEE802.16m/D9 |  |
| Date Submitted | 2010-11-07 |  |
| Source(s) | Mohammad A. (Amir) Khojastepour, Narayan Prasad, Guosen Yue, Sampath Rangarajan, Nader Zein, Linghang Fan NEC Corporation | amir@nec-labs.com nader.zein@eu.nec.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D9 |  |
| Abstract | This contribution proposes enhancements to DL Closed-loop MU-MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Enhancements to DL Closed-loop MU-MIMO in IEEE802.16m/D9

Mohammad A. (Amir) Khojastepour, Narayan Prasad, Guosen Yue, Sampath Rangarajan, Nader Zein, Linghang Fan

NEC

- Introduction

For downlink multi-user MIMO (MU-MIMO) two transmission modes MIMO mode 3 (open loop MU-MIMO with non-adaptive precoding) and MIMO mode 4 (closed loop MU-MIMO with adaptive precoding) have been defined in 802.16m.

Out of these two transmission modes, only MIMO mode 4 (CL MU-MIMO) allows two-stream simultaneous transmission to a scheduled AMS. In particular, there is one stream assignment with 3 total streams, there are three stream assignments with 4 total streams and eight stream assignments with 8 total streams (the latter being applicable to only an ABS with eight transmit antennas), that allow two-stream simultaneous transmission to one or more co-scheduled AMS. For an ABS with polarized TX antennas, which has been identified as an important configuration for practical deployment, the possibility of scheduling an MU-MIMO AMS with 2 streams is particularly beneficial.

On the other hand, there is no support for such two-stream (or two-layer) per-AMS transmission in the feedback obtained from each potential MU-MIMO AMS. For supporting MU-MIMO, three MIMO feedback modes (MFM 5, 6 and 7) have been defined in 802.16m. Out of these, MFM 6 targets CL MU-MIMO with SLRU or NLRU allocation. We note that all of these three MFMs allow the feedback of (sub-band or wideband) PMI only from a rank-1 codebook. Moreover, the CQI is also computed assuming that the AMS will be served only one stream using the recommended PMI.

Clearly, there is a need to enhance feedback support for CL MU-MIMO with two-layer transmission to one or more scheduled users.

Towards this end, we make the following suggestions:

- Support for rank-2 (STC rate-2) CL MU-MIMO PMI feedback. The rank-2 base codebook and base codebook subsets for CL MU-MIMO can be defined to be identical to their SU-MIMO counterparts just as done for the rank-1 case.

- Allowing the ABS to suggest either rank-1 (STC-rate 1) or rank-2 (STC-rate 2) to a potential MU-MIMO user, by appropriately modifying the feedback allocation A-MAP IE.

In addition, there is a need to clarify MU-MIMO CQI computation at the AMS. In 802.16m/D9 for MU-MIMO feedback modes with codebook-based feedback (MFM 6 and MFM 7), a CQI is calculated at the AMS assuming that there are (MaxMt -1) interfering streams and that the interfering streams are scheduled by the ABS using rank-1 precoders orthogonal to each other and orthogonal to the rank-1 precoder represented by the reported PMI. If MaxMt = 1, the AMS feeds back rank-1 CL SU-MIMO CQI.

We note that while the use of MaxMt in MU-MIMO CQI computation allows for a better prediction of post-scheduling interference at the AMS, a value of the power split (or fraction) needs to be assumed by the AMS to account for such interference in its CQI computation. In other words, the AMS must assume a value for the fraction of the transmit power that will be used for its desired stream(s) (with the remaining fraction being used for the interfering streams). We emphasize that such a fraction is only employed by the AMS for CQI computation while the actual transmission by the ABS will employ equal-power-per-stream. The main purpose of CQI computation using such a fraction is to ensure that the AMS correctly balances the signal strength received in the signaling subspace (determined by its reported precoder) and the interference received from the orthogonal complement to the signaling subspace. For instance, with MaxMt=8 and equal power-per-stream assumption for CQI computation, the AMS will compute a CQI assuming that 1/8 of the total power is assigned to its desired stream whereas 7/8 of the total power is shared among interfering streams. This will make the CQI computation overtly conservative and lead to a loss in the achieved spectrum efficiency.

To address this issue, we make the following suggestion:

- For MU-MIMO feedback modes with codebook-based feedback, define a parameter  that specifies the power-split between the desired stream(s) and the interfering streams that the AMS must assume. In particular, while computing its CQI the AMS should assume that a fraction  of the transmit power will be used by the ABS for transmitting its desired stream(s), whereas the remaining fraction 1- will be shared equally among the interfering streams intended for the other co-scheduled users. A default value of  can denote equal power split among all the co-scheduled streams.

Simulation Results

| Parameter | Assumptions used for evaluation |
| --- | --- |
| Duplex method and bandwidth | FDD: 10MHz for downlink |
| Cell layout | Hexagonal grid 19 sites, 3 cells per site |
| Number of AMSs per sector | 10 |
| Network synchronization | Synchronized |
| Antenna configuration (ABS) | 4 TX co-polarized antennas with 0.5-lambda spacing |
| Antenna configuration (AMS) | 2 RX co-polarized antennas with 0.5-lambda spacing |
| Downlink transmission scheme | MU-MIMO: Maximum 2 co-scheduled AMSs per RB. Each AMS can have rank-1 or rank-2 |
| Codebook | 4 bit codebook |
| Downlink scheduler | Proportional fair in time and frequency Scheduling granularity: 5 PRBs |
| Feedback assumptions | Report is with 5ms periodicity and 4ms delay. Sub-band 4 bit CQI and 4 bit PMI without feedback errors. Sub-band granularity: 5 PRBs |
| Downlink HARQ scheme | Chase Combining |
| Downlink receiver type | LMMSE. |
| Channel estimation error | NA |
| Feedback channel error | NA |

Table 1: Simulation assumptions.

The following table lists our simulation results.

|  | SU-MIMO with SU-report (CQI + dynamic rank-1 or rank-2 PMI) | MU-MIMO with SU-report (CQI+ dynamic rank-1 or rank-2 PMI) | MU-MIMO with MU-report (CQI+rank-1 PMI) : For highgeometry users CQI is computed assuming equal power-per-stream and MaxMt=4; for low geometry users CQI is computed as in rank-1 CL SU-MIMO | MU-MIMO with MU-report (CQI+rank-1 PMI) : For highgeometry users CQI is computed assuming ½ power for desired stream and ½ power shared equally among MaxMt-1 (=3) streams; for low geometry users CQI is computed as in rank-1 CL SU-MIMO |
| --- | --- | --- | --- | --- |
| Average cell spectral efficiency | 2.0426 | 2.3829 | 2.4993 | 2.5972 |
| Cell-edge spectral efficiency | 0.0688 | 0.0822 | 0.0889 | 0.0885 |

Table 2: Simulation results

The modified feedback allocation IE given below supports up-to rank-2 CL MU-MIMO PMI feedback by allowing the ABS to suggest either rank-1 (STC-rate 1) or rank-2 (STC-rate 2) to a user, using the MU-rank field. In addition, it includes a parameter  that specifies the power-split between the desired stream(s) and the interfering streams. In particular, while computing its CQI the AMS should assume that a fraction  of the transmit power will be used by the ABS for transmitting its (1 or 2) desired stream(s), whereas the remaining fraction 1- will be used to transmit the (MaxMt-1 or MaxMt-2) interfering streams intended for the other co-scheduled users. If MaxMt = MU-rank, the AMS feeds back MU-rank CL SU-MIMO CQI. (In general MaxMt>=MU-rank)

Text Proposal

| Table 849: Feedback Allocation A-MAP IE |  |  |
| --- | --- | --- |
| Syntax | Size (bit) | Description/Notes |
| if (MFM==6) or ( MFM== 7){ | - | - |
| MaxMt | 2 | If Nt=4 (any MFM) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 If Nt=8 (MFM=6) 0b00: 1 0b01: 2 0b10: 4 0b11: 8 If Nt=8 (MFM=7) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| } |  |  |
| if (MFM==7){ |  |  |
| α | 1 | power split fraction 0b0: Equal split among MaxMt streams 0b1: ½ power for the desired stream and ½ power for the MaxMt-1 interfering streams |
| } |  |  |
| if (MFM==6){ | - | - |
| MU-rank | 1 | Suggested rank 0b0: STC rate-1 0b1: STC rate-2 |
| If (MU-rank<MaxMt){ |  |  |
| α | 1 | power split fraction 0b0: Equal split among MaxMt streams 0b1: ½ power for the desired MU-rank stream(s) and ½ power for the interfering MaxMt – MU-rank stream(s) |
| } |  |  |
| } |  |  |