| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Inefficiency related to DID management and randomization of Paging Offset |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Vladimir Yanover, Oleg Marinchenco, Alvarion | vladimir.yanover@alvarion.com oleg.marinchenco@alvarion.com |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D9) |  |
| Abstract | Inefficiency related to DID management and randomization of Paging Offset |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Inefficiency related to DID management and randomization of Paging Offset

Vladimir Yanover, Oleg Marinchenco, Alvarion

Problem Statement

Current DID management solution imposes significant overhead caused by randomization of Paging Offset.

Section 16.2.18:

“The assignment of paging offset shall be performed in such a way that the paging offset of different idle mode AMSs is pseudo-randomized”.

This Paging Offset randomization results in the following overheads:

- An ABS cannot be aware of the possible Paging Intervals in the Paging Group a priori, hence, it MUST include PGID_info message in all  the Superframes.

This presents significant overhead vs. pre-engineered Paging Cycle/ Offset.

Section 16.2.18.2.3:

“The ABS shall transmit the PGID-Info at a predetermined location in the paging listening interval in order to advertise the paging group(s) that is supported by the ABS. The PGID-Info shall be transmitted by the ABS regardless of whether or not there any notifications for AMSs.

The ABS transmits the PGID-Info right after SFH and A-MAP of the 1st AAI subframe during AMS's paging listening interval as shown in Figure 464.”

- Randomization of Paging Offset makes it complicate (if possible at all) to group PAG-ADV for multiple paging events in an efficient way. So, there is an additional overhead of the broadcast message itself (multi-bytes message).

In reasonable scenarios these two sources provide for essentially more overhead than addition of 8 bits (24 bit DID vs. 12 bit DID + 4 PC) to AAI-PAG-ADV suggested in this contribution.

Suggested Solution

Redefine DID management scheme as following:

- Increase DID length to 24 bits;

- Use 24-bit AMSID* Hash as a DID value for Advanced ASN when AMSID privacy is enabled (similar to MS’ MAC address Hash in Legacy ASN);

- Define Paging Offset as an engineering parameter (no need for pseudo-randomization) and remove it from AAI-PAG-ADV message.

- Remove Paging Cycle/ Offset in AAI-RNG-REQ IM re-entry messages

Proposed changes:

Change #1:

16.2.1.2.3 Deregistration Identifier (DID)

A 24-bit hash value of the MAC address is used to identify Idle Mode AMSs in legacy network mode or when AMSID privacy is disabled. A 24-bit DID is used to identify Idle Mode AMSs in advanced network mode when AMSID privacy is enabled.

The 24-bit DID value is set to the hash of the 48-bit AMSID* used by the AMS in the initial network entry.

The hash value shall be the remainder of the division (Modulo 2) of the 48-bit AMSID* multiplied by the polynomial D24 with the generator polynomial g(D) = D24+ D23+ D18+ D17+ D14+ D11+ D10+ D7+ D6+ D5+ D4 + D3 + D + 1 (hex = 1864cfb)

The legacy network uses a 48bit MAC address for Idle Mode AMSs. The legacy network shall use a 24-bit hash value of the MAC address in AAI-PAG-ADV message when paging the AMS. An AMS in Idle Mode shall decode the MAC address hash value, to determine whether or not it is being paged.

Change #2:

16.2.3.1 AAI-RNG-REQ

…

Table 678—AAI-RNG-REQ message Field Description

| … |  |  |  |
| --- | --- | --- | --- |
| } else if (Ranging Purpose Indication == 0b0010) { |  | // Network reentry from idle mode |  |
| if (legacy network mode or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address | Presented in legacy network mode only or in case that AMSID privacy is disabled |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 24 | The ID which is used for idle mode AMS in advanced network mode when AMSID privacy is enabled. | Presented in advanced network mode only when AMSID privacy is enabled. |
| } |  |  |  |
| Paging Controller ID | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. |  |
| PGID | 16 | The identification of the paging group that the AMS is previously belonging to. |  |
| … |  |  |  |
| } else if (Ranging Purpose Indication == 0b0011 ¦0b0110¦0b0111¦ 0b1011) { |  | // Idle mode location update (and with other additional purposes) |  |
| if (legacy network mode or AMSID privacy is disabled){ |  |  |  |
| AMS MAC address | 48 | AMS's real MAC address | Presented in legacy network mode only or in case that AMSID privacy is disabled |
| } else{ |  |  |  |
| Deregistration Identifier (DID) | 12 | The ID which is used for idle mode AMS in advanced network mode when AMSID privacy is enabled.. | Presented in advanced network mode only when AMSID privacy is enabled. |
| } |  |  |  |
| Paging Controller ID | 48 | The Paging Controller ID which the AMS currently maintains in idle mode. |  |
| PGID | 16 | The identification of the paging group that the AMS is previously belonging to. |  |

Change #3:

16.2.3.2 AAI-RNG-RSP

…

Table 679—AAI-RNG-RSP message Field Description

| New Paging Offset | 9 | New paging offset of AMS which determines the Superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than the Paging Cycle value. | Shall be included when the Paging parameters are changed. |
| --- | --- | --- | --- |

Change #4:

Annex R.2 MAC Control Message Definitions

…

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Network entry / re-entry messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- single type definition for ranging messages

MACAddress 	::= BIT STRING (SIZE(48))

MACVersion 	::= INTEGER (0..255)

CRID 		::= BIT STRING (SIZE(72))

DID 		::= BIT STRING (SIZE(24))

…

AddressOrDID ::= CHOICE {

-- be selected for legacy network mode

macAddress 			MACAddress,

-- be selected for non legacy network mode when AMSID privacy is enabled

deregistrationID 		DID

}

PagingControlInfo ::= SEQUENCE {

pagingControllerID 	PCID,

pagingGroupID 		PGID,

}

…

SuccessOfLocationUpdate ::= SEQUENCE {

newPagingCycle 		PgCycle 	OPTIONAL,

newPagingGroupID 	PGID 		OPTIONAL,

newPagingOffset 		PgOffset 	OPTIONAL,

newPagingControllerID 	PCID 		OPTIONAL,

embsZoneInfo 		SEQUENCE(SIZE(0..7)) OF

EMBSZoneInfoItem,

Change #5:

16.2.3.23 AAI-PAG-ADV (paging advertisement) Message

…

Table 700—AAI-PAG-ADV Message Field Description

| For (j=0; j<Num_AMSs; j++) { |  |  |  |
| --- | --- | --- | --- |
| Deregistration Identifier | 24 | Used to indicate the MS to be paged 0..2^24-1 | Used for the AMS in advanced network when AMSID privacy is enabled. |
| MAC Address Hash | 24 | used to identify the MS to be paged | Present if the Network Configuration indicates ABS is attached to the legacy network |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| } |  |  |  |

Change #6:

16.2.3.22 AAI-DREG-RSP message

Table 699—AAI-DREG-RSP message format

| … |  |  |  |
| --- | --- | --- | --- |
| If (Action code == 0x05) { |  |  |  |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Paging offset | 9 | Used to indicate Paging offset for the AMS. Determines the superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than Paging cycle value. |  |
| Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS's idle mode information 0..2^48-1 |  |
| Paging group ID | 16 | Used to indicate Paging group which the AMS is located in 0..2^16-1 |  |
| … |  |  |  |
| If (Action code == 0x07) { |  |  |  |
| Paging cycle | 4 | Used to indicate Paging cycle for the AMS 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 0x08-0x15: reserved |  |
| Paging offset | 9 | Used to indicate Paging offset for the AMS. Determines the superframe within the paging cycle from which the paging listening interval starts. Shall be smaller than Paging cycle value. |  |
| Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the AMS's idle mode information 0..2^48-1 |  |
| Paging group ID | 16 | Used to indicate Paging group which the AMS is located in 0..2^16-1 |  |
| … |  |  |  |

Change #7:

16.2.18.4.1 Location update trigger conditions

…

During location update, the network may update, paging cycle and paging offset for the AMS.