| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Text Additions for Visited NSP Realm in AAI_SBC-REQ/RSP messages (16.2.3.5) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Danny Moses, Joey Chou Intel Youngkyo Baek Samsung | Voice : +1 503-264-7867 E-mail: shaocheng.wang@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-3 for P802.16m/D9. |  |
| Abstract | The contribution proposes Text Additions for Visited NSP Realm in AAI_SBC-REQ/RSP messages (16.2.3.5) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Text Additions for NSP lists in AAI_SBC-REQ/RSP messages (16.2.3.5)

Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Danny Moses, Joey Chou

Intel

- Introduction

In the 802.16m/D9, NSP lists were added in AAI_SBC-REQ/RSP messages to align with NSP acquisition procedures in 802.16e. It is noted that, “Visited NSP Realm” shall also be included when Visited NSP ID is found in the AAI-SBC-REQ.

This contribution proposes text additions to add “Visited NSP ID” & “Visited NSP Realm” in AAI_SBC-REQ/RSP messages.

- Suggested changes in the 802.16m/D9

The following is the proposed change in the 802.16m/D9. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1:

Modify Table 682 in page 109, line 40 with the following additional rows

------------------------------------------------- Start of proposed text I--------------------------------------------------

Table 682—AAI-SBC-REQ message Field Descriptions

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| Relay mode | 1 | 0b0: TTR relay 0b1: STR relay |  |
| } |  |  |  |
| Visited NSP ID | 24 | NSP ID of the Network Service Provider the MS intends to be the conduit for authentication to the MS home network. | Present as needed |
| } |  |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

Suggested change #2:

Modify Table 683 in page 112, line 44 with the following additional rows

------------------------------------------------- Start of proposed text II--------------------------------------------------

Table 683—AAI-SBC-RSP message Field Descriptions

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| CAPABILITY_INDEX | 5 | It refers to the "Capability Class" that the AMS can sup-port. Value: 0~31 |  |
| … | … | … |  |
| MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. | Present as needed |
| Visited NSP Realm | Variable (1 ... 128 x 8) | Visited NSP Realm | Present if the Visited NSP ID is found in the AAI-SBC-REQ |
| } |  |  |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

Suggested change #3:

Modify page 978, line 23 with the following additional text in blue

------------------------------------------------- Start of proposed text III--------------------------------------------------

VerboseName ::= IA5String (SIZE(1..128))

VisitedNSPRealm ::= IA5String (SIZE(1..128))

------------------------------------------------- End of proposed text III --------------------------------------------------

Suggested change #4:

Modify page 1000, line 59 with the following additional text in blue

------------------------------------------------- Start of proposed text IV--------------------------------------------------

mihCapabilitySupported FeatureSupport OPTIONAL,

visitedNspRealmName 	VisitedNSPRealm OPTIONAL

}

},

...

}

------------------------------------------------- End of proposed text IV--------------------------------------------------

VerboseName ::= IA5String (SIZE(1..128))

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”