| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Cleanup Text for Visited NSP Realm in AAI_SBC-REQ/RSP messages (16.2.3.5) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Danny Moses, Joey Chou Intel | Voice : +1 503-264-7867 E-mail: shaocheng.wang@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-4 for P802.16m/D10. |  |
| Abstract | The contribution proposes Text Additions for Visited NSP Realm in AAI_SBC-REQ/RSP messages (16.2.3.5) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Cleanup Text for NSP lists in AAI_SBC-REQ/RSP messages (16.2.3.5)

Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Danny Moses, Joey Chou

Intel

- Introduction

In the 802.16m #70 meeting (Dallas), “Visited NSP ID” & “Visited NSP Realm” were agreed to be added in AAI_SBC-REQ/RSP messages, but were NOT implemented correctly as the original contribution suggested. This contribution proposes some clean-up text for “Visited NSP ID” & “Visited NSP Realm” in AAI_SBC-REQ/RSP messages.

- Suggested changes in the 802.16m/D10

The following is the proposed change in the 802.16m/D10. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1:

Modify Table 682 in page 145, line 33 as the following:

[Note to editor: black text is the existing text; blue text is the new text to insert; red text is the text to delete]

------------------------------------------------- Start of proposed text I--------------------------------------------------

Table 682—AAI-SBC-REQ message Field Descriptions

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … |  |  |  |
| If (ARS is a sender of AAISBC- REQ) { |  |  | //only available during ARS network entry phase |
| Relay mode | 1 | 0b0: TTR relay 0b1: STR relay |  |
| Visited NSP ID | 24 | NSP ID of the Network Service Provider the MS intends to be the conduit for authentication to the MS home network. | Present as needed |
| if (Relay mode == 0b0){ |  |  |  |
| ARSTTG |  | ARSTTG value (s). It shall be less than 50s. |  |
| ARSRTG |  | ARSRTG value (s). It shall be less than 50s. |  |
| } |  |  |  |
| } |  |  |  |
| Visited NSP ID | 24 | NSP ID of the Network Service Provider the MS intends to be the conduit for authentication to the MS home network. | Present as needed |
| } |  |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

Suggested change #2:

------------------------------------------------- Start of proposed text II--------------------------------------------------

------------------------------------------------- End of proposed text II --------------------------------------------------

Suggested change #4:

Modify page 1045, line 37 with the following additional text in blue

aRSnetworkentry SEQUENCE {

aRSTTG INTEGER (0..49),

aRSRTG INTEGER (0..49),

relaymode BOOLEAN

} OPTIONAL,

visitedNSPID	NSPID 	OPTIONAL

},

visitedNSPID	NSPID 	OPTIONAL

},

...

}

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D10, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”