| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on AAI-UL-POWER-ADJ message (16.2.3.33) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Wookbong Lee, Jinsoo Choi, Jin-Sam Kwak LG Electronics | wookbong.lee@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Text proposal on AAI-UL-POWER-ADJ |  |
| Purpose | To be discussed and adopted in D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on AAI-UL-POWER-ADJ message (16.2.3.33)

Wookbong Lee, Jinsoo Choi, Jin-Sam Kwak

LG Electronics

Introduction

Need to update AAI-UL-POWER-ADJ message.

Proposed Text

Remedy:

In the P802.16m/D9, page 180, line 3, replace underscores by hyphens in the message name and replace Table 713 with the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.33 AAI-UL-POWER-ADJ message

An AAI-UL-POWER-ADJ message shall be transmitted by the ABS to control the transmit power level of AMS.

Table 713—AAI-UL-POWER-ADJ message Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| offsetData | 6 | offsetData is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | N/A |
| offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among -15.5 to 16 dB with 0.5 dB step | N/A |
| channelIndex | 8 | This value represents “Channel Index” in Feedback Allocation A-MAP IE. Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between channel index and the AMS | Present when ABS wants to change Fast Feedback Control FF Channel Period by using AAI-UL-POWER-ADJ message. (More details can be found in 16.3.5.5.2.4.5.) |
| shortTermFeedbackPeriod | 3 | This value represents “Short-term feedback period (p)” in Feedback Allocation A-MAP IE. Short-term feedback is transmitted on the FBCH every 2p frames, where p is the value specified by shortTermFeedbackPeriod | Present when ABS wants to change Fast Feedback Control Channel Period by using AAI-UL-POWER-ADJ message. (More details can be found in 16.3.5.5.2.4.5.) |
| longTermFeedbackPeriod | 2 | This value represents “Long-term feedback period (q)” in Feedback Allocation A-MAP IE. Long-term feedback is transmitted on the FBCH every 2q short-term feedback reports are transmitted, where q is the value specified by longTermFeedbackPeriod. If q = 0b00, long-term feedback is not used | Present when ABS wants to change Fast Feedback Control Channel Period by using AAI-UL-POWER-ADJ message. (More details can be found in 16.3.5.5.2.4.5.) |
| frameOffsetNumber | 2 | This represents “Frame index” in Feedback Allocation A-MAP IE. The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame number. If the current frame is specified, the AMS starts reporting in eight frames | Present when ABS wants to change Fast Feedback Control Channel Period by using AAI-UL-POWER-ADJ message. (More details can be found in 16.3.5.5.2.4.5.) |
| subframeIndex | 3 | This represents “Subframe index” in Feedback Allocation A-MAP IE. Indicates the UL AAI subframe index in the UL portion of the frame | Present when ABS wants to change Fast Feedback Control Channel Period by using AAI-UL-POWER-ADJ message. (More details can be found in 16.3.5.5.2.4.5.) |
| allocationDuration | 3 | This represents “Allocation Duration (d)” in Feedback Allocation A-MAP IE. An FBCH is transmitted on the FBCH channels indexed by Channel Index for 8×2d frames, where d is the value given by allocationDuration. If d = 0b000, the FBCH is deallocated. If d = 0b111, the AM reports until the ABS command for the AMS to stop | Present when ABS wants to change Fast Feedback Control Channel Period by using AAI-UL-POWER-ADJ message. (More details can be found in 16.3.5.5.2.4.5.) |
| If (Carrier-specific PHY control mode=0b0) { |  |  |  |
| PhysicalCarrierIndex | 6 | The relevant active carrier that this control message is associated with | Present if the Carrier-specific PHY control mode is disabled |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------