| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on AAI-SCD (16.2.3.31) and AAI-ULPC-NI (16.2.3.32) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Wookbong Lee, Jinyoung Chun, Jinsoo Choi, Jin-Sam Kwak LG Electronics Jeongho Park Samsung Electronics | wookbong.lee@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Text proposal on AAI-SCD messages |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on AAI-SCD (16.2.3.31) and AAI-ULPC-NI (16.2.3.32)

Wookbong Lee, Jinyoung Chun, Jinsoo Choi, Jin-Sam Kwak

LG Electronics

Jeongho Park

Samsung Electronics

Introduction

Some of conditions should be updated.

Proposed Text

All changes are marked with yellow highlight

In addition to that, if we want to add some text we used Blue text with underline, while if we want to delete some text we used Red text with strikeout.

Remedy 1:

In the P802.16m/D9, page 185, line 8, replace Table 708 using the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 708—AAI-SCD Message Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Configuration Change Count | 4 | The value is increased whenever the contents of this message are changed The value rolls over from 0 to 15 |  |
| BS_Restart_Count | 4 | The value is incremented by one whenever BS restarts The value rolls over from 0 to 15 |  |
| SA_Preamble Partition for BS type | 20 | Indicates the SA-Preamble partition information. Each 4 bits represent a partition range for each cell type, as defined in 16.3.5.1.2 and Table 819818 |  |
| Trigger definitions | Variable | Refer to Table 768 |  |
| If (OL MIMO Region parameters are needed) { | - |  |  |
| OL-Region-Type0-ON | 1 | 0 or 1 |  |
| OL-Region-Type1-NLRU-Size | 4 | 0 to 15 |  |
| OL-Region-Type1-SLRU-Size | 4 | 0 to 15 |  |
| OL-Region-Type2-SLRU-Size | 4 | 0 to 15 |  |
| } |  |  |  |
| If ( (not a Femtocell) && (not a WirelessMAN-OFDMA Advanced co-existing System with FDM-based UL PUSC zone)) { |  |  |  |
| periodicityOfRngChSync | 2 | The periodicity of the S-RCH allocation (Table 919) |  |
| cntlStartCodeOfRngChSync | 4 | The parameter ks controlling the start root index of the RP codes for the S-RCH. |  |
| rangingPreambleCodeSync | 2 | The number of the RP codes for periodic ranging (Table 920917). |  |
| } |  |  |  |
| periodOfPeriodicRngTimer | 3 | It is the period of Periodic Ranging timer which is broadcasted by the ABS. It has 3 bits to represent the value among {2, 4, 7, 10, 15, 20, 25, 35} second |  |
| If(Frequency Partition 0 is used FPS0 from UFPC > 0){ |  |  |  |
| gammaIotFp0 | 4 | gammaIotFp ( IoT γIOT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) |  |
| } |  |  |  |
| If(Frequency Partition 1 is used FPS1 from UFPC > 0){ |  |  |  |
| gammaIotFp1 | 4 | gammaIotFp (IoT γIOT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) |  |
| } |  |  |  |
| If(Frequency Partition 2 is used FPS2 from UFPC > 0){ |  |  |  |
| gammaIotFp2 | 4 | gammaIotFp (IoT γIOT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) |  |
| } |  |  |  |
| If(Frequency Partition 3 is used FPS3 from UFPC > 0){ |  |  |  |
| gammaIotFp3 | 4 | gammaIotFp (IoT γIOT) is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It is different for each frequency partition (FP0, FP1, FP2, FP3) |  |
| } |  |  |  |
| Alpha | 3 | alpha (α) is the factor according to the number of receive antennas at the ABS. It has 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved} |  |
| Beta | 1 | It beta(β) is used to indicate disable or enable of the power de-boosting for uplink multi-stream transmission. 0: disable 1: enable |  |
| dataSinrMin | 4 | dataSinrMin is the SINR requirement for the minimum data rate expected by ABS. dataSinrMin SINRmin_Data has 4 bits to represent the value in dB among{-INF ,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} |  |
| dataSinrMax | 4 | dataSinrMax is the maximum SINR threshold defined by ABS. dataSinrMax SINRmmax_Data has 4 bits to represent the value in dB among {10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40} |  |
| targetHarqSinr | 4 | It is the HARQ feedback channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4} dB |  |
| targetSyncRangingSinr | 4 | It is the synchronized channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-9, -8.5, -8, -7.5, -7, -6.5, -6, -5.5, -5, -4.5, -4, -3.5, -3, -2.5, -2, -1.5} dB |  |
| targetPfbchSinr | 4 | It is the P-FBCH target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB |  |
| targetSfbchBaseSinr | 4 | It is defined as 4 bits to represent {0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5} dB. |  |
| targetSfbchDeltaSinr | 3 | It is defined as 3 bits to represent {0, 0.20, 0.21, 0.22, 0.23, 0.24, 0.25, 0.26} |  |
| targetBwRequestSinr | 4 | It is the bandwidth request channel target SINR value broadcasted by the ABS. It has 4 bits to represent the value among {-4.5, -4, -3.5, -3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3} dB |  |
| If (Uplink AAI subframes for sounding != 0b000){ |  |  | Uplink AAI subframes for sounding is transmitted in SFH-SP1 |
| gammaIotSounding | 4 | It is 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. |  |
| soundingSinrMin | 4 | It is the minimum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { -4, -3.5, -3, -2.5, -2. -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5} |  |
| soundingSinrMax | 4 | It is the maximum SINR requirement for sounding expected by ABS. It has 4 bits to represent the value in dB among { 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20} |  |
| } |  |  |  |
| T_ReTx_Interval | 3 | 1-8 if DL_N_MAX_ReTx = 4; 1-4 if DL_N_MAX_ReTx = 8 The unit is a frame. |  |
| BR_Channel Configuration MIN Access Class of the (i+0)-th frame | 2 | INTEGER (0..3) | Optional |
| BR_Channel Configuration MIN Access Class of the (i+1)-th frame | 2 | INTEGER (0..3) | Optional |
| BR_Channel Configuration MIN Access Class of the (i+2)-th frame | 2 | INTEGER (0..3) | Optional |
| BR_Channel Configuration MIN Access Class of the (i+3)-th frame | 2 | INTEGER (0..3) | Optional |
| If (Uplink AAI subframes for sounding != 0b000) { |  |  | Uplink AAI subframes for sounding is transmitted in SFH-SP1 |
| multiplexingType | 1 | 0b0: decimation separation 0b1: cyclic shift separation | Present when Uplink AAI subframes for sounding in S-SFH SP1 is not set to 0b000 |
| If (multiplexingType == 0b0 is decimation separation) { |  |  |  |
| decimationValueD | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) |  |
| }else if (multiplexingType == 0b1 is cyclic shift separation) { |  |  |  |
| maxCyclicShiftIndexP | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) |  |
| } |  |  |  |
| shiftValueUForSoundingSymbol | 8 | 0 – 255 |  |
| } |  |  |  |
| If (relay is supported) { |  |  |  |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | 0 – 1 |  |
| } |  |  |  |
| If (E-MBS is configured) { |  |  |  |
| Zone_Allocation-Bit-MAP | 20 or 9 or 3 | The number of bits in the bit map = the maximum number of sub-bands for a given bandwidth (i.e., 20MHz, 10MHz, 5MHz) - 1. 20 MHz: b0-b19 10 MHz: b0-b8 5 MHz: b0-b2 bi = 1 if resource is changed between sub-band i and sub-band i+1. bi = 0 if resource is not changed between sub-band i and sub-band i+1. See section 16.9.3.1. 16.2.3.60. |  |
| ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1. 16.2.3.60. |  |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms (NMSI = 2) 0b01: 4 superframes, 80 ms (NMSI = 4) 0b10: 8 superframes, 160 ms (NMSI = 8) 0b11: 16 superframes, 320 ms (NMSI = 16) |  |
| E-MBS AAI frame offset | Variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 3 bits MSI length == 0b01: 4 bits MSI length == 0b10: 5 bits MSI length == 0b11: 6 bits |  |
| } |  |  |  |
| For(i=0; i<N-UL-Feedback-Info; i++) { |  | N-UL-Feedback-Info is the number of UL feedback Information included here [1…8] |  |
| If (multicarrier is configured in multicarrier Aggregation with DL-only secondary partially configured carriers) { |  |  |  |
| physical PrimaryCarrierIndex | 6 | The physical carrier index for primary carrier | Present when all the AMSs do not support a single identical primary carrier |
| startDLRUIndex | 7 | The start DLRUs index for feedback channel |  |
| dLRUNum | 4 | The number of DLRUs for feedback channel per UL AAI sub-frame (Refer to 16.3.8.3.3.2 16.3.7.3.3) |  |
| hARQChannelNum | 2 | The number of HARQ feedback channel per HARQ feedback region. Describes LHFB in 16.3.8.3.3.2 16.3.7.3.3. Channel numbers represented by the two bits (0, 1, 2, 3) are as follows. For 5 MHz band, 6, 12, 18, 24 For 10 MHz band, 6, 12, 24, 30 For 20 MHz band, 12, 24, 48, 60 |  |
| } |  |  |  |
| } |  |  |  |
| If (FPS2 FPS2 from DFPC >0) { |  | See Table 805 to Table 807 |  |
| resourceMetricFP2 | 4 | Resource_Metric_FP2 Resource Metric of the first power de-boosted frequency partition which is defined in Table 796 792. This parameter does not affect "Configuration Change Count" |  |
| } |  |  |  |
| If(FPS3 FPS3 from DFPC >0) { |  | See Table 805 to Table 807 |  |
| resourceMetricFP3 | 4 | Resource_Metric_FP3 Resource Metric of the second power de-boosted frequency partition which is defined in Table 796 792. This parameter does not affect "Configuration Change Count" |  |
| } |  |  |  |
| Network synchronization indicator | 1 | Indicates whether ABS achieves synchronization from backhaul network (0b01) or not (0b00) | Optional |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D9, page 190, line 50, modify Table 709 using the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

16.2.3.32 AAI-UL Noise and Interference Level Broadcast Message

An AAI-ULPC_NI AAI-ULPC-NI broadcast NI values for the UL power control.

Table 712—AAI-ULPC_NI AAI-ULPC-NI Field Descriptions

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| If (Sounding is used Uplink AAI subframes for sounding != 0b000) |  |  | Uplink AAI subframes for sounding is transmitted in SFH-SP1 |
| iotSounding | 7 | IoT value of sounding, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. |  |
| } |  |  |  |
| If(Frequency Partition 0 is used FPS0 from UFPC > 0){ |  |  |  |
| gammaIotFp0 iotFp0 | 7 | IoT value of Frequency Partition #0, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. |  |
| } |  |  |  |
| If(Frequency Partition 1 is used FPS1 from UFPC > 0){ |  |  |  |
| gammaIotFp1 iotFp1 | 7 | IoT value of Frequency Partition #1, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. |  |
| } |  |  |  |
| If(Frequency Partition 2 is used FPS2 from UFPC > 0){ |  |  |  |
| gammaIotFp2 iotFp2 | 7 | IoT value of Frequency Partition #2, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. |  |
| } |  |  |  |
| If(Frequency Partition 3 is used FPS3 from UFPC > 0){ |  |  |  |
| gammaIotFp3 iotFp3 | 7 | IoT value of Frequency Partition #3, quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB. |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------