| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on AAI-Multi_BS_SOUNDING-CAL (16.2.3.41) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Wookbong Lee, Jinyoung Chun, Jin Sam Kwak LG Electronics | wookbong.lee@lge.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Text proposal on AAI-Multi_BS_SOUNDING-CAL MAC messages and corresponding ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on AAI-Multi_BS_SOUNDING-CAL (16.2.3.41)

Wookbong Lee, Jinyoung Chun and Jin Sam Kwak

LG Electronics

Introduction

We need to update AAI-Multi_BS_SOUNDING-CAL message as well as corresponding ASN.1 code.

Proposed Text

Remedy 1:

In the P802.16m/D9, page 204, modify Table 718 as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 718—AAI-Multi_BS_SOUNDING-CAL AAI-MBS-SOUNDING-CAL

| Field | Size (bit) | Value/Description | Condition |
| --- | --- | --- | --- |
| Calibration mode calibrationMode | 1 | 0: The calibration sounding mode which carries calibration phases of each involved ABS 1: The calibration sounding mode which carries differential values of calibration phases from involved adjacent ABSs to S-ABS |  |
| Superframe number superframeNumber | 2 | The superframe number where multiBS sounding calibration is scheduled If this message is transmitted in i-th superframe, then the sounding signal shall be transmitted in i+Superframe number superframe 0b00: 2 0b01: 4 0b10: 6 0b11: 8 |  |
| Frame number frameNumber | 2 | The frame number where multiBS sounding calibration is scheduled to start from |  |
| Sounding AAI subframe soundingAAISubframe | 3 | The AAI subframe where multiBS sounding calibration is scheduled to start from |  |
| Sounding subband bitmap soundingSubbandBitmap | 6-24 | FFT size dependent 512 FFT: 6 bit 1024 FFT: 12 bit 2048 FFT: 24 bit |  |
| Decimation offset d decimationOffset | 5 | Unique decimation offset |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy 2:

In the P802.16m/D9, line 14-35 of page 1071, modify texts as follows:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AAI-MBS-SOUNDING-CAL ::= SEQUENCE {

calibrationMode INTEGER (0..1),

superframeNumber ENUMERATED {

two,

four,

six,

eight

},

frameNumber INTEGER (0..3),

soundingAAISubframe INTEGER (0..7),

soundingSubbandBitmap BIT STRING (SIZE(6..24)),

decimationOffset INTEGER (0..63),

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------