| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Review AAI-RNG-RSP ASN.1 code |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Review AAI-RNG-RSP ASN.1 code |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Review AAI-RNG-RSP ASN.1 code

Joey Chou

Intel

Introduction

This contribution proposes changes to AAI-RNG-RSP ASN.1 code that uses CHOICE to indicate RNG-RSP based on ringing purpose in the AAI-RNG-REQ.

Proposed Text

Remedy 1:

In the P802.16m/D9, page993, line 11, Change as the following:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

RngRspForHoReentryInfo ::= SEQUENCE {

embsZoneInfoList               SEQUENCE (SIZE(0..7)) OF

EMBSZoneInfoInHandover,

multicastInfo                  SEQUENCE(SIZE(1..15)) OF SEQUENCE {

currentMulticastGroupID      BIT STRING(SIZE(12)),

currentFID                   BIT STRING(SIZE(4)),

newMulticastGroupID          BIT STRING(SIZE(12)),

newFID                       BIT STRING(SIZE(4))} OPTIONAL

}

LocationUpdateResponse  ::= SEQUENCE {

locationUpdateRsp              LocationUpdateRsp OPTIONAL,

locationUpdateResult           CHOICE {

-- locationUpdateResponse = 0x0

successOfLocationUpdate      SuccessOfLocationUpdate,

others                       NULL}

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Ranging Response Message

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-RNG-RSP ::= SEQUENCE {

-- set to 1 when an ABS rejects the AMS

rangingAbortFlag               BOOLEAN,

timerOrSTID                    CHOICE {

-- Timer defined by an ABS to prohibit the AMS from attempting

-- network entry at this ABS, for a specific time duration

-- Value 0 (do not try ranging again at the ABS)

-- Value 1-65535, in units of seconds

rangingAbortTimer            INTEGER(0..65535),

tempStidAndMAPMaskSeed       SEQUENCE {

temporarySTID              STID,

mapMaskSeed                BIT STRING (SIZE(15))}

},

amsidOrMacAddress              CHOICE {

-- selected for advanced network mode and AMSID privacy is enabled

amsidStarHashValue           MACAddress,

-- selected for other cases

macAddress                   MACAddress},

stid                           STID OPTIONAL,

mapMaskSeed                    BIT STRING (SIZE(15)) OPTIONAL,

crid                           CRID OPTIONAL,

-- response based on ringing purpose sent in AAI-RNG-REQ

ringingPurpose                 CHOICE {

emergencyCallSetup           SEQUENCE {

emergencyServiceFID            INTEGER(2..15)},

nsEpCallSetup                SEQUENCE {

nsEPServiceFID                 INTEGER(2..15)},

locationUpdatePowerDown      LocationUpdateResponse,

locationUpdateEmbsFlows      LocationUpdateResponse,

idleModeLocationUpdate       LocationUpdateResponse,

locationUpdateToDcrMode      LocationUpdateResponse,

dcrModeExtension             LocationUpdateResponse},

-- bitmap for Reentry Process Optimization

reentryProcessOptimization     ReentryProOptimization OPTIONAL,

activationDeadline             INTEGER (0..63) OPTIONAL,

-- 1: perform neighbor station measurement report

nbrBsMeasurementRptIndicator   BOOLEAN OPTIONAL,

resourceRetainTime             INTEGER(0..255) OPTIONAL,

flowUpdating                   SEQUENCE (SIZE(1..16)) OF SEQUENCE {

sfid                         BIT STRING (SIZE(32)),

updateOrDelete               BOOLEAN,

updtedQoSInfo                QosParameter} OPTIONAL,

unsolicitedBsGrantIndicator    BOOLEAN OPTIONAL,

clcResponse                    CLCResponse OPTIONAL,

csgid                          CSGID OPTIONAL,

nbrAbsRedirectInfoList         SEQUENCE (SIZE(1..8)) OF RedirectionInfo OPTIONAL,

rangingRequestBit              BOOLEAN OPTIONAL,

invalidFIDList                 SEQUENCE (SIZE(1..16)) OF

FID OPTIONAL,

saidUpdateBitMap               BIT STRING (SIZE(16)) OPTIONAL,

rngRspForHandoverReentry       RngRspForHoReentryInfo OPTIONAL,

...

}

--------------------------------------------------  Text End -----------------------------------------------------------------