| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on the MSI Length in IEEE P802.16m (16.2.3.31) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Jaejeong Shim, Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang Samsung Electronics | Email : brian.shim@samsung.com kjosiam@sta.samsung.com ty33.kim@samsung.com wang.hai@samsung.com |
| Re: | “P802.16m/D9,” in response to the IEEE 802.16 Working Group Sponsor Ballot Recirc #3 |  |
| Abstract | E-MBS operation to support carrier switch mode on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on the MSI Length in IEEE P802.16m (16.2.3.31)

Jaejeong Shim, Kaushik Josiam, Taeyoung Kim, Hai Wang

Samsung Electronics

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS) operation. Originally the MSI Length is unique in a single E-MBS Zone. But now we have a single MSI Length for all the E-MBS Zones. In that sense, the value of 2 is not a reasonable and too short to be a common MSI length for all the E- MBS Zone. Proposed to change the range of MSI Length from 2, 4, 8, and 16 to 4, 8, 16, and 32.

# References

- IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” Oct. 2010

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text in 16.2.3.31 in 802.16m/D9, pp.189]

16.2.3.31 AAI-System Configuration Descriptor (SCD) Message

…………………………

Table 708—AAI-SCD Message Field Description

| ZF | 1 | Zone Flag bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS See section 16.9.3.1. |  |
| --- | --- | --- | --- |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 24 superframes, 2040 ms (NMSI = 2) 0b01: 48 superframes, 4080 ms (NMSI = 4) 0b10: 816 superframes, 80160 ms (NMSI = 8) 0b11: 1632 superframes, 320640 ms (NMSI = 16) |  |
| E-MBS AAI frame offset | Variable | The location of the AAI frame where the E-MBS data burst ends MSI length == 0b00: 34 bits MSI length == 0b01: 45 bits MSI length == 0b10: 56 bits MSI length == 0b11: 67 bits |  |

[Remedy2: Add the following changes (pp.230) in Table 734—AAI-DSA-REQ Message Field Description of 802.16m/D9]

Table 734—AAI-DSA-REQ Message Field Description

| If(Carrier Switching Mode==0b0) { |  |  |  |
| --- | --- | --- | --- |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit |  |
| } |  |  |  |

[Remedy3: Add the following changes (pp.237) in Table 735—AAI-DSA-RSP Message Field Description of 802.16m/D9]

Table 735—AAI-DSA-RSP Message Field Description

| If(Carrier Switching Mode==0b0) { |  |  |  |
| --- | --- | --- | --- |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit |  |
| } |  |  |  |

[Remedy4: Add the following changes (pp.246) in Table 737—AAI-DSC-REQ Message Field Description of 802.16m/D9]

Table 737—AAI-DSC-REQ Message Field Description

| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | In case of ABS-initiated DSC for carrier switching to update Shall be present if Carrier Switching Mode == 0b0 |
| --- | --- | --- | --- |

[Remedy5: Add the following changes (pp.279) in Table 753—AAI-E-MBS-RSP Message Field Description of 802.16m/D9]

Table 753—AAI-E-MBS-RSP Message Field Description

| Unicast Available Interval Bitmap | Variable (maximum: 1632) | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2...bN-1 If bi==0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 24 superframes: N = 4 bits NMSI = 48 superframes: N = 48 bits NMSI = 816 superframes: N = 816 bits NMSI = 1632 superframes: N = 1632 bits Depending on the NMSI, the number of bits per subframe changes, 4 frames per bit If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present when the report mode in AAI-E-MBS-REP message is 0b01 |
| --- | --- | --- | --- |

[Remedy6: Add the following changes (pp.620) in Table 829—E-MBS MAP Structure

of 802.16m/D9]

Table 829—E-MBS MAP Structure

| S-SFH SP2 transmission time offset | Variable | Indicates the superframe offset at which the changed SSFH SP2 is transmitted. The size of this field depends on MSI MSI == 0b00: 12 bits MSI == 0b01: 23 bits MSI == 0b10: 34 bits MSI == 0b11: 45 bits |
| --- | --- | --- |

[Remedy7: Add the following changes (pp.702) in Table 860—E-MBS DATA Information Elements of 802.16m/D9]

Table 860—E-MBS DATA Information Elements

| E-MBS AAI subframe offset | Variable | Includes the index of the AAI subframe where the E-MBS data burst ends and the first AAI subframe of the first frame of each superframe is reserved to transmit superframe header, where any E-MBS data burst is not transmitted. MSI == 0b00: 67 bits MSI == 0b01: 78 bits MSI == 0b10: 89 bits MSI == 0b11: 910 bits The AAI subframe offset index begins at the beginning of the MSI. |
| --- | --- | --- |

[Remedy8: Add the following changes (pp.954) in 16.9.3.3 E-MBS Scheduling Interval (MSI) of 802.16m/D9]

16.9.3.3 E-MBS Scheduling Interval (MSI)

To be updated.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]