| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Ranging Channel for ARS (16.6.3) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot Recirc #3 |  |
| Abstract | This contribution provides the clarification of the ranging channel for ARS. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the Ranging Channel for ARS (16.6.3)

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

In IEEE 802.16m/D9, there is no description about ranging channel for ARS. At first, ARS entries ABS’s network in Access zone and the ARS performs NS-RNG channel in Access zone. And then the ARS connects in Relay zone with ABS and performs S-RNG in Relay zone. Therefore S-RNG may be existed in relay zone for ARS. Even though S-RNG defined by AAI_SCD message is in relay zone, ARS may not be able to read AAI_SCD message. And if S-RNG defined by AAI_SCD message is in access zone, new S-RNG channel is needed for ARS in relay zone. Therefore we suggest S-RNG channel for ARS is defined in AAI-ARS-CONFIG-CMD message.

- Text proposal in the P802.16m/D9

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Add the below rows in line 14 on the page 277 in the section 16.2.3.57]

Table 751 – AAI-ARS-CONFIG-CMD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| R_UCASi | 3/2/1 | AAI_UL_Relay_zone_Cell_Specific_parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  |
| Allocation periodicity of the S-RCH | 2 | Indicates the periodicity of the S-RCH allocation. 0b00: every frame 0b01: the second frame in every superframe 0b10: the second frame in every 4th superframe, i.e., mod(superframe number, 4)=0 0b11: the second frame in every 8th superframe, i.e., mod(superframe number, 8)=0 |  |
| Subframe offset of the S-RCH | 2 | Indicates the subframe offset (OSF) of the S-RCH allocation. The range of values is . S-RCH is allocated in the (OSF + UAZ) subframe. |  |
| Start RP code information of the S-RCH | 4 | Indicates the ks which is the parameter controlling the start root index of the RP codes (rs0). The range of values is . |  |
| NPE | 2 | Indicates the number of periodic code (NPE) according to the Table 917. |  |
| } |  |  |  |

[Add the below rows in line 13 on the page 936 in the section 16.6.3.6]

16.6.3.6 Uplink Control Structure

…

If an R-RTI is present, then the ARS shall not use sounding channel in the first AAI subframe of the AAI UL Relay zone. In the ARS FDD frames, if an R-TTI is present and the last AAI subframe of the AAI Relay zone is a type-1 subframe, then the ABS shall not allocate sounding channel in this AAI subframe.

There is no additional NS-RCH for ARS. S-RCH may be allocated in the UL Relay Zone by the allocation field of AAI-ARS-CONFIG-CMD Message.

-----------------------------------------------   Text End  -----------------------------------------------------------