| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Amendments for AAI-L2-XFER |  |
| Date Submitted | 2010-10-26 |  |
| Source(s) | Junghoon Jee, Jaesun Cha ETRI Peretz Feder, Suresh Nair, Ajay Rajkumar ALU | E-mail: jhjee@etri.re.kr |
| Re: | Category: D9 / Area: Chapter 16.2.3.30, Annex R.2 (AAI-L2-XFER) |  |
| Abstract | This contribution proposes amendments for AAI-L2-XFER in the IEEE 802.16m specification. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Amendments for AAI-L2-XFER in IEEE 802.16m (16.2.3.30, Annex R.2)

Junghoon Jee, Jaesun Cha

ETRI

Peretz Feder, Suresh Nair, Ajay Rajkumar

ALU

- Introduction

AAI-L2-XFER does not need to directly encapsulate other RAT information because the MIH frame with the subtype, '7' already contains that information.

This contribution proposes amendments for AAI-L2-XFER in 16.2.3.30 and Annex R.2 of IEEE P802.16m/D9.

- Proposed Text for IEEE P802.16m/D9

============================ Start of Proposed Text ==================================

[Editor’s Note: The following is the proposed change in the 802.16m/D9. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: page 183, line 27 in 16.2.3.30

[Change text as follows:]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| L2-Xfer Type | 8 | Transfer-Type=1; GNSS assistance (DL) Transfer-type=2 ; LBS measurement [Terrestrial meas. and GNSS pseudo ranges] (UL) Transfer-Type=3; Device Bootstrap (DL/UL) Transfer-Type=4; WirelessMAN- OFDMA Advanced Air Interface System network boundary indication (DL) Transfer-Type=5; ORAT-MSG (DL) Transfer-Type=65; SMS Transfer-Type=76; MIH Frame Transfer-Type=87; ASN control mes- sages for Relay support (DL/UL) Transfer-Type=98; Emergency Alert Transfer-Type=109-127; reserved Transfer-Type=128-255; Vendor spe- cific types |  |
| If (Transfer-Type=5) { |  |  |  |
| L2-Xfer Sub Type | 4 | Sub-Type=1:GERAN(GSM/GPRS/ EGPRS) Sub-Type=2: UTRAN Sub-Type=3: E-UTRAN Sub-Type=4: TDSCDMA Sub-Type=5: CDMA2000 |  |
| L2-Xfer payload |  |  |  |
| } else if (Transfer-Type= 65) { |  |  |  |
| L2-Xfer Sub Type | 4 | Sub-Type=1: SMS data Sub-Type=2: SMS confirmation |  |
| L2-Xfer payload |  |  |  |
| } else if (Transfer-Type= 76) { |  |  |  |
| L2-Xfer Sub Type | 4 | Sub-Type=1: Service Management Sub-Type=2: Event Service Sub-Type=3: Command Service Sub-Type=4: Information Service |  |
| L2-Xfer payload |  |  |  |
| } else { |  |  |  |
| L2-Xfer payload |  |  |  |
| } |  |  |  |

The enumeration of Transfer-Type is as follows:

a) 	Transfer-Type = 1; GNSS assistance (DL)

b) 	Transfer-Type = 2; LBS measurement [Terrestrial meas. and GNSS pseudo ranges] (UL)

c) 	Transfer-Type = 3; Device Bootstrap (DL/UL)

d) 	Transfer-Type = 4; WirelessMAN-OFDMA Advanced Air Interface System network boundary indication (DL)

e) 	Transfer-Type = 5; ORAT-MSG (DL)

i)  Sub-Type = 1: GERAN (GSM/GPRS/EGPRS)

ii)   Sub-Type = 2: UTRAN

iii)   Sub-Type = 3: E-UTRAN

iv)   Sub-Type = 4: TDSCDMA

v)   Sub-Type = 5: CDMA2000

f)	Transfer-Type = 65: SMS

i)   Sub-Type = SMS data

ii)   Sub-Type = SMS confirmation

g)  Transfer-Type = 76: MIH Frame

i)   Sub-Type =1 : Service Management

ii)  Sub-Type =2 : Event Service

iii)  Sub-Type = 3 : Command Service

iv)  Sub-Type = 4 : Information Service

h)   Transfer-Type = 87; ASN control messages for Relay support (DL/UL)

i)   Transfer-Type = 98; Emergency Alert

j)   Transfer-Type = 109-127; reserved

k)   Transfer-Type = 128-255; Vendor specific types

Suggested change #2: page 1081, line 13 in Annex R.2

[Change text as follows:]

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-L2-XFER ::= SEQUENCE {

transferType INTEGER {

gnssAssistance (1),

lbsMeasurement (2),

deviceBootstrap (3),

wirelessMAN (4),

oratMsg (5),

sms (65),

mihFrame (76),

relaySupport (87),

emergencyAlert (98)

--values 10 to 127 are reserved

--values 128 to 255 are for vendor-specific types

} (0..255),

--if transferType = 5, transferSubtype shall have one of the following values:

--1: GERAN (GPS, GPRS, EGPRS)

--2: UTRAN

--3: E-UTRAN

--4: TD-SCDMA

--5: CDMA2000

--if transferType = 65, transferSubtype shall have one of the following values:

--1: SMS data

--2: SMS confirmation

--if transferType = 76, transferSubtype shall have one of the following values:

--1: service management

--2: event service

--3: command service

--4: information service

--otherwise, transferSubtype shall be absent

transferSubtype INTEGER (0..15) OPTIONAL,

payload OCTET STRING (SIZE(1..9999)) OPTIONAL, -- ??? upper bound?

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D9. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.