| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Missing ASN.1 code of Inner IP Protocol Attributes |  |
| Date Submitted | 2010-12-18 |  |
| Source(s) | Joey Chou Intel | Joey.Chou@intel.com |
| Re: | Sponsor Ballot for P802.16m (IEEE P802.16m/D9) |  |
| Abstract | Missing ASN.1 code of Inner IP Protocol Attributes |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Missing ASN.1 code of Inner IP Protocol Attributes

Joeyn Chou

Intel

Introduction

This contribution proposes the ASN.1 code of inner IP protocol attributes that are in AAI-DSA-REQ and AAI-DSC-REQ message tables, but are missing in Annex R.2.

Proposed Text

Remedy 1:

In the P802.16m/D10, page 1021, line 54, changes as shown below:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

ClassificationRule ::= SEQUENCE {

priority                       INTEGER (0..255) OPTIONAL,

protocolField                  INTEGER (0..255) OPTIONAL,

ipMaskAndSrcAddr               IpAddrAndMask OPTIONAL,

ipMaskAndDestAddr              IpAddrAndMask OPTIONAL,

protocolSrcPortRange           ProtocolPortRange OPTIONAL,

protocolDestPortRange          ProtocolPortRange OPTIONAL,

associatedPhsiIndex            INTEGER (0..255) OPTIONAL,

classificationRuleIndex        INTEGER (0..65535) OPTIONAL,

vendorID                       OCTET STRING (SIZE (3)) OPTIONAL,

ipV6FlowLabel                  BIT STRING (SIZE (24)) OPTIONAL,

classificationAction           ENUMERATED {

none (0),

discard (1)

} OPTIONAL,

dscpValue                      INTEGER (0..63) OPTIONAL,

phsDscAction                   PHSDSCAction OPTIONAL,

phsRuleAdded                   PhsRuleActions OPTIONAL,

ethernetAttributes             EthernetAttributes OPTIONAL,

innerIpProtocolInfo            InnerIpProtocolInfo OPTIONAL

}

InnerIpProtocolInfo ::= SEQUENCE {

innerSrcIpAddr                 BIT STRING (32) OPTIONAL,

innerDestIpAddr                BIT STRING (32) OPTIONAL,

innerProtocolField             BIT STRING (8) OPTIONAL,

innerTos                       BIT STRING (8) OPTIONAL,

innerIpv6FlowLable             BIT STRING (8) OPTIONAL

}

--------------------------------------------------  Text End  -----------------------------------------------------------------