| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on the E-MBS Operation on Multicarrier Deployment in IEEE P802.16m (16.2.3.2 & 16.9.2.4) |  |
| Date Submitted | 2010-11-09 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D9,” in response to the IEEE 802.16 Working Group Sponsor Ballot Recirc #3 |  |
| Abstract | To be clear E-MBS operation to support carrier switch mode on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comments on the E-MBS Operation on Multicarrier Deployment in IEEE P802.16m (16.2.3.2 & 16.9.2.4)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

IEEE 802.16m Amendment Draft Standard [1] describes the Enhanced Multicast Broadcast Service (E-MBS) operation. In the part of E-MBS HO procedure, service flow related to E-MBS may be included in the AAI_RNG-RSP for the MBS location update. To be more specific, in order to support E-MBS operation on the multicarrier deployment, unicast available interval bitmap to perform carrier switching is defined. Besides, when the service flow is changed due to change of E-MBS zone, the unicast available interval may be updated.

Therefore, some parameter related to E-MBS operation on the multicarrier deployment is proposed in this contribution.

# References

- IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” October 2010

# Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Remedy1: Add the following text in the end of 16.9.2.4 in 802.16m/D9]

16.9.2.4 E-MBS Operation on the multi-carrier deployment

…………………………

When the AMS performing carrier switching transits to a new E-MBS Zone, the AMS shall send AAI-RNG-REQ message with Ranging Purpose Indication = 0b0110 at the T-ABS and the ABS shall include Carrier Switching Mode and Unicast Available Interval Bitmap in AAI-RNG-RSP parameters to provide continuous reception of E-MBS contents.

[Remedy2: Add the following rows in Table 679-AAI_RNG-RSP in 16.2.3.2 of 802.16m/D9]

Table 679—AAI-RNG-RSP

| Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- |
| … | … | … | … |
| If (Location Update Response == 0x0) { |  |  |  |
| … | … | … | … |
| Physical Carrier Index | 6 | The physical carrier index of the T-ABS that supports the new EMBS Zone. | Present only if the new E-MBS Zone ID is served on a different carrier from the previous one at the T-ABS. |
| Carrier Switching Mode | 1 | 0b0: carrier switching based on Unicast Available Interval 0b1: carrier switching parameters using AAI_E-MBS-REP message | Present only if AMS is operated in carrier switching and to update Unicast Available Interval Bitmap |
| Unicast Available Interval Bitmap | Variable | Indicates when the AMS should be available in the primary carrier using N bits b0b1b2 …bN-1 If bi == 0, then AMS is available for E-MBS data scheduling in secondary carrier If bi==1, then AMS is available for unicast scheduling in primary carrier NMSI = 2 superframes: N = 4 bits NMSI = 4 superframes: N = 4 bits NMSI = 8 superframes: N = 8 bits NMSI = 16 superframes: N = 16 bits Depending on the NMSI, the number of bits per subframe changes If NMSI = 2, then 2 frames per bit If NMSI = 4, 8 and 16, then 4 frames per bit | Present only if AMS is operated in carrier switching and if Carrier Switching Mode = 0b0 |
| If(service flow update indicator == 0b0) { | - |  |  |
| … |  |  |  |
| }//end of If(Location Update Response == 0x0) |  |  |  |
| … |  |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]