| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification to support macro-diversity mode for multicast transmission in IEEE 802.16m (16.3.6) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Youngsoo Yuk, Jeongki Kim and Jinsoo Choi LG Electronics | E-mail: youngsoo.yuk@lge.com, jeongki.kim@lge.com , js.choi@lge.com |
| Re: | Sponsor Ballot Recirc #3 |  |
| Abstract | The contribution provides the text for multicast operation in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification to support macro-diversity mode for multicast transmission in IEEE 802.16m (16.3.6)

Youngsoo Yuk, Jeongki Kim and Jinsoo Choi

LG Electronics Inc.

- Introduction

The macro-diversity transmission is helpful for enhancing the transmission efficiency of the network wide common data such as multicast traffic. Even though E-MBS is defined for supporting this feature, it is not flexible and hard to apply to the multicast service.

However, at the receiver size, the only difference of macro-diversity transmission is the use of the pilot pattern which has higher density in frequency domain, which is already defined in E-MBS PHY.

In this contribution, we proposed a method to provide macro-diversity transmission by using simple indication for pilot pattern inside the Broadcast assignment A-MAP IE for multicast traffic.

By referring the pilot type inside the A-MAP, AMSs can know whether macro-diversity transmission is applied to the allocated resource or not.

- Proposed Text Changes

Removed text

Added text

[Remedy #2, Modify the following text in Table 858, page 698, section 16.3.5.5.2.4.13 as follows,]

16.3.5.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast or multicast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 858 describes the fields in a broadcast assignment A-MAP IE.

Table 858 – Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 2 | 0b00: This IE carries broadcast assignment information 0b01: This IE carriers ranging channel allocation information 0b10: This IE carriers multicast assignment information 0b11: reserved |
| If(Function Index == 0b00){ |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst shall be transmitted one more time 0b10: the same burst shall be transmitted two more times 0b11: the same burst shall be transmitted three more times |
| Reserved | 13 | Reserved bits |
| }else{ |  |  |
| Reserved | 15 | Reserved bits |
| } |  |  |
| }else if{Function Index == 10b01}{ |  |  |
| Number of Ranging Opportunities (N) | 1 | 0: one NS-RCH 1: two NS-RCHs |
| for(i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| Ranging opportunity index | 2 | Opportunity index of the ranging channel |
| } |  |  |
| Reserved | 10 | Reserved bits |
| } else if{Function Index == 0b10}{ |  |  |
| Multicast Group ID | 12 | ID of a group that receives multicast assignment |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Pilot Type | 1 | Indicates the pilot pattern used by the allocated resource 0b0: Use Normal pilot patterns 0b1: Use E-MBS zone specific pilot patterns shown in 16.3.4.4.4. |
| Long_TTI_Indicator | 1 | Indicates number fo AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default TTI) 0b1: 4 DL AAI subframe for FDD or all DL AAI subframes for TDD (long TTI) |
| Reserved | 43 | Reserved bits |
| } |  |  |
| } |  |  |

*A 16 bit CRC is generated based on the randomized contents of the Broadcast Assignment A-MAP IE. The CRC is masked by the 16-bit CRC mask generated according to Table 846. If Function index == 0b00 or 0b01, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b001. If Function index == 0b10, the CRC is masked by the 16-bit CRC mask with masking prefix = 0b0 and message type indicator = 0b010.

Multicast Group ID is the ID of a group that receives multicast burst and is unique to an ABS. Multicast Group ID is assigned by AAI-DSA-REQ/RSP message and changed by AAI-DSC_REQ message.

The broadcast and multicast bursts signaled by the Broadcast Assignment A-MAP IE isare always transmitted using SFBC as the MIMO encoder format and QPSK as the modulation scheme.

------------------------ End of the proposed text ------------------------------------------------------------