| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Consideration of Emergency Alert Indication in AAI-TRF-IND-RSP message (16.2.3.29) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Yeongmoon Son, Hyunjeong Kang Samsung Electronics | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to include Emergency Service Indication in AAI-TRF-IND-RSP message. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Consideration of Emergency Alert Indication in AAI-TRF-IND-RSP message

Yeongmoon Son, Hyunjeong Kang

Samsung Electronics

- Introduction

An AAI-TRF-IND message in 16m/D9 contains ‘Emergency Alert Indication’ which is used to indicate the presence of emergency information for supporting the emergency alert service. That is, when Emergency Alert Indication is set to ‘1’, the AMSs shall keep awake to receive the emergency information regardless their own Sleep Cycle pattern.

AAI-TRF-IND-REQ/RSP message is used for synchronization of Sleep Cycle between AMS and ABS when AMS fails to get its own traffic indication during Listening Window. However, AAI_TRF-IND-RSP message doesn’t have ‘Emergency Alert Indication’ which is included in the AAI-TRF-IND message. For the consistency, the AAI_TRF-IND-RSP needs to have ‘Emergency Alert Indication’ to support Emergency Service.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Modify the Table 706 as follows:]

| Table 709—Parameters for AAI-TRF_IND-RSP |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| AAI-TRF_IND-RSP message_format() { | — | — |  |
| Frame_Number | 10 | The least significant 10 bits of the frame number in which incoming LW will start 0~1023 |  |
| Sleep Cycle Length | 10 | The length of Sleep Cycle which contains the next scheduled Listening Window. If the AMS receives the negative traffic indication during the next scheduled Listening Window, the (current) Sleep Cycle shall be set to this value at that time. Value: 0~1023 Sleep Cycle Length = Value + 1 |  |
| } | — | — |  |

-------------------------------  Text End  ---------------------------------------------------