| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI-DSA-REQ/RSP (16.2.3) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee Institute for Information Industry (III) Yih Guang Jan, Yang-Han Lee, Hsien-Wei Tseng Tamkang University (TKU) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) | cwchen@iii.org.tw |
| Re: | Sponsor Ballot P802.16m |  |
| Abstract | This contribution propose the clarification on AAI-DSA-REQ/RSP |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI-DSA-REQ/RSP (16.2.3)

Chiu-Wen Chen, Chun-Yen Hsu, Yi-Ting Lin, Tsung-Yu Tsai, Chun-Che Chien, Youn-Tai Lee
Institute for Information Industry (III)

Yih Guang Jan, Yang-Han Lee, Hsien-Wei Tseng
Tamkang University (TKU)

Shiann-Tsong Sheu
National Central University (NCU)

Whai-En Chen
National Ilan University (NIU)

Introduction

When an ABS commences E-MBS service flow, some parameters (e.g., E-MBS service, E-MBS Zone ID, E-MBS Service Flow Parameter, etc.) shall be included in the AAI-DSA-REQ/RSP message. In this contribution, the corresponding text is modified to clarify the “Num of E-MBS Zone ID”.

Proposed text for the P80216m/D9:

-----------------------------------------------Start of the Text Proposal 1--------------------------------------------

[Make the following changes to Section 16.2.3.47.1 in Page 229 beginning Line 36 in P802.16m/D9.]

#### 16.2.3.47.1 AAI-DSA-REQ

Table 734—AAI-DSA-REQ Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| E-MBS Service | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present when both ABS/AMS initiates AAI-DSA-REQ |
| If(E-MBS is supported) { |  |  |  |
| For (i=0; i<Num of N-E-MBS Zone ID; i++) { | 3 | Num of N-E-MBS Zone ID is the number of E-MBS zone IDs to add [1…8] |  |
| E-MBS_Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| for (i=0; i< N-E-MBS-ID; i++) { |  | N-E-MBS-ID is the number of E-MBS identifiers [1...15] |  |
| E-MBS ID | 12 | E-MBS identifier |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-----------------------------------------------End of the Text Proposal 1--------------------------------------------

-----------------------------------------------Start of the Text Proposal 2--------------------------------------------

[Make the following changes to Section 16.2.3.47.2 in Page 236 beginning Line 22 in P802.16m/D9.]

#### 16.2.3.47.2 AAI-DSA-RSP

Table 735—AAI-DSA-RSP Message Field Description

| Field | Size (bits) | Value/Description | Conditions |
| --- | --- | --- | --- |
| E-MBS Service | 3 | Indicates whether the E-MBS service is being requested or provided for the connection that is being setup. 1 indicates support, 0 indicates not support. Bit#0: E-MBS in S-ABS Only Bit#1: E-MBS in a multi-ABS Zone supporting macro-diversity Bit#2: E-MBS in a multi-ABS Zone not supporting macro-diversity If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| If(E-MBS is supported) { |  |  |  |
| For (i=0; i< Num of N-E-MBS Zone ID; i++) { | 3 | Num of N-E-MBS Zone ID is the number of E-MBS zone IDs to add [1..8] |  |
| E-MBS _Zone_ID | 7 | Indicates an E-MBS zone where the connection for associated service flow is valid |  |
| Physical Carrier Index | 6 | Target carrier which the AMS switches or is redirected by ABS to | Present only if the E-MBS Zone ID is served on a different carrier from the current carrier. |
| for (i=0; i< N-E-MBS-ID; i++) { |  | N-E-MBS-ID is the number of E-MBS identifiers [1...15] |  |
| E-MBS ID | 12 | E-MBS identifier |  |
| FID | 4 | Flow ID |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

-----------------------------------------------End of the Text Proposal 2---------------------------------------------

[1] IEEE P80216m/D9, “P802.16m/D9: Advanced Air Interface”