| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup on message format of AAI-SII-ADV (16.2.3.50) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Youngkyo Baek Samsung Electronics | youngkyo.baek@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | Cleanup on message format of AAI-SII-ADV |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup on message format of AAI-SII-ADV (16.2.3.50)

Youngkyo Baek

Samsung Electronics

Introduction

Current MAC control message table of AAI-SII-ADV in 16m/D9 has some redundant and non- required attributes and is not aligned with the ASN.1 encoding.

Therefore, cleanup on message table and its ASN.1 code is suggested in this contribution.

Proposed Text

Remedy #1

Modify Table 744, page 264 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 744— AAI-SII-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Number of NSP IDs | 4 | Number of Network Service Providers |  |
| for (i=0; i<number of NSP IDs; i++) { |  | List of the Network Service Provider identifiers. Number of NSP IDs[1..16] is the number of NSP identifier to be included here. |  |
| NSP identifier | 24 | NSP identifier which ABS supports |  |
| } |  |  |  |
| for (i=0; i< number of NSP IDs; i++) { |  | List of the verbose names of the NSPs. The value of Verbose NSP Name List is a compound list of verbose NSP name lengths and verbose NSP names. The order of the Verbose NSP Name Lengths and Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List. Number of NSP IDs[1..16] is the number of NSP identifier to be included here. |  |
| verbose NSP name length | 7 | Length of verbose NSP name (in byte) |  |
| verboseNSPspName | Variable (1 ... 128 x 8) | Verbose NSP name string |  |
| } |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #2

Modify texts, line 26, page 981 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Service Identity Information Advertisement

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-SII-ADV ::= SEQUENCE {

-- List of the verbose names of the NSPs. The value of Verbose NSP Name List is a list of verbose NSP names. The order of the Verbose NSP Names presented shall be in the same order as the NSP IDs presented in the NSP List.

nspInfoList SEQUENCE (SIZE(10..165)) OF NSPID,

-- Verbose NSP name string

verboseNspNameList SEQUENCE (SIZE(10..165)) OF VerboseName,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------