| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for network entry-related messages (16.2.3) |  |
| Date Submitted | 2010-10-25 |  |
| Source(s) | Youngkyo Baek Jaeyun Ko Sangheon Kim Ji-yun Seol Samsung Electronics | youngkyo.baek@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to clean-up for network entry-related messages in 802.16m. Especially it gives some modification on AAI-RNG-RSP, AAI-RNG-ACK and AAI-SBC-RSP messages. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for network entry-related messages (16.2.3)

Youngkyo Baek, Jaeyun Ko,Sangheon Kim, Ji-yun Seol

Samsung Electronics

- Introduction

Some MAC control message tables (e.g. AAI-RNG-RSP, AAI-RNG-ACK and AAI-SBC-RSP) in 16m/D9 have some redundant attributes, which are not aligned with their ASN.1 encoding, and some fixes are required..

Therefore, this contribution suggests cleanup on table formats and ASN.1 code of AAI-RNG-RSP, AAI-RNG-ACK and AAI-SBC-RSP messages.

- Text change

Remedy #1

Modify Table 679,line 44, page 96 as follows.

--------------------------------------------------  Text Start -----------------------------------------------------------------

Table 679— AAI-RNG-RSP message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ….. |  | ………….. | ………….. |
| MAPMaAsk Seed | 15 | This parameter shall be used at the target ABS to initiate the PRBS generator used to scramble the 40-bit A-A-MAP IE when the STID is used to mask the CRC. See 16.3.5.3.2.4. | Shall be present if STID is present |
| ….. |  | ………….. | ………….. |
| N-E-MBS-Zone-IDs | 3 | Number of E-MBS Zone IDs to be updated in the T-ABS |  |
| For(i=0; i< N-E-MBS-Zone- IDs; i++) { |  | Number of E-MBS Zone IDs [1..8] to be updated in the T-ABS | Present only if E-MBS related information is changed in the T-ABS. |
| ………….. |  | ………….. | ………….. |
| } |  |  |  |
| }else if ( Service flow update indicator == 0b1) { |  |  |  |
| For (j=0; j<N_EMBS_IDs; j++) { |  | N_EMBS_IDs is the number of EMBS IDs listed here[1..8]. Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| ………….. |  | ………….. | ………….. |
| } |  |  |  |
| }//end of if |  |  |  |
| }//end of for (N-E-MBS-Zone- IDs) |  |  |  |
| Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the T-ABS Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| For (i=0; i< M; i++) { |  | Number of Multicast Group ID and FID (M) to update in the T-ABS[1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| …………. |  | ………….. | ………….. |
| } |  |  |  |
| …………. |  | ………….. | ………….. |
| SAID update bitmap |  | ………….. | ………….. |
| If(it is under network re-entry for HO){ |  |  |  |
| N-E-MBS-Zone-IDs | 3 | Number of E-MBS Zone IDs to be updated in the T-ABS |  |
| For(i=0; i< N-E-MBS-Zone- IDs; i++) { |  | Number of E-MBS Zone IDs [1..8] to be updated in the T-ABS | Present only if E-MBS related information is changed in the T-ABS. |
| …………… |  | ………….. | ………….. |
| }else if ( Service flow update indicator == 0b1) { |  |  |  |
| For (j=0; j<N_EMBS_IDs; j++) { |  | N_EMBS_IDs is the number of EMBS IDs listed here[1..8]. Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. |  |
| …………… |  | ………….. | ………….. |
| }//end of for (N-E-MBS-Zone- IDs) |  |  |  |
| Num of Multicast Group ID and FID (M) to update | 4 | Number of Multicast Group ID and FID to update in the T-ABS | Presented if it needs to be updated. |
| For (i=0; i< M; i++) { |  | Number of Multicast Group ID and FID (M) to update in the T-ABS[1..16]. Mapping of current Multicast Group ID and FID and new Multicast Group ID and FID to be updated. Based on the value of Num of Multicast Group ID and FID to be updated | Presented if it needs to be updated. |
| …………… |  | ………….. | ………….. |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #2

Modify texts, line 34, page 984 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

SuccessOfLocationUpdate ::= SEQUENCE {

newPagingCycle             PgCycle OPTIONAL,

newPagingGroupID          PGID OPTIONAL,

newPagingOffset            PgOffset OPTIONAL,

deregistrationID             DID OPTIONAL,

newPagingControllerID      PCID OPTIONAL,

embsZoneInfo              SEQUENCE(SIZE(10..87)) OF

EMBSZoneInfoItem OPTIONAL,

multicastInfo               SEQUENCE(SIZE(10..165)) OF SEQUENCE {

currentMulticastGroupID   BIT STRING(SIZE(12)),

currentFID               BIT STRING(SIZE(4)),

newMulticastGroupID     BIT STRING(SIZE(12)),

newFID                 BIT STRING(SIZE(4))} OPTIONAL,

smsMessage              SMS OPTIONAL

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #3

Modify Table 680,line 12, page103 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 680— AAI-RNG-ACK message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ….. |  | ………….. | ………….. |
| N_Frame_Identifiers | 3 | The number of frame identifiers included in this RNG-ACK message. |  |
| For (i=0; i<N_Frame_Identifiers; i++) { |  | The number of frame identifiers (i.e. N_Frame_Identifiers) [1..8] included in this RNG-ACK message |  |
| ….. |  | ………….. | ………….. |
| N_Received_Codes | 6 | The number of ranging preamble code indices detected in this corresponding ranging opportunity. |  |
| For (j=0; j<N_Received_Codes; j++) { |  | The number of ranging preamble code indices(i.e. N_Received_Codes)[1..64] detected in this corresponding ranging opportunity. |  |
| ….. |  | ………….. | ………….. |
| If(API Bit#0 == 0b1) { |  |  |  |
| Timing offset adjustment | 15 | Amount of time required to adjust AMS transmission. Signed (in units of 1/Fs). MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1..16384] that corresponds to 0x0000 ~ 0x3FFF, respectively The AMS shall advance its transmission time if the value is negative (i.e.MSB = 0b1) and delay its transmission time if the value is positive (i.e. MSB = 0b0). |  |
| } |  |  |  |
| If(API Bit#1 == 0b1) { |  |  |  |
| Power level adjustment | 34 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the AMS shall apply to its current transmission power for initial ranging. MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 3 bits represent power level correction value of [1..8] that corresponds to 0b000 ~ 0b111, respectively The bit size represents power level correctionin the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. |  |
| } |  |  |  |
| If(API Bit#2 == 0b1) { |  |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (•ƒf) multiplied by the 9-bit number interpreted as a signed integer MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively |  |
| } |  |  |  |
| …… |  |  |  |
| }//end of for |  |  |  |
| } else if (transmitted in a DL resource allocated by DL basic assignment A-MAP IE) { |  |  |  |
| Ranging Status | 2 | Indicate whether ranging preamble code is received within acceptable limits by ABS. 0b00 = success 0b01 = abort 0b10 = continue |  |
| …… |  |  |  |
| If(API Bit#0 == 0b1) { |  |  |  |
| Timing offset adjustment | 15 | Amount of advance time required to adjust AMS transmission. Signed (in units of 1/Fs). MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 14 bits represent timing offset correction value of [1..16384] that corresponds to 0x0000 ~ 0x3FFF, respectively The AMS shall advance its transmission time if the value is negative(i.e MSB = 0b1) and delay its transmission time if the value is positive(i.e. MSB = 0b0). |  |
| } |  |  |  |
| If(API Bit#1 == 0b1) { |  |  |  |
| Power level adjustment | 34 | Power level adjustment which expresses the change in power level (in multiples of 1 dB) that the AMS shall apply to its current transmission power for initial ranging. MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 3 bits represent power level correction value of [1..8] that corresponds to 0b000 ~ 0b111, respectively The bit size represents power level correctionin the set {-8, -7, -6, -5, -4, -3, -2, -1, +1, +2, +3, +4, +5, +6, +7, +8} which corresponds 0b0000 to 0b1111. |  |
| } |  |  |  |
| If(API Bit#2 == 0b1) { |  |  |  |
| Frequency offset adjustment | 9 | Frequency offset adjustment. Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (•ƒf) multiplied by the 9-bit number interpreted as a signed integer MSB 1bit represents the sign of the value. That is, the value is negative(-) if the MSB=0b1, and the value is positive(+) if the MSB=0b0. LSB 8 bits represent frequency offset correction value of [1..256] that corresponds to 0x00 ~ 0xFF, respectively |  |
| } |  |  |  |
| …… |  |  |  |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #4

Modify texts, line 34, page 992 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment        BIT STRING (SIZE(15))    OPTIONAL,

powerLevelAdjustment        BIT STRING (SIZE(4))    OPTIONAL,

frequencyOffsetAdjustment    BIT STRING (SIZE(9))    OPTIONAL

}

Sign ::= ENUMERATED {

positive,

negative}

AdjustmentParameters ::= SEQUENCE {

timeingOffsetAdjustment        SEQUENCE {

sign                         Sign,

timingOffset                 INTEGER (1..16384)}, -- unit = 1/Fs

powerLevelAdjustment           SEQUENCE {

sign                         Sign,

powerLevelOffset             INTEGER (1..8)},  -- unit = dB

frequencyOffsetAdjustment      SEQUENCE {-- unit = 2% of subcarrier

sign                         Sign,     -- spacing

frequencyOffset              INTEGER (1..256)}

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #5

Modify texts, line 22,39 and 47, page 995 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AAI-RNG-RSP ::= SEQUENCE {

-- set to 1 when an ABS rejects the AMS

rangingAbortFlag BOOLEAN,

……………………………..

………………………………..

locationUpdateResult CHOICE {

-- locationUpdateResponse = 0x0

successOfLocationUpdate SuccessOfLocationUpdate,

others NULL},

-- bitmap for Reentry Process Optimization

reentryProcessOptimization ReentryProOptimization OPTIONAL,

activationDeadline INTEGER (0..63) OPTIONAL,

-- 1: perform neighbor station measurement report

nbrBsMeasurementRptIndicator BOOLEAN OPTIONAL,

resourceRetainTime INTEGER(0..255) OPTIONAL,

flowUpdating SEQUENCE (SIZE(10..16)) OF SEQUENCE {

sfid BIT STRING (SIZE(32)),

updateOrDelete BOOLEAN,

updtedQoSInfo QosParameter} OPTIONAL,

unsolicitedBsGrantIndicator BOOLEAN OPTIONAL,

clcResponse CLCResponse OPTIONAL,

csgid CSGID OPTIONAL,

-- ??? need to define maximum size of the array, 64 is temp used

nbrAbsRedirectInfoList SEQUENCE (SIZE(10..64)) OF RedirectionInfo OPTIONAL,

rangingRequestBit BOOLEAN OPTIONAL,

-- ??? in table, no information of maximum N_Invalid_FIDs

invalidFIDList SEQUENCE (SIZE(10..16)) OF

FID OPTIONAL,

saidUpdateBitMap BIT STRING (SIZE(16)) OPTIONAL,

rngRspForHandoverReentry RngRspForHoReentryInfo OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------