| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for SBC/REG message Tables (16.2.3) |  |
| Date Submitted | 2010-11-03 |  |
| Source(s) | Youngkyo Baek Samsung Electronics | youngkyo.baek@samsung.com +82-31-279-5084 |
| Re: | Sponsor ballot recirc #3 on P802.16m/D9 |  |
| Abstract | The contribution proposes to clean-up SBC/REG messages in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for SBC/REG message Tables (16.2.3)

Youngkyo Baek

Samsung Electronics

- Introduction

Some conditions in AAI-SBC/REG message tables are wrong or redundant or missed.

Cleanup is required.

- Text change

Remedy #1

Modify Table685, line32, page116 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 685— AAI-REG-REQ message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ….. |  | ………….. | ………….. |
| If (LBS Capabilities are required to setup) { |  |  |  |
| Capability for supporting A-GPS method | 1 | 0b0: no support 0b1: support | Present if needed |
| } |  |  |  |
| ….. |  | ………….. | ………….. |
| Capability for sounding antenna switching support | 1 | 0b0: no support 0b1: support | Present if needed |
| ….. |  | ………….. | ………….. |
| Host-Configuration-Capability-Indicator IE | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI-REG-RSP message. (One bit indicator) 0b0: not support 0b1: support | To be included only when Requested-Host-Configurations IE is not included Shall be present |
| Requested-Host-Configurations IE | Variable | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI-REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. | Shall be presentTo be included when additional host configurations are required. |
| ….. |  | ………….. | ………….. |

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #2

Modify ASN.1 code, line18, page1001 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

AAI-REG-REQ ::= SEQUENCE {

amsMacAddress   MACAddress,

amsCapNegotiation   AmsCapabilities,

csCapabilities     CsCapabilities,

hostConfigCapability CHOICE {

hostCfgCapIndicator   FeatureSupport,

-- Includes requested host configuration options in DHCP Options

-- format. no size given ???

requestedHostConfig   OCTET STRING (SIZE(0..999)) OPTIONAL },

globalCarrierConfigChangeCount   INTEGER (0..7),

amsInitAgpServiceAdaptation    FeatureSupport OPTIONAL,

...

}

--------------------------------------------------  Text End  -----------------------------------------------------------------

Remedy #3

Modify Table686, line49, page121 as follows.

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 686— AAI-REG-RSP message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| ….. |  | ………….. | ………….. |
| E-MBS capabilities | 3 | If Bit#0 is set to 1, it indicates E-MBS in S-ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present if needed |
| ….. |  | ………….. | ………….. |

--------------------------------------------------  Text End  -----------------------------------------------------------------